# 3GPP TSG SA WG3 (Security) Meeting #88	S3-171952

# 7 – 11 August, 2017, Dali, PR China	revision of S3-17xabc

# Source:	Nokia

# Title:	Requirements on subscriber privacy management

# Document for:	Approval

Agenda Item:	7.3 / TS 33.501

# 1	Decision/action requested

Subscriber privacy management in 5G includes several functions, for which privacy requirements need to be defined.

# 2	Rationale

5G implements subscriber privacy. For this, it is not sufficient to only provide a decryption / deconcealing function, but a generic privacy management is needed. In particular, even if there is only one public key for all subscribers of one operator envisioned, in reality, several private keys may be needed, e.g. for the operator to update the public key at all UEs or to disable a key. Thus, the appropriate key needs to be selected based on the received key identifier and managed.

It is proposed to provide the privacy management as a service. SA2 may decide which network function should host the privacy management service (PMS). Requirements on several functions are listed in this pCR that, Nokia believes, at least should be part of the PMS.

# 3	pCR

<<< START OF CHANGES >>>

<<< CHANGE 1 >>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

5GC	5G Core Network

5G-AN	5G Access Network

5G-RAN	5G Radio Access Network

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE: If it is not clear from the context, which meaning the acronym AMF has, it is spelled out.

IKE	Internet Key Exchange

KRF	Privacy key request function

NAS	Non Access Stratum

PDB	Privacy data base

PHF	Privacy handling function

PMS	Privacy management system

QoS	Quality of Service

SEG	Security Gateway

SIDF	SUPI decryption function

UE	User Equipment

UPF	User Plane Function

<<< CHANGE 2>>>

## 5.X	Requirements for subscriber privacy management

Editor’s note: Whether other functions for privacy management are needed is ffs and may depend on SA2 and CT4 decision.

### 5.X.1 	Privacy management service (PMS)

A privacy management service (PMS) shall be responsible for the handling of all subscriber privacy management related functions.

PMS shall be located in the home network environment.

PMS functions shall be located in a high secure module (HSM) environment, at least those that store the private key(s), select a private key, and are responsible for the decryption of the SUCI.

PMS takes care of the handling of the key identifiers, selecting the correct key for decryption of SUPI, initiating the decryption, providing back the SUPI to the requesting network element.

PMS shall receive the SUCI, resolve SUPI based on SUCI, and return SUPI after involving the appropriate PMS functions.

Access rights to the PMS shall be defined.

### 5.X.2 	Privacy handling function (PHF)

PHF is a function of PMS. PHF shall be the only contact point from network element to the PMS.

### 5.X.3	Privacy database (PDB)

The privacy database (PDB) shall be part of the PMS HSM. PDB shall hold one or several key identifiers and the related private keys used to protect the SUPI.

### 5.X.4	Key selection and key request (KRF)

The key selection and request function (KRF) shall be part of the PMS HSM. The KRF shall request the PDB for the private key.

### 5.X.3 	Decryption function (SIDF)

The SUPI decryption function (SIDF) shall be part of the PMS HSM.

<<< END OF CHANGE >>>