3GPP TSG-SA WG3 Meeting #91Bis 	S3-181999

# La Jolla (USA), 21- 25 May 2018                                                                revision of S3-181680

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | - | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Clause 6.6 UP Security mechanisms |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2018-03-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections based on, - SA2 agreement that the UE Security policy from the SMF to the gNB contains ( Required, Preferred, ). If the Policy is ‘Required’, gNB shall enable UP integrity and if it cannot be supported, the PDU session set up shall be rejected and informed to AMF and SMF. If the UE Security policy is ‘Preferred’, gNB shall enable UP integrity and if there sufficient resources to support it. - 6.6.2 UP security activation mechanism: Clarification added that, if the policy is ‘Preferred’ there may be change in activation or de-activation of UP integrity or UP confidentiality during handover. - “Editor’s Note: gNB action in case of UP security mismatch are FFS. “This EN can be deleted, the paragraph above this EN has been updated in the last SA3, and sufficiently describes the gNB action incase of policy mismatch. - “Editor's Note: The input parameters for confidentiality algorithm are FFS. “This EN can be deleted, the paragraph above this EN has been updated in the last SA3, and sufficiently describes how the gNB applies the confidentiality algorithm. - 6.6.4 UP integrity mechanisms, typo “ … a 5-bit bearer identity BEARER which value is assigned as specified …. corrected to “a 5-bit bearer identity BEARER value of which is assigned “ for clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As described above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.1, 6.6.2, 6.6.3, 6.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## ********************* Start of 1st change *************************

## 6.6	UP security mechanisms

Editor's Note: This sub-clause is meant to contain content corresponding to TS 33.401 [10], sub-clause 7.3, which is about UP security mechanisms.

### 6.6.1	UP security policy

The SMF shall provide UP security policy for a PDU session to the gNB during the PDU session establishment procedure as specified in TS 23.502 [8].

The UP security policy shall indicate whether UP confidentiality and/or UP integrity protection shall be activated or not for all DRBs belonging to that PDU session. The UP security policy shall be used to activate UP confidentiality and/or UP integrity for all DRBs belonging to the PDU session.

The gNB shall activate UP confidentiality and/or UP integrity protection per DRB, according to the received UP security policy, using RRC signalling as defined in clause 6.6.2. If the user plane security policy indicate “Required” or “Not needed”, the gNB shall not overrule the UP security policy provided by the SMF. If the gNB cannot activate UP confidentiality and/or UP integrity protection when the received UP security policy is “Required”, the gNB shall reject establishment of UP resources for the PDU Session and indicate reject-cause to the SMF.

NOTE: 	Local SMF can override the confidentiality option in the UP security policy received from the home SMF based on its local policy, roaming agreement and/or regulatory requirements.

At an Xn-handover from the source gNB to the target gNB, the source gNB shall include in the HANDOVER REQUEST message, the UE's UP security policy. If the UP security policy is ‘Required’, the target gNB shall reject all PDU sessions for which it cannot comply with and indicate the reject-cause to the AMF. For the accepted PDU sessions, the target gNB shall activate UP confidentiality and/or UP integrity protection per DRB according to the received UE's UP security policy and shall indicate that to the UE in the HANDOVER COMMAND by the source gNB.

If the UE receives an indication in the HANDOVER COMMAND that UP integrity protection and/or UP encryption for a PDU session is enabled at the target gNB, the UE shall generate the UP encryption key and/or UP integrity protection key and shall activate UP encryption and/or UP integrity protection for the respective PDU session.

NOTE2: If the security policy is ‘Preferred’, it is possible to have a change in activation or deactivation of UP integrity after the handover.

Further, in the Path-Switch message, the target gNB shall send the UE's UP security policy and corresponding PDU session ID received from the source gNB to the AMF. The AMF shall verify that the UE's UP security policy received from the target gNB are the same as the UE's UP security policy that the AMF has locally stored. If there is a mismatch, the AMF shall send its locally stored UE's UP security policy of the corresponding PDU sessions to the target gNB in the Path-Switch Acknowledge message. Additionally, the AMF may log the event and may take additional measures, such as raising an alarm.

If the target gNB receives UE's UP security policy from the AMF in the Path-Switch Acknowledge message, the target gNB shall update the UE's UP security policy with the received UE's UP security policy. If UE's current UP confidentiality and/or UP integrity protection activation is different from the received UE's UP security policy, then the target gNB shall initiate RRC Connection Reconfiguration procedure to reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the recived policy from AMF.

At an N2-handover the AMF shall include in the handover request message to the target gNB, the UE's UP security policy. The target gNB shall reject all PDU sessions for which it cannot comply with the corresponding received UP security policy and indicate the reject-cause to the target AMF. For all other PDU sessions, the target gNB shall activate UP confidentiality and/or UP integrity protection per DRB according to the received UE's UP security policy.

### 6.6.2	UP security activation mechanism

AS UP integrity protection and ciphering activation shall be done as part of the DRB addition procedure using RRC Connection Reconfiguration procedure as described in this clause, see Figure 6.6.2-1.

The SMF shall send the UP security policy

to the gNB as defined in Clause 6.6.1.

Figure 6.6.2-1: User plane (UP) security activation mechanism

1a.	This RRC Connection Reconfiguration procedure which is used to add DRBs shall be performed only after RRC security has been activated as part of the AS security mode command procedure defined in Clause 6.7.4.

1b.	The gNB shall send the RRC Connection Reconfiguration message to the UE for UP security activation containing indications for the activation of UP integrity protection and ciphering for each DRB according to the security policy.

1c.	If UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, the gNB shall generate KUPint and UP integrity protection for such shall start at the gNB  Similarly, if UP ciphering is activated for  DRBs as indicated in the RRC Connection Reconfiguration message, the gNB shall generate KUPenc and UP ciphering for such DRBs shall start at the gNB.

2a.	UE shall verify the RRC Connection Reconfiguration message. If successful:

2a.1	If UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, the UE shall generate KUPint and UP integrity protection for such DRBs shall start at the UE.

2a.2	Similarly, if UP ciphering is activated for DRBs as indicated in the RRC Connection Reconfiguration message, the UE shall generate KUPenc and UP ciphering for such DRBs shall start at the UE

2b.	If the UE successfully verifies integrity of the RRC Connection Reconfiguration message, the UE shallsend the RRC Connection Reconfiguration Complete message to the gNB.

If UP integrity protection is not activated for DRBs, the gNB and the UE shall not integrity protect the traffic of such DRB and shall not put MAC-I into PDCP packet.

If UP ciphering is not activated for DRBs, the gNB and the UE shall not cipher the traffic of such DRB.

### 6.6.3	UP confidentiality mechanisms

The PDCP protocol, as specified in TS 38.323 [23] between the UE and the 5G-RAN, shall be responsible for user plane data confidentiality protection.

The use and mode of operation of the 128-bit 5G-EA algorithms are specified in Annex B.

The input parameters to the confidentiality algorithms as described in Annex B are the message packet, an 128-bit cipher key KUPenc as KEY, a 5-bit bearer identity BEARER which value is assigned as specified by TS 38.323 [23], the 1-bit direction of transmission DIRECTION, the length of the keystream required LENGTH and a bearer specific, and direction dependent 32-bit input COUNT which corresponds to the 32-bit PDCP COUNT.

### 6.6.4	UP integrity mechanisms

The PDCP protocol, as specified in TS 38.323 [23] between the UE and the 5G-RAN, shall be responsible for user plane data integrity protection.

The use and mode of operation of the 128-bit 5G-IA algorithms are specified in Annex B.

The input parameters to the 128-bit 5G-IA algorithms as described in Annex B are, the message packet, a 128-bit integrity key KUPint as KEY, a 5-bit bearer identity BEARER value of which is assigned as specified by TS 38.323 [23], the 1-bit direction of transmission DIRECTION, and a bearer specific, and direction dependent 32-bit input COUNT which corresponds to the 32-bit PDCP COUNT.

If the gNB or the UE receives a PDCP PDU which fails integrity check with faulty or missing MAC-I after the start of integrity protection, the PDU shall be discarded.

.

## ********************* End of 1st change *************************