3GPP TSG-SA3 Meeting #98bis-e	S3-200785r1

- e-meeting, 14 – 17 April 2020

Title:	[DRAFT] Reply LS on AUSF role in slice specific authentication

Response to:	S2-1910668

Release:	Rel-16

Work Item:	eNS

Source:	SA3

To:	SA2

Cc:

Contact Person:

#### Name:	Zander Lei

Tel. Number:

E-mail Address:	lei.zhongding@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 thanks SA2 for the LS Response S2-1910668 on AUSF role in Network Slice Specific Authentication and Authorization (NSSAA). SA3 understands the motivation to include a contact point within the HPLMN for facilitating the execution of the NSSAA procedures using SBA services with the AMF and translating them to AAA protocols towards the AAA-S/AAA-P. SA3 also understands that this contact point can also assist the AAA-S to select the AMF serving the UE for the execution of the AAA-S initiated Re-authentication and Revocation procedures.

However, SA3 has a security requirement to fully isolate the following two SBA service types,

- primary authentication services towards UDM currently supported by AUSF

- NSSAA services involving interactions with an external AAA-S

SA3 have discussed two options to meet this security requirement for isolation:

- Option A: AUSF provides both primary authentication and NSSAA services, but in separate AUSF instances. 3GPP TSs shall mandate that implementation and deployment of the AUSF instances supporting the NSSAA services is different from the AUSF instances supporting the primary authentication services. From AMF point of view, this will require that AMF selects AUSF instances exposing primary authentication and NSSAA services separately. Thus, introducing new complexity for the AMF as an AUSF would only offer a subset of services.

- Option B: A new dedicated NF (e.g. AAA-IWF or NSSAAF) provides the NSSAA related services. From a specification point of view this will require the definition of a new NF within the SBA architecture. From AMF point of view, this requires that AMF selects the new NF in the HN following existing SBA principles similarly as AUSF or UDM are selected by AMF. The AMF logic of selecting NFs is preserved and the AUSF properly secured.

From a security and management point of view, Option B provides better isolation between the two services. However, this will introduce a new NF and may increase complexity in terms of architectural changes in the specifications. SA3 would like SA2 to take into consideration the above information and feedback on the most appropriate options.

2. Actions:

To SA2, CT3 and CT4 groups

ACTION: 	To take the above information into consideration and confirm on the most appropriate option to support NSSAA SBI related service from an architecture and security point of view.

3. Date of Next TSG-SA WG3 Meetings:

SA3#99-e	11 – 15 May 2020	Online

SA3#100	6 -10 July 2020	Bath, Great Britain