3GPP TSG-SA WG3 Meeting #98Bis-e 	S3-200647

- e-Meeting, 14-17 April 2020        										                    revision of S3-19abcd

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | Draft CR | Rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [33.180] R16 Fix XML references |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCSec |  |  |  |  |  |  | Date: |  | 2020-14-04 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | D.2.2 and D.3.1 reference D.3.5.2 which immediately references D.3.5.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Have D.2.2 reference D.3.5.1 directly. Have D.3.1 reference D.3.5.1 directly. Void clause D.3.5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconvenient |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.2, D.3.1, D.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************** start of 1st change *****************

## D.2.2	KMS request security

An optional security extension may be used to authenticate the KMS request from the client. To use the optional security extension, the POST request shall be accompanied with an XML payload MIME type containing details of the request, signed by the shared InK or TrK.

The content of the KMS Request Type XML payload is:

Table D.2.2-1: Contents of a KMS Request Type XML

| Name | Description |
| --- | --- |
| Version | (Attribute) The version number of the key provision XML (1.1.0). |
| UserUri | URI of the user for which is making the request. |
| KmsUri | The URI of the KMS to which the request is sent. |
| ClientId | (Optional) A string representing the client |
| DeviceId | (Optional) A string representing the device |
| Time | Date/time that the request is made by the client. |
| ClientReqUrl | The resource URI to which the HTTP POST request is sent. |
| KrrList | (Optional) Zero or more KMS Redirect Responses (KRRs). Only used when posting to the ‘redirect' subdirectory. |
| ClientError | (Optional) If a previous failure had occurred, this complex type can provide error information to the KMS |

This payload is signed using the InK or TrK. The XML schema for the SignedKmsRequestType is provided in Clause D.3.5.1.

If the KMS supports authenticated requests, upon receipt of a SignedKmsRequestType attached to a KMS Request, the KMS shall verify that:

-	the signature is valid, based on the UserUri and the InK or TrK used to sign the message.

-	the XML is valid.

-	the KmsUri is the KMS's KMS URI.

-	the Time is within a recent time window (e.g. 5 seconds).

-	the ClientReqUrl is the same as the resource URI to which the HTTP POST request is sent.

If so, the request is accepted and processed.

***************** end of 1st change *****************

***************** start of 2nd change *****************

## D.3.1	General

This clause defines the HTTP responses made by the KMS to KMS requests. The KMS attaches XML content to the HTTP responses. The XML serves to provision the client based upon its request.

Though a "KmsResponse" message containing a "KmsMessage" Type is the general response to any request, the content of the "KmsMessage" varies depending on the exact response type (i.e. KmsInit, KmsKeyProv, KmsCertCache, KmsLookup).

The content provided within a KmsInit, KmsKeyProv, KmsCertCache or KmsLookup may include a TrK, InK, KMS URIs, (public) KMS Certificates, (private) user Key Set provisioning, or combinations thereof.

The “KmsResponse” message is shown in Table D.3.1-1.

Table D.3.1-1: Contents of a "KmsResponse" message

| Name | Description |
| --- | --- |
| UserUri | URI of the user for which the response is intended. |
| KmsUri | The URI of the KMS sending the response. |
| KmsId | (Optional) The ID of the KMS providing the response message. |
| Time | Date/time that the response is sent by the KMS. |
| ClientReqUrl | The resource client URI from where the request originated. |
| KmsMessage | One of the following response types: KmsInit, KmsKeyProv, KmsCertCache, or KmsLookup. |

In response to a "KMS Initialize" request, the KMS shall respond with the KMS's own certificate (the Root KMS certificate), and may respond with a new TrK and/or a new InK. The data is returned within a "KMSInit" tag.

In response to a "KMS KeyProvision" request, the KMS shall provision appropriate user Key Sets within a "KMSKeyProv" tag, and may also respond with a new TrK and/or a new InK.

In response to a "KMS CertCache" request, the KMS shall provision a cache of KMS certificates allowing inter-domain communications within a "KMSCertCache" tag.

In response to a "KMS Cert" request, the KMS shall provision a single KMS certificate within a "KMSCertCache" tag. If the requested KMS Certificate is not available, then an error message is returned.

In response to a "KMS Lookup" request, the KMS shall provide information on the KMS URI associated with the requested SIP URI, within a "KMSLookup" tag.

The KMS does not respond to a "KMS Redirect Upload" message, unless an error occurs.

This payload may be signed using the InK or TrK. The XML schema for the SignedKmsRequestType is provided in Clause D.3.5.1.

***************** end of 2nd change *****************

***************** start of 3rd change *****************

### D.3.5.2	VOID

***************** end of 3rd change *****************