3GPP TSG-SA3 Meeting #98Bis-e 	draft-S3-200684-r2

- e-meeting, 14-17 April 2020

- Source:	ZTE Corporation

- Title:	Update clause 5.3.3.2.2

- Document for:	Approval

Agenda Item:	2.10

# 1	Decision/action requested

This pCR proposes to update clause 5.3.3.2.2

# 2	References

[1]	3GPP TS 33.536 v1.0.0: "Security aspect of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

# 3	Rationale

In clause 5.3.3.2.2 step1, MSB of KNRP-sess ID shall be included in Link Identifier Update Request message. Some minor editorial corrections.

# 4	Detailed proposal

This contribution proposes to update clause 5.3.3.2.2.

- **** START OF CHANGE 1 ****

##### 5.3.3.2.2	Procedures

Figure 5.3.3.2.2-1 shows the flows for changing the identities of the UEs involved in unicast communication. The figure and the following steps only show the security parameters (KNRP ID and KNRP-sess ID) that are changed and the Layer-2 ID but not the other parameters described in TS 23.287 [2].

Figure 5.3.3.2.2-1: Link identifier update procedure

The procedures proceeds with the following steps and give the additional processing on top of that given in TS 23.287 [2].

0. UE_1 and UE_2 are communicating via a unicast link and have established the security for the link.

1. UE_1 decides to change the identifiers and sends a Link Identifier Update Request message to UE_2 (see TS 23.287 [2]). UE_1 shall include the MSB of KNRP-sess ID  in the Link Identifier Update Request message. These bits shall be chosen so that they uniquely identify KNRP-sess at UE_1.

2. UE_2 shall choose the LSB of KNRP-sess ID so that they uniquely identify KNRP-sess  at UE_2. UE_2 shall form the new KNRP-sess ID from the MSB received from UE_1 and the LSB that UE_2 chose. UE_2 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_2 shall send the LSB of KNRP-sess ID to UE_1 along with the received MSB of KNRP-sess ID in the Link Identifier Update Response message. UE_1 shall check that the returned MSB of KNRP-sess ID are the same as it sent in step 1.

3. UE_1 shall form the new KNRP-sess ID from the LSB received from UE_2 and the MSB was chosen by UE_1(in step 1). UE_1 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_1 shall send the Link Identifier Update Ack message to UE_2 including the LSB of KNRP-sess ID. UE_2 shall check that the returned LSB of KNRP-sess ID are the same as it sent in step 2.

**** END OF CHANGES ****