- 3GPP TSG-SA3 Meeting #98Bis-e 	S3-200693r1

- e-meeting, 14-17 April 2020

- Source:	Huawei, Hisilicon

- Title:	eV2X: Clarification on the security policy provisioning

- Document for:	Approval

Agenda Item:	2.10

# 1	Decision/action requested

This contribution proposes to clarify the security policy provisioning.

# 2	References

[1]	3GPP TS 33.536 Security Aspect of 3GPP Support for Advanced Vehicle-to-Everything (V2X) Services

# 3	Rationale

This contribution proposes to clarify the security policy provisioning, i.e. the provisioned security policy for the list of V2X services shall be within the same application layer ID.

As described in the TS 33.536 clause 5.3.3.1.4.2.3, a new v2x service cannot be added to an existing connection in the following case, i.e.

When adding a V2X service to an existing connection, the UE responding to the request shall reject the request if signalling security in use does not match the policy for the new application

It means that all the services within the same connection has the same activated protection mechanism. Moreover, the PC5 unicast link is similar with the PDU session which we defined in Rel15. Hence, it is preferred to reuse the PDU session granularity security policy handling method in the PC5 unicast link. On the other hand, currently RAN2 and SA2 only supports the PC5 unicast granularity handling on the UP security protection.

Therefore, it is proposed to clarify that provisioned security of the list of V2X services within the NR PC5 unicast link identified by the same Application Layer ID shall be the same.

Furthermore, not only the signalling, but also the UP security shall be considered while a new V2X service is added to an existing connection. Therefore, clarification on the following text is required, i.e.

When adding a V2X service to an existing connection, the UE responding to the request shall reject the request if signalling security in use does not match the policy for the new application

# 4	Detailed proposal

It is proposed to approve the following change.

***	START OF THE CHANGES	***

5.3.3.1.4.2.3		Security policy handling

For a NR PC5 Unicast, the UE shall be provisioned with the following security policy:

The list of V2X services, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s) and their security policy which indicates the following:

•	Signalling integrity protection: REQUIRED/PREFERRED/OFF

•	Signalling confidentiality protection: REQUIRED/PREFERRED/OFF

•	User plane integrity protection: REQUIRED/PREFERRED/OFF

•	User plane confidentiality protection: REQUIRED/PREFERRED/OFF

NOTE 1: No integrity protection on signalling traffic enables services that do not require security, e.g. emergency services.

Editor’s note: Whether policy is OFF or NOT NEEDED is FFS

Editor’s note: Which granularity shall be used for UP protection activation is FFS.

The signalling integrity protection security policy being OFF means that the UE shall only establish a connection with no security. The signalling integrity protection security policy being PREFFERED means that the UE may try to establish security but will accept the connection with no security.. With the integrity protection security policy set to REQUIRED, the UE may only accept the connection if a non-NULL integrity algorithm is used for protection of the signalling traffic.

For the other cases, a setting of OFF means that the UE shall only use NULL confidentiality algorithm for that traffic or apply no integrity protection, while a REQUIRED setting means that the UE shall use a non-NULL algorithm. If the security policy is PREFERRED, then the UE may accept any algorithm for that particular protection. One use of PREFERRED is to enable a security policy to be changed without updating all UEs at once.

At initial connection, the UE include its signalling security policy in the Direct Communication Request message. The UE(s) responding to this take account of this when choosing the algorithms in the Direct Security Mode Command message. The initiating UE can reject the Direct Security Mode Command if the algorithm choice does not match its policy.

When adding a V2X service to an existing connection, the UE responding to the request shall reject the request if signalling security in use does not match the policy for the new application.

Editor’s note: Whether the request shall be reject if the UP data security in use does not match the policy for the new application is FFS.

The combination of security policies for UP Integrity Protection will result in the following activation of integrity protection:

Case 1: Both UP security policies indicate UP Integrity Protection "required", or one UP security policy indicates “required” and the other indicates “preferred”:

Activation of UP integrity protection for each user plane bearer individually of the service type when the PC5 unicast is established.

Case 2: Both UP security policies indicate UP Integrity Protection "preferred":

Activate or deactivate of UP integrity protection for each user plane bearer individually of the service type when the PC5 unicast is established based on local policy.

Case 3: For the other scenarios besides Case 1 and Case 2:

Deactivation of UP integrity protection for each user plane bearer individually of the service type when the PC5 unicast is established.

For UP Ciphering Protection, the resulting activation is the same as the UP integrity protection activation.

Editor’s note: The security policy handling related part needs to be clearly defined. It is FFS that how the initiating UE and the receiving UE deal with the security policy, e.g., whether to accept the communication or not with their security policy and local policy

*** END OF THE CHANGES	***