3GPP TSG-SA3 Meeting #98Bis-e 	S3-200715

- e-meeting, 14 – 17 April 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	Kakma ID format

- Document for:	Approval

Agenda Item:	2.4

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

NA

# 3	Rationale

This contribution specifies the elements of KAKMA identifier. For the RANDOM value part, we propose to add an Editor’s Note for further discussion.

# 4	Detailed proposal

***************** Start of Change*****************

## 6.1	Deriving AKMA key during UE registration

There is no separate authentication of the UE to support AKMA functionality. Instead, it reuses the 5G primary authentication procedure executed during the UE Registration to authenticate the UE. A successful 5G primary authentication results in KAUSF being stored at the AUSF and the UE.

Figure 6.1-1 Deriving AKMA root key during UE registration

The AUSF shall generate the AKMA Anchor Key (KAKMA) and the associated key Identifier from KAUSF as part of the UE Registration procedure.

The UE shall generate the AKMA Anchor Key (KAKMA) and the associated key Identifier from the KAUSF before initiating communication with an AKMA Application Function.

The KAKMA key identifier identifies the KAKMA key of the UE from which other AKMA keys are derived.

The KAKMA key identifier shall be NAI format as specified in clause 2.2 of IETF RFC 7542, i.e. username@realm. The usernamepart includes the Routing Identifer and the A-TID, and the realm part shall be identical to the realm part of the Network Specific Identifier.

Editor’s Note: the value of A-TID is FFS.

The key derivation of KAKMA shall be performed using the key derivation function (KDF) specified in TS 33.220 [4]. KAKMA is computed (as per Annex A.2) as KAKMA=KDF (KAUSF, "AKMA", SUPI), where the key derivation parameters consist of a static string "AKMA", and SUPI.Editor’s Note: Format and derivation of KAKMA key identifier and its association with UE identifier is FFS.

Editor’s Note: Whether the AUSF generates the KAKMA key identifier and the associated AKMA Anchor Key (KAKMA) during the primary authentication or as needed (i.e., on-demand), is FFS.

Since AKMA keys are based on KAUSF from primary authentication run, the AKMA keys can only be refreshed by running a fresh primary authentication.

***************** End of Change*****************