3GPP TSG-SA3 Meeting #98Bis-e 	S3-200769-r1

- e-meeting, 14-17 April 2020																Revision of S3-200365

- Source:	Qualcomm Incorporated

- Title:	pCR : Requirements on AKMA Key Identifier

- Document for:	Approval

Agenda Item:	2.4

# 1	Decision/action requested

This pCR proposes requirements for AKMA Key Identifier.

# 2	References

[1]	S3-200523 – AKMA draft TS 33.535 v0.3.0

# 3	Rationale

This pCR proposes Requirements for AKMA Key Identifier in the draft TS [1]. Note that the AKMA Key Identifier is analogous to the B-TID in GBA architecture.

Note that this is a revision of pCR in S3-200365 to implement the removal of AAnF from the 3rd requirement.

# 4	Detailed proposal

SA3 is kindly requested to approve the below pCR.

**** START OF CHANGES ****

### 4.4.x Requirements on AKMA Key Identifier

Requirements for AKMA Key Identifier (A-KID) are:

-	A-KID shall be globally unique;

-	A-KID shall be usable as a key identifier in protocols used in the reference point Ua*;

-	AKMA AF shall be able to identify the home network of the UE from the A-KID.

**** END OF CHANGES ****