3GPP TSG-SA3 Meeting #99-e	S3-200934

- Online, , 11th May 2020 - 15th May 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.210 | CR | 0068 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections to NDS/IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Juniper Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, DS_Backhaul |  |  |  |  |  | Date: |  |  | 2020-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR proposes editorial changes to tto NDS/IP, for example: - Removng 3G from the title (as was already done for NDS/AF) - Replacing “must” with “shall” - Repalcing “should” with “could” and “would” - Replacing “IPSec” with “IPsec” And on and on. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Comliance to TS21.801 and RFC4301. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A deviant specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Title, 5.2.1, 5.3.0, 5.3.5, A.1, A.2, B.2, B.3, C.2, D.1, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************* Start of change  **************************

3rd Generation Partnership Project;
Technical Specification Group Services and System Aspects;

Network Domain Security (NDS);
IP network security
(Release 16)

******************* Next change  **************************

5.2.1	Security Policy Database (SPD)

The Security Policy Database (SPD) is a policy instrument to decide which security services are to be offered and in what fashion.

The SPD shall be consulted during processing of both inbound and outbound traffic. This also includes traffic that shall not/need not be protected by IPsec. In order to achieve this, the SPD shall have unique entries for both inbound and outbound traffic such that the SPD can discriminate among traffic that shall be protected by IPsec, that shall bypass IPsec or that shall be discarded by IPsec.

The SPD plays a central role when defining security policies, both within the internal security domain and towards external security domains. The security policy towards external security domains will be subject to roaming agreements.

******************* Next change  **************************

### 5.3.0	General

This section gives an overview of the features of IPsec that are used by NDS/IP. The overview given here defines a minimum set of features that shall be supported. In particular, this minimum set of features is required for interworking purposes and constitutes a well-defined set of simplifications.

The accumulated effect of the simplifications is quite significant in terms of reduced complexity. This is achieved without sacrificing security in any way. It shall be noted explicitly that the simplifications are specified for NDS/IP and that they may not necessarily be valid for other network constellations and usages.

Within their own network, operators are free to use IPsec features not described in this section although there should be no security or functional reason to do so.

NOTE:	Clause 5.3 contains the general 3GPP IPsec ESP profile. Other 3GPP specifications (e.g. TS 33.203 [10], etc.) may point to clause 5.3. Thus parts of clause 5.3 may also apply to devices and network nodes as specified in other specifications. New specifications using ESP should refer to this profile with as few exceptions as possible.

******************* Next change  **************************

### 5.3.5	Requirements on the construction of the IV

The following strengthening of the requirements on how to construct the IV shall take precedence over the description given in RFC-2451 [24] section 3 and all other descriptions that allow for predictable IVs.

-	For CBC mode: the IV field shall be the same size as the block size of the cipher algorithm being used. The IV shall be chosen at random, and shall be unpredictable to any party other than the originator.

-	For CTR, GCM, CCM, and GMAC mode: the IV field shall be 8 octets. The IV shall be generated in a manner that ensures uniqueness. The same IV and key combination shall not be used more than once. -	It is explicitly not allowed to construct the IV from the encrypted data of the preceding encryption process.

The common practice of constructing the IV from the encrypted data of the preceding encryption process means that the IV is disclosed before it is used. A predictable IV exposes IPsec to certain attacks irrespective of the strength of the underlying cipher algorithm. The second bullet point forbids this practice in the context of NDS/IP.

These requirements imply that the network elements shall have a capability to generate random data. RFC 4086 [69] gives guidelines for hardware and software pseudorandom number generators.

******************* Next change  **************************

Annex A (informative):
Other issues

# A.1	Network Address Translators (NATs) and Transition Gateways (TrGWs)

Network Address Translators (NATs) are not designed to be part of the network domain control plane of NDS/IP-networks. Since network domain security employs a chained-tunnel approach it may be possible to use NATs provided that the network is carefully configured.

NDS/IP provides no explicit support for Transition Gateways (TrGWs) to be used in the network domain control plane of NDS/IP-networks, but the NDS/IP architecture will not itself prohibit the use of TrGWs. However, the inclusion of TrGWs needs to be carefully executed in order not to create interoperability problems.

# A.2	Filtering routers and firewalls

In order to strengthen the security for IP based networks, border gateways and access routers would normally use packet filtering strategies to prevent certain types of traffic to pass in or out of the network. Similarly, firewalls are used as an additional measure to prevent certain types of accesses towards the network.

The rationale behind the application of packet filters and firewalls could be found in the security policy of the network operator. Preferably, the security policy would be an integral part of the network management strategy as a whole.

While network operators are strongly encouraged to use filtering routers and firewalls, the usage, implementation and security policies associated with these are considered outside the scope of this document.

Simple filtering may be needed before the Security Gateway (SEG) functionality. The filtering policy allows key protocols such as DNS and NTP  to pass. This will include traffic over the Za interface from IKEv1/IKEv2 and IPsec ESP in tunnel mode. Unsolicited traffic is rejected.

******************* Next change  **************************

# B.2	Policy discrimination of GTP-C and GTP-U

It shall be possible to discriminate between GTP-C messages, which shall receive protection, and other messages, including GTP-U, that shall not be protected. Since GTP-C is assigned a unique UDP port-number in (TS29.060 [6]) IPsec can easily distinguish GTP-C datagrams from other datagrams that may not need IPsec protection.

Security policies shall be checked for all traffic (both incoming and outgoing) so datagrams can be processed in the following ways:

-	discard the datagram;

-	bypass the datagram (do not apply IPsec);

-	apply IPsec.

Under this regime GTP-U will simply bypass IPsec while GTP-C will be further processed by IPsec in order to provide the required level of protection. The SPD has a pointer to an entry in the Security Association Database (SAD) which details the actual protection to be applied to the datagram.

NOTE 1:	Selective protection of GTP-C relies on the ability to uniquely distinguish GTP-C datagrams from GTP-U datagrams. For R99 and onwards this is achieved by having unique port number assignments to GTP-C and GTP-U. For previous version of GTP this is not the case and provision of selective protection for the control plane parts of pre-R99 versions of GTP is not possible. Although NDS/IP was not designed for protection of pre-R99 versions of GTP, it is recognized that NDS/IP may also be used for protection of GTP pre-R99. It should be noted that NDS/IP support for pre-R99 versions of GTP is not mandatory.

NOTE 2:	NDS/IP has been designed to protect control plane protocols. However, it is recognized that NDS/IP may also be used to protect GTP-U. It should be noted that NDS/IP support for GTP-U is outside the scope of this specification.

# B.3	Protection of GTP-C transport protocols and interfaces

IPsec ESP shall be used with both encryption and integrity protection for all GTP-C messages traversing inter-security domain boundaries.

Gn and Gp control plane traffic shall be routed via a SEG when it takes place between different security domains. In order to do so, operators shall operate NDS/IP Za-interface between SEGs. If a network node has implemented SEG functionality within the same physical entity, transport mode IPsec is optional for implementation and use on the Gn and Gp interfaces.

It will be for the operator to decide whether and where to deploy Zb-interfaces in order to protect the GTP-C messages over the Gn and Gp interfaces within the same security domain.

******************* Next change  **************************

# C.2	Protection of IMS protocols and interfaces

IMS control plane traffic within the IMS core network shall be routed via a SEG when it takes place between different security domains (in particular over those interfaces that may exist between different IMS operator domains). In order to do so, IMS operators shall operate NDS/IP Za-interface between SEGs as described in clause 5.6.2.

When SEGs are deployed to secure a Za reference point potentially carrying IMS session keys (i.e. in IMS roaming scenarios, when SEGs are deployed between a P-CSCF and I-CSCF located in different security domains), IPsec ESP shall be used with both encryption and integrity protection for all SIP signalling traversing inter-security domain boundaries.

It will be for the IMS operator to decide whether and where to deploy Zb-interfaces in order to protect the IMS control plane traffic over those IMS interfaces within the same security domain.

******************* Next change  **************************

# D.1	The need for security protection

The control plane in question is used to transfer signalling messages in UTRAN/GERAN IP transport network. The UTRAN IP transport option is specified in Rel-5 UTRAN Technical Specifications. UTRAN Iu interface signalling transport is specified in 3GPP TS 25.412 [28] and Iur interface signalling transport in TS 25.422 [38]. The architecture for the UTRAN Iuh/Iurh interfaces is specified in 3GPP TS 25.467 [39], stage 3 specification is contained in 3GPP TS 25.468 [40] and TS 25.471 [41]. Based on the known security threats in IP networking, the traffic shall be protected properly. This is in order not to restrict the application of IP in UTRAN and GERAN only to closed network environments.

The security solution for IP based UTRAN/GERAN transport shall follow the principles introduced in the NDS/IP since the IPsec provides application independent security solution for all IP traffic.

Iu/Iuh and Iur/Iurh interfaces are carrying information that is classified as sensitive. Iu/Iuh and Iur/Iurh are used for conveying e.g. subscriber specific security keys. These keys are vital for the end-user security. Hence Iu/Iuh and Iur/Iurh shall be encrypted along with the integrity check.

# D.2	Protection of UTRAN/GERAN IP transport protocols and interfaces

IPsec ESP shall be used with both encryption and integrity protection for all RANAP and RNSAP messages traversing inter-security domain boundaries.

Iu/Iuh and Iur/Iurh control plane traffic shall be routed via a SEG when it takes place between different security domains (in particular over those interfaces that may exist between different operator domains). In order to do so, operators shall operate NDS/IP Za-interface between SEGs. If a UTRAN node has implemented SEG functionality within the same physical entity, transport mode IPsec is optional for implementation and use on the Iur/Iurh interface.

It will be for the operator to decide whether and where to deploy Zb-interfaces in order to protect the RANAP and RNSAP messages over the Iu/Iuh and Iur/Iurh interfaces within the same security domain.

******************* Next change  **************************

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New | WI |
| 03-2002 | SA_15 | SP-020117 | - | - | Approved at TSG SA#15 and placed under change control | 2.0.0 | 5.0.0 |  |
| 06-2002 | SA_16 | SP-020355 | 0001 |  | NDS/IP Confidentiality protection for IMS session keys | 5.0.0 | 5.1.0 |  |
| 06-2002 | SA_16 | SP-020356 | 0002 |  | Strengthening the requirements on IV construction to prevent attacks based on predictable IV | 5.0.0 | 5.1.0 |  |
| 12-2002 | SA_18 | SP-020719 | 0003 |  | Adding requirement to provide mandatory support for 3DES encryption in NDS/IP.Remove AES references and dependencies | 5.1.0 | 5.2.0 |  |
| 12-2002 | SA_18 | SP-020720 | 0004 |  | Securing UTRAN/GERAN IP Transport interfaces and specifically the Iu interface with NDS/IP mechanisms (Implemented after Rel-5 CR 003 included) | 5.1.0 | 6.0.0 | SECNDSIP |
| 03-2003 | SA_19 | SP-030104 | 0006 |  | Za-interface and roaming agreements | 6.0.0 | 6.1.0 | SECNDSIP |
| 03-2003 | SA_19 | SP-030105 | 0008 |  | Clarification to the re-keying aspects of network domain security | 6.0.0 | 6.1.0 | SECNDSIP |
| 06-2003 | SA_20 | SP-030225 | 0010 |  | Use of IPsec ESP with encryption on the Za-interface | 6.1.0 | 6.2.0 | SECNDSIP |
| 09-2003 | SA_21 | SP-030488 | 0012 |  | Change of IKE profiling | 6.2.0 | 6.3.0 | SECNDSIP |
| 09-2003 | SA_21 | SP-030489 | 0014 |  | Update draft-ietf-ipsec-sctp-04.txt reference to new standard RFC: RFC 3554 | 6.2.0 | 6.3.0 | SECNDSIP |
| 03-2004 | SA_23 | SP-040153 | 0015 | - | Addition of AES transform | 6.3.0 | 6.4.0 | SECNDSIP |
| 06-2004 | SA_24 | SP-040374 | 0016 | - | Diffie-Hellman groups in NDS/IP | 6.4.0 | 6.5.0 | SEC-NDS-IP |
| 2005-12 | SP- 30 | SP-050841 | 0017 | 2 | Extension of scope to encompass TISPAN NGN | 6.5.0 | 7.0.0 | FBI |
| 2006-09 | SP-33 | SP-060492 | 0019 | - | Clarifying the use of RFC3554 | 7.0.0 | 7.1.0 | SEC1-NDS |
| 2006-12 | SP-34 | SP-060808 | 0020 | 1 | Clarifying the use of transit security domains | 7.1.0 | 7.2.0 | SEC7-NDS |
| 2006-12 | SP-34 | SP-060808 | 0021 | 1 | Addition of reference to NDS/AF specification | 7.1.0 | 7.2.0 | SEC7-NDS |
| 2007-09 | SP-37 | SP-070590 | 0022 | 1 | Clarification on the use of the IPsec mode for the Zb-reference point | 7.2.0 | 7.3.0 | SEC1-NDS |
| 2008-03 | SP-39 | SP-080142 | 0024 | - | Introducing the support of IKEv2 for EPS | 7.3.0 | 8.0.0 | SAES |
| 2008-03 | SP-39 | SP-080142 | 0025 | 1 | Introducing the support of RFC-4303 for EPS | 7.3.0 | 8.0.0 | SAES |
| 2008-09 | SP-41 | SP-080544 | 0023 | 3 | Introduction of Network Domain Security support for 3GPP2 IMS | 8.0.0 | 8.1.0 | IMS-Sec |
| 2008-12 | SP-42 | SP-080747 | 0026 | - | Update of IKEv2 SA profile | 8.1.0 | 8.2.0 | TEI8 |
| 2009-06 | SP-44 | SP-090273 | 0027 | -- | Clarification about the encryption on Za reference point | 8.2.0 | 8.3.0 | TEI8 |
| 2009-12 | - | - | - | - | Update to Rel-9 version (MCC) | 8.3.0 | 9.0.0 | - |
| 2010-06 | SP-48 | SP-100251 | 0028 | - | Correction of explanations of abbreviations CSCF and IKEvx | 9.0.0 | 9.1.0 | TEI9 |
| 2010-10 | SP-49 | SP-100474 | 0029 | 2 | IPsec Alignment | 9.1.0 | 10.0.0 | TEI10 |
| 2010-10 | SP-49 | SP-100482 | 0031 | - | Clarification on usage of ESP authentication and encryption transforms | 9.1.0 | 10.0.0 | TEI10 |
| 2010-12 | SP-50 | SP-100731 | 0033 | - | NDS corrections | 10.0.0 | 10.1.0 | TEI10 |
| 2010-12 | SP-50 | SP-100833 | 0034 | 2 | Correction of IKEv2 references and IKE usage | 10.1.0 | 11.0.0 | TEI11 |
| 2011-03 | SP-51 | SP-110019 | 0036 | 1 | Correction of Iur security | 11.0.0 | 11.1.0 | TEI10 |
| 2011-03 | SP-51 | SP-110020 | 0038 | 1 | IKEv1 usage | 11.0.0 | 11.1.0 | TEI11 |
| 2011-06 | SP-52 | SP-110269 | 0039 | - | Clarification of algorithm names and DH group usage in IKEv2 | 11.1.0 | 11.2.0 | TEI10 |
| 2011-06 | SP-52 | SP-110264 | 0041 | - | Correction of Iuh/Iurh security | 11.1.0 | 11.2.0 | TEI11 |
| 2011-12 | SP-54 | SP-110848 | 0032 | - | Introduction of reference to RFC 4301 in overview clause | 11.2.0 | 11.3.0 | Sec11 |
| 2012-06 | SP-56 | SP-120338 | 0042 | 1 | Implementation requirements for IPsec authentication transforms | 11.3.0 | 12.0.0 | SEC12 |
| 2012-09 | SP-57 | SP-120605 | 0044 | - | Clarification of integrity and confidentiality requirements for GTP-C [Rel-12] | 12.0.0 | 12.1.0 | SEC11 |
| 2012-12 | SP-58 | SP-120856 | 0045 | 1 | Specification of missing IKEv2 reauthentication | 12.1.0 | 12.2.0 | SEC12 |
| 2015-12 | SP-70 | SP-150731 | 0046 | 1 | Updating IKEv2 profiles in TS 33.210 | 12.2.0 | 13.0.0 | SEC13 |
|  |  |  | 0047 | 1 | Updating ESP profiles in TS 33.210 |  |  |  |
|  |  |  | 0048 | - | Removing IKEv1 from TS 33.210 |  |  |  |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-12 | SA#74 | SP-160788 | 0049 | 1 | F | 3GPP security profile update – IPsec | 14.0.0 |
| 2018-06 | - | - | - | - | - | Update to Rel-15 version (MCC) | 15.0.0 |
| 2018-09 | SA#81 | SP-180706 | 0050 | 1 | B | Update NDS/IP scope with application layer crypto profiles | 15.1.0 |
| 2018-12 | SA#82 | SP-181022 | 0055 | - | F | Adding references for the TLS Protocol Profiles clause | 15.2.0 |
| 2018-12 | SA#82 | SP-181030 | 0056 | - | B | Update NDS/IP scope with application layer crypto profiles | 16.0.0 |
| 2019-03 | SA#83 | SP-190104 | 0057 | - | F | Correcting TLS crypto profiles | 16.1.0 |
| 2019-06 | SA#84 | SP-190354 | 0058 | 1 | F | Deprecation of TLS 1.1 | 16.2.0 |
| 2019-06 | SA#84 | SP-190354 | 0059 | - | F | References to several obsoleted RFCs | 16.2.0 |
| 2020-03 | SA#87E | SP-200143 | 0064 | 1 | B | ESP profile update | 16.3.0 |
| 2020-03 | SA#87E | SP-200143 | 0065 | - | B | TLS Recommended Cipher Suites | 16.3.0 |
| 2020-03 | SA#87E | SP-200143 | 0066 | 1 | B | Required TLS extenstions and algorithms | 16.3.0 |
| 2020-03 | SA#87E | SP-200143 | 0067 | - | B | IKEv2 profile update 33.210 | 16.3.0 |

******************* End of changes  **************************