3GPP TSG-SA WG3 Meeting #99-e	draft_S3-200939-r7

E-meeting, 11-15 May 2020                                                                           Revision of

- Source:	InterDigital

- Title:	Clarifications related to link identifier update messages

- Document for:	Approval

Agenda Item:	4.16

# 1	Decision/action requested

# 2	References

[1]	S3-200756

[2]	TS 33.536 v1.1.0

# 3	Rationale

This pCR is a revision of S3-200939 based on online and offline comments received during SA3#99-e. It attempts to resolve the misunderstanding between SA2, CT1, and SA3 Working Groups for completion of R16 specifications in these groups. More specifically, this pCR adds clarifications as to when and how is signalling protected for link identifier update.

# 4				Detailed proposal

It is proposed that SA3 approve the following pCR for inclusion in TS 33.536 [2].

**** BEGINNING OF CHANGES ****

#### 5.3.3.2 	Identity privacy for the PC5 unicast link

##### 5.3.3.2.1	General

The link identifier update procedure given in TS 23.287 [2] is used to provide privacy for the identities in the unicast link. This procedure only provides privacy if a non-NULL confidentiality algorithm is selected. This means the messages in this procedure are sent confidentiality protected (i.e. using a non-NULL confidentiality algorithm) and hence the new identities agreed by the UEs are only known to the involved UEs. A three-way message exchange procedure is required with this procedure since both UEs need to change their identifiers during the same procedure, to avoid trackability and linkability attacks, and to allow these new values to be acknowledged before them being used.

NOTE: From a security point of view, it is assumed that the link identifier update procedure is used with a protected connection.

**** END OF CHANGES ****