3GPP TSG-SA3 Meeting ad-hoc 	S3-200979

- e-meeting, 11 – 15 May 2020									      				Revision of S3-20xxxx

- Source:	CATT

- Title:	AKMA subscription data confirmation

- Document for:	Approval

Agenda Item:	4.10

# 1	Decision/action requested

It is proposed to add AKMA subscription data confirmation procedure before AAnF derives the KAF. SA3 is kindly requested to approve this contribution.

# 2	References

[1]	3GPP TS 33.535, v0.4.0

# 3	Rationale

For AKMA, the application key is provided by the operator. For the perspective of security control, the operator shall derive the KAF after checking whether the UE and the AF subscribe or use AKMA. It will relate to the privacy of the UE and the AF AKMA service agreement with the operator, as maybe not all the UEs or AFs support AKMA.

# 4	Detailed proposal

# *** START of CHANGE ***

## 6.2	Deriving AKMA Application key for a specific AF

Figure 6.2-1 AF Key generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF needs to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF.

Editor’s Note: It is FFS whether the capability to negotiate the use of AKMA between the UE application and the AKMA AF also needs to be supported.

1.	When the UE initiates communication with the AKMA AF, it shall include the derived A-KID in the Application Session Establishment request message (cf. clause 6.1).

2.	If the AF does not have an active context associated with the A-KID, then the AF sends a Naanf_key_create request to AAnF with the A-KID to request application function-specific AKMA keys for the UE.  The AF also includes its identity (AF Id) in the request.

3. The AAnF shall check whether the AAnF can provide the service to the AF by checking the AF Id. The AAnF shall check the AKMA subscription data and the AF authentication information stored locally. If the AKMA subscription data is not stored in the AAnF, the AAnF shall send AKMA service subscription confirmation request to the UDM to obtain the AKMA subscription data for the subscriber.

Editor’s Note: It is FFS how the AAnF get AF authorization information if the subscription data is not available locally.

4. If the UDM checks the AKMA is allowed for the subscriber, the UDM will response with the successful confirmation result to the AAnF and the AAnF stores this information locally. The following procedures are executed. Otherwise, the AAnF shall reject the procedure.

If the AAnF is in possession of the AF specific key (KAF), it responds to the AF with the KAF key. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the A-KID. The Step 3 to step 5 are skipped.

If KAKMA is available in AAnF, it shall derive the AF specific AKMA key (KAF) from KAKMA and respond to the AF with KAF and lifetime. The Step 3 and step 4 are skipped.

If KAKMA is not available, the procedure continued.

5.	The AAnF shall send a Nausf_AKMAkey_Create request to the AUSF to obtain the KAKMA key specific to the UE. It includes the A-KID in the request.

6.	The AUSF shall respond Nausf_AKMAkey_Create with the KAKMA key identified by the A-KID.

7.	The AAnF derives the AF specific key (KAF) from KAKMA.

8.	The AAnF sends Naanf_key_Create response to the AF with KAF and lifetime.

9.	The AF response the Application Session Establishment request to the UE.

Editor’s Note: Derivation of the AF specific KAF is FFS.

*** END of CHANGE***