3GPP TSG-SA WG3 Meeting #99e 	S3-201002

- E-meeting, 11-15 May 2020

- Source:	Apple

- Title:	pCR Clarification in Clause 6.2

- Document for:	Approval

Agenda Item:	4.10

# 1	Decision/action requested

Modificaiton to TS 33.535

# 2	Rationale

The Clause 6.2 is on the key request procedure when AF is located IN the operator’s network, while Clause 6.3 is describing the key request procedure when AF is located OUTSIDE of the operator’s network. This pCR is proposing to add some decribtion to make the difference between Clause 6.2 and 6.3 more clearer.

# 3	CHANGES

********************** First Change ****************************

## 6.2	Deriving AKMA Application key for a specific AF

Figure 6.2-1 shows the procedure used by the AF to request application function specific AKMA keys from 5GC directly, when the AF is located in the operator’s network.

Figure 6.2-1 AF Key generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF needs to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF.

Editor’s Note: It is FFS whether the capability to negotiate the use of AKMA between the UE application and the AKMA AF also needs to be supported.

1.	When the UE initiates communication with the AKMA AF, it shall include the derived A-KID in the Application Session Establishment request message (cf. clause 6.1).

2.	If the AF does not have an active context associated with the A-KID, then the AF sends a Naanf_key_create request to AAnF with the A-KID to request application function-specific AKMA keys for the UE.  The AF also includes its identity (AF Id) in the request. The AAnF shall check whether the AAnF can provide the service to the AF by checking the AF Id. If succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

Editor’s Note: It is FFS how the AAnF knows whether it can provide the service to a specific AF

If the AAnF is in possession of the AF specific key (KAF), it responds to the AF with the KAF key. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the A-KID. The Step 3 to step 5 are skipped.

If KAKMA is available in AAnF, it shall derive the AF specific AKMA key (KAF) from KAKMA and respond to the AF with KAF and lifetime. The Step 3 and step 4 are skipped.

If KAKMA is not available, the procedure continued.

3.	The AAnF shall send a Nausf_AKMAkey_Create request to the AUSF to obtain the KAKMA key specific to the UE. It includes the A-KID in the request.

4.	The AUSF shall respond Nausf_AKMAkey_Create with the KAKMA key identified by the A-KID.

5.	The AAnF derives the AF specific key (KAF) from KAKMA.

6.	The AAnF sends Naanf_key_Create response to the AF with KAF and lifetime.

7.	The AF response the Application Session Establishment request to the UE.

Editor’s Note: Derivation of the AF specific KAF is FFS.

********************** End of pCR********************