3GPP TSG-SA3 Meeting #99e 	S3-201011

- e-meeting, 11 – 15 May 2020															Revision of S3-20xxxx

- Source:	Ericsson

- Title:	UE configuration for non-3GPP access

- Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

Approve the proposed updates to the living CR in [1].

# 2	References

[1]	3GPP, S3-200818, "Living CR for 5WWC"

[2]		3GPP TS 33.501: "Security architecture and procedures for 5G system"

# 3	Rationale

According to the procedures in clause 7.1a of the living CR [1], the non-3GPP access networks, which are trusted, can be pre-configured in the USIM or UE.

This document proposes that the home network can configure the trust status of non-3GPP access networks to the UE by using the User Parameter Update mechanism.

It is proposed approve the proposed updates to the living CR in [1]

# 4	Detailed proposal

*** 1st CHANGE***

## 7.1a	Determining trust relationship in the UE

The non-3GPP access networks, which are trusted, can be pre-configured in the UE. Additionally, during primary authentication the UE may receive an indication whether the non-3GPP IP access is trusted or not.  If such an indication is sent it shall be sent by the AUSF as part of an EAP-AKA' request. If no such indication is received by the UE (for example if 5G-AKA is used for primary authentication), and there is no pre-configured information in the UE, the UE shall consider the non-3GPP IP access as untrusted. In case of pre-configured information and indication received as part of an EAP-AKA' request are in conflict, the received indication shall take precedence. If the USIM supports non-3GPP access networks service, the home network operator may configure in the USIM a list of trusted non-3GPP access networks. In case of pre-configured information in the UE, the list of trusted non-3GPP access networks pre-configured by the home network operator in the USIM shall take precedence over information pre-configured in the ME.

The home network operator may update the information of trusted non-3GPP access networks in the USIM or ME (if the USIM does not support non-3GPP access networks service) using the UE Parameter Update mechanism as defined in clause 6.15 of the present specification.

*** END OF CHANGES***