3GPP TSG-SA3 Meeting #99e 	S3-201011

- e-meeting, 11 – 15 May 2020															Revision of S3-20xxxx

- Source:	Ericsson

- Title:	UE configuration for non-3GPP access

- Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

Approve the proposed updates to the living CR in [1].

# 2	References

[1]	3GPP, S3-200818, "Living CR for 5WWC"

[2]		3GPP TS 33.501: "Security architecture and procedures for 5G system"

# 3	Rationale

According to the procedures in clause 7.1a of the living CR [1], the non-3GPP access networks, which are trusted, can be pre-configured in the USIM or UE.

This document proposes that the home network can configure the trust status of non-3GPP access networks to the UE by using the User Parameter Update mechanism.

It is proposed approve the proposed updates to the living CR in [1]

# 4	Detailed proposal

*** 1st CHANGE***

## 7.1a	Determining trust relationship in the UE

The non-3GPP access networks, which are trusted, can be pre-configured in the UE. Additionally, during primary authentication the UE may receive an indication whether the non-3GPP IP access is trusted or not.  If such an indication is sent it shall be sent by the AUSF as part of an EAP-AKA' request. If no such indication is received by the UE (for example if 5G-AKA is used for primary authentication), and there is no pre-configured information in the UE, the UE shall consider the non-3GPP IP access as untrusted. In case of pre-configured information and indication received as part of an EAP-AKA' request are in conflict, the received indication shall take precedence. If the USIM supports non-3GPP access networks service, the home network operator may configure in the USIM a list of trusted non-3GPP access networks. In case of pre-configured information in the UE, the list of trusted non-3GPP access networks pre-configured by the home network operator in the USIM shall take precedence over information pre-configured in the ME.

The home network operator may update the information of trusted non-3GPP access networks in the USIM or ME (if the USIM is not able to store the PLMN list) using the UE Parameter Update mechanism as defined in clause 6.15 of the present specification.

*** END OF CHANGES***