3GPP TSG-SA WG3 Meeting #99e 	draft_S3-201079-r1

- e-meeting, 11 – 15 May 2020                                                                      revision of S3-201079

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0816 | Rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AS rekeying failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  |  | Date: |  | 2020-05-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | On the Xn Handover, the AS level rekeying case is a new feature for 5G NR added in the SA3#88bis. Originally, in the AS rekeying scenario, it presents as the UE CONTEXT MODIFICATION procedure which is triggered by the AMF, and the NG-RAN node responds. During the Xn Handover, the Path Switch procedure is triggered by the NG-RAN node, and the AMF responds. Current method of Xn-handover procedure, as specified in TS 33.501, was designed considering an optimization to combining the Path Switch procedure with AS rekeying procedure during the Xn Handover. As dicussed in the related document S3-20XXXX, it no needs to send this response message in scenario 1a, 1b and 1d. While in other scenario (scenario 1c and scenario 2), it needs to send an acknowledge message to the AMF to inform the AS rekeying of the gNB is failure. Otherwise the AMF cannot or does not know when to another NAS security mode procedure when AS rekeying fails specified in subclause 6.9.4.4 of the TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add some description in subclause 6.9.2.3.2 to align with the context in subclause 6.9.4.4. Add an Editor’s note to illustrate “the appropriate response” needs to be specified by RAN3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will cause execution problems in the gNB implementations. The AMF does not know when to perform operations when AS rekeying fails on Xn-handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

##### 6.9.2.3.2	Xn-handover

In Xn handovers the source gNB/ng-eNB shall perform a vertical key derivation in case it has an unused {NH, NCC} pair. The source gNB/ng-eNB shall first compute KNG-RAN* from target PCI, its frequency ARFCN-DL/EARFCN-DL, and either from currently active KgNB in case of horizontal key derivation or from the NH in case of vertical key derivation as described in Annex A.11/A.12.

Next, the source gNB/ng-eNB shall forward the { KNG-RAN*, NCC} pair to the target gNB/ng-eNB. The target gNB/ng-eNB shall use the received KNG-RAN* directly as KgNB to be used with the UE. The target gNB/ng-eNB shall associate the NCC value received from source gNB/ng-eNB with the KgNB. The target gNB/ng-eNB shall include the received NCC into the prepared HO Command message, which is sent back to the source gNB/ng-eNB in a transparent container and forwarded to the UE by source gNB/ng-eNB.

When the target gNB/ng-eNB has completed the handover signalling with the UE, it shall send a NGAP PATH SWITCH REQUEST message to the AMF. Upon reception of the NGAP PATH SWITCH REQUEST, the AMF shall increase its locally kept NCC value by one and compute a new fresh NH from its stored data using the function defined in Annex A.10. The AMF shall use the KAMF from the currently active 5G NAS security context for the computation of the new fresh NH. The AMF shall then send the newly computed {NH, NCC} pair to the target gNB/ng-eNB in the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message. The target gNB/ng-eNB shall store the received {NH, NCC} pair for further handovers and remove other existing unused stored {NH, NCC} pairs if any.

If the AMF had activated a new 5G NAS security context with a new KAMF, different from the 5G NAS security context on which the currently active 5G AS security context is based, but has not yet successfully performed a UE Context Modification procedure, the sent NGAP PATH SWITCH REQUEST ACKNOWLEDGE message shall in addition contain a NSCI (New Security Context Indicator). The AMF shall in this case derive a new initial KgNB from the new KAMF and the uplink NAS COUNT in the most recent NAS Security Mode Complete message as specified in Annex A.9. The AMF shall associate the derived new initial KgNB with a new NCC value equal to zero. Then, the AMF shall use {the derived new initial KgNB, the new NCC value initialized to zero} pair as the newly computed {NH, NCC} pair to be sent in the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message. The gNB/ng-eNB shall in this case set the value of keySetChangeIndicator field to true in further handovers. The gNB/ng-eNB should in this case perform an intra-gNB-CU/intra-ng-eNB handover immediately and send appropriate response to the AMF similar to the response in UE context modification procedure.

NOTE 1:	Because the NGAP PATH SWITCH REQUEST message is transmitted after the radio link handover, it can only be used to provide keying material for the next handover procedure. Thus, for Xn-handovers key separation happens only after two hops because the source gNB/ng-eNB knows the target gNB/ng-eNB keys. The target gNB/ng-eNB can immediately initiate an intra-gNB-CU/intra-ng-eNB handover to take the new NH into use once the new NH has arrived in the PATH SWITCH REQUEST ACKNOWLEDGE message.

NOTE 2: The key derivation mechanism described in this clause is also applicable to CHO defined in TS 38.300[52].

**** END OF CHANGES ****