3GPP TSG-SA3 Meeting #99e 	S3-201107

- e-meeting, 11 -15 May 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	pCR for 33.818: Security requirement for type 1 GVNP

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

This contribution proposes to add interface security requirements and a few editorial fixes.

# 2	References

# 3	Rationale

This contribution proposes to:

- add security requirements on the interface defined by ETSI for type 1 GVNP; and

- add isolation requirements on VNF management traffic.

Also, a few editorial changes are proposed including format fixes, reference fixes, adding abbreviation, etc.

# 4	Detailed proposal

SA3 is kindly requested to agree with the pCR for 33.818 below.

*********************************  1st change *****************************************

#### 5.2.5.5 Security functional requirements and related test cases for GVNP of type 1

##### 5.2.5.5.1 Introduction

The present clause describes the security functional requirements and the corresponding test cases, independent of a specific virtualised network product class of type 1. According to security threats and security requirements in the above clauses, there are threats relating to ETSI-defined interfaces and Security functional requirements related to virtualization layer, hardware and resource isolation etc. (ref. clause 5.2.4.2.2 and clause 5.2.5.2). So, the proposed security requirements for GVNP of type 1 are classified in three groups:

-	Security functional requirements deriving from 3GPP specifications and detailed in clause 5.2.5.5.2

-	General security functional requirements which include requirements not already addressed in the 3GPP specifications but whose support is also important to ensure a network product conforms to a common security baseline detailed in clause 5.2.5.5.3, clause 5.2.5.5.4, clause 5.2.5.5.5 and clause 5.2.5.5.6.

- Security functional requirements related to virtualization layer, hardware and resource isolation, among others.  These requirements can be called security functional requirements deriving virtualisation for simplify and detailed in clause 5.2.5.5.7.

The threat cooperation between GVNP of type 1 and physical network products are summarized in clause 5.2.4.3.2.10. Except threats relating to ETSI-definer interfaces, other threat categories can apply to threat categories for GVNP of type 1. So, the security requirements of the above first and second group will base on the security requirements in clause 4.2 of TS 33.117 [4] to identify the different security requirements for GVNP of type 1.

Editor’s note: whether the security functional requirements and related test cases of 3GPP virtualized network product classes are to be contained in TS 33.117 [4] or not is FFS.

##### 5.2.5.5.2 Security functional requirements deriving from 3GPP specifications and related test cases

- 5.2.5.5.2.1 Security functional requirements deriving from 3GPP specifications – general approach

The clause 4.2.2.1 in TS 33.117 [4] describes the general approach taken towards security functional requirements deriving from 3GPP specifications and the corresponding test cases, independent of a specific network product class. The general approach is generic and applies to security functional requirements deriving from 3GPP specifications and the corresponding test cases of GVNP type 1.

##### 5.2.5.5.3 Technical baseline for general security functional requirements

- 5.2.5.5.3.1 Introduction

The technical baseline is a generic set of security requirements to be fulfilled by all virtualized network products.

In particular these requirements counter the security threats identified in clause 5.2.4.2.2 and they basically aim to guarantee the network product confidentiality, integrity and availability.

- 5.2.5.5.3.2 Protecting data and information

All text from TS 33.117 [4], clause 4.2.3.2 applies to GVNP of type 1.

- 5.2.5.5.3.3 Protecting availability and integrity

- 5.2.5.5.3.3.1 System handling during overload situations

All text from TS 33.117 [4], clause 4.2.3.3.1 applies to GVNP of type 1.

- 5.2.5.5.3.3.2 Boot from intended memory devices only

All text from TS 33.117[4], clause 4.2.3.3.2 applies to GVNP of type 1.

- 5.2.5.5.3.3.3 System handling during excessive overload situations

All text from TS 33.117 [4], clause 4.2.3.3.3 applies to GVNP of type 1.

- 5.2.5.5.3.3.4 System robustness against unexpected input

All text from TS 33.117 [4], clause 4.2.3.3.4 applies to GVNP of type 1.

- 5.2.5.5.3.3.5 Virtualized Network product software package integrity

All text from TS 33.117 [4], clause 4.2.3.3.5 applies to GVNP of type 1.

In addition, VNF package and VNF image integrity shall be validated when on board, and VNF image integrity shall be validated when in instantiated. The detailed security requirements and related test cases are as following.

5.2.5.5.3.3.5.1 VNF package and VNF image integrity

Requirement Name: VNF package and VNF image integrity

Requirement Description:

1) VNF package and image shall contain integrity validation value (e.g. MAC).

2) VNF package shall be integrity protected prior to on board. NFVO shall validate the VNF package integrity.

Editor’s note: The related test case will be added later.

- 5.2.5.5.3.4 Authentication and authorization

All text from TS 33.117 [4], clause 4.2.3.4 applies to virtualized network products.

- 5.2.5.5.3.5 Protecting sessions

All text from TS 33.117 [4], clause 4.2.3.5 applies to virtualized network products.

- 5.2.5.5.3.6 Logging

All text from TS 33.117 [4], clause 4.2.3.6 applies to virtualized network products.

- 5.2.5.5.3.7 Security functional requirement related to virtualization

Editor's Note: Threat analysis for each of the requirements and correspoing test cases below is to be added.

- 5.2.5.5.3.7.1 Integrity protection of the interface between VNF and VNFM

Requirement Name: Integrity protection of the interface between VNF and VNFM

Requirement Description:

The communication between VNF and VNFM shall be integrity protected.

- 5.2.5.5.3.7.2 Confidentiality protection of the interface between VNF and VNFM

Requirement Name: Confidentiality protection of the interface between VNF and VNFM

Requirement Description:

The communication between VNF and VNFM shall be confidentiality protected.

5.2.5.5.3.7.3 Replay protection of the interface between VNF and VNFM

Requirement Name: replay protection of the interface between VNF and VNFM

Requirement Description:

The communication between VNF and VNFM shall be replay protected.

Editor's Note: Test cases are to be added.

##### 5.2.5.5.4 Operating systems

All text from TS 33.117 [4], clause 4.2.4 is generic and applies to guest operating systems for GVNP of type 1.

##### 5.2.5.5.5 Web servers

All text from TS 33.117 [4], clause 4.2.5 applies to GVNP of type 1.

##### 5.2.5.5.6 Network devices

All text from TS 33.117 [4], clause 4.2.6 applies to GVNP of type 1.

#### 5.2.5.x Security requirements and related test cases to Hardening for GVNP of type 1

##### 5.2.5.x.y1 Introduction

The requirements proposed in the present clause aim to securing network products (including the network functions in service-based architecture) by reducing its surface of vulnerability. In particular the identified requirements aim to ensure that all the default network product configurations (including operating system software, firmware and applications) are appropriately set.

##### 5.2.5.x.y2 Technical Baseline

##### 5.2.5.x.y3 Operating System

###### 5.2.5.x.y3.z Genertic operating system requirements and test cases

##### 5.2.5.x.y4 Web Severs

##### 5.2.5.x.y5 Network Devices

###### 5.2.5.x.y5.z Separation of inter-VNF and intra-VNF traffic

Requirement Name: inter-VNF and intra-VNF Traffic Separation

Requirement Description:

The network used for the communication between the VNFCs of a VNF and the network used for the communication between VNFs shall be separated.

Editor’s Note: Threat analysis for this requirement and corresponding the test cases is to be added. A figure illustrating the scenario needs to be upded.

*********************************  End of 1st change *****************************************

***************************************   2nd Change    *************************************

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]  3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]  3GPP TR 33.916: "Security Assurance Methodology (SCAS) for 3GPP network products"

[3]  3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes"

[4]  3GPP TR 33.117: "Catalogue of general security assurance requirements"

[5] 3GPP TS 28.500: "Management concept, architecture and requirements for mobile networks that include virtualized network functions"

[6] ETSI GS NFV-SEC 001: "Network Functions Virtualisation (NFV); NFV Security; Problem Statement"

[7] GSMA FS.16: "Network Equipment Security Assurance Scheme – Development and Lifecycle Security Requirements"

[8] ETSI GR NFV-SEC 007: "Functions Virtualisation (NFV); Trust; Report on Attestation Technologies and Practices for Secure Deployments"

[9]3GPP TR 33.848: "Study on security impacts of virtualisation"

[10] 3GPP TR 33.805: "Study on security assurance methodology for 3GPP network products (Release 12) "

[11] ETSI GS NFV 002: " Network Functions Virtualisation (NFV); Architectural Framework"

[12] ETSI GS NFV-EVE 001: “Network Functions Virtualisation (NFV); Virtualisation technologies; Hypervisor Domain Requirements Specification”

[13] ETSI GS NFV-IFA008: "Network Functions Virtualisation (NFV); Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification"

[14] ETSI GS NFV-IFA019: "Network Functions Virtualisation (NFV); Acceleration Technologies; Acceleration Resource Management Interface Specification"

*************************************** End of  2nd Change    *************************************

***************************************    3nd change   ******************************************

- 5.2.4.3.2.10 Summary of threats for GVNP of type 2

The threats for GVNP of type 1 can be compared to TR 33.926[3] and summarized as following:

| Threat Category | Detailed threat | Comparison to TR33.926[3] |
| --- | --- | --- |
| Threats relating to 3GPP-defined interfaces | - | All threats can be applied. |
| Threats relating to ETSI-defined interfaces | - | New threats: - The threats on interface between 3GPP VNF and VNFM - The threats on interface between 3GPP VNF and virtualisation layer - The threats on interface between virtualisation layer and VNF - The threats on interface between virtualisation layer and hardware - The threats on interface between virtualisation layer and VIM |
| Spoofing identity | Default Accounts | Threats can be applied with difference that access through VNC instead of physical console interface. |
|  | Weak Password Policies | Same as above. |
|  | Password peek | Same as above. |
|  | Direct Root Access | Threats can be applied. |
|  | IP Spoofing | Threats can be applied with difference that objective is VNF and virtuliasation layer rather than computer. |
|  | Malware | Threats can be applied. |
|  | Eavesdropping | Threats can be applied. |
| Tampering | Software Tampering | Threats can be applied. |
|  | Ownership File Misuse | Threats can be applied. |
|  | Boot tampering for GVNP of type 2 | Different threats. See detail in clause 5.2.4.3.2.5.3. |
|  | Log Tampering | Threats can be applied. |
|  | OAM traffic Tampering | Threats can be applied. |
|  | File Write Permissions Abuse | Threats can be applied. |
|  | User Session Tampering | Threats can be applied. |
| Repudiation | Lack of User Activity Trace | Threats can be applied. |
| Information disclosure | - | All threats can be applied. |
| Denial of Service | - | Different threats. See detail in clause 5.2.4.3.2.8. |
| Elevation of privilege | - | All threats can be applied. |

*******************************End of 3rd change *********************************************

***************************************    4th change   ******************************************

- 5.2.4.3.2.10 Summary of threats for GVNP of type 2

The threats for GVNP of type 2 can be compared to TR 33.926[3] and summarized as following:

| Threat Category | Detailed threat | Comparison to TR33.926[3] |
| --- | --- | --- |
| Threats relating to 3GPP-defined interfaces | - | All threats can be applied. |
| Threats relating to ETSI-defined interfaces | - | New threats: - The threats on interface between 3GPP VNF and VNFM - The threats on interface between 3GPP VNF and virtualisation layer - The threats on interface between virtualisation layer and VNF - The threats on interface between virtualisation layer and hardware - The threats on interface between virtualisation layer and VIM |
| Spoofing identity | Default Accounts | Threats can be applied with difference that access through VNC instead of physical console interface. |
|  | Weak Password Policies | Same as above. |
|  | Password peek | Same as above. |
|  | Direct Root Access | Threats can be applied. |
|  | IP Spoofing | Threats can be applied with difference that objective is VNF and virtualisation layer rather than computer. |
|  | Malware | Threats can be applied. |
|  | Eavesdropping | Threats can be applied. |
| Tampering | Software Tampering | Threats can be applied. |
|  | Ownership File Misuse | Threats can be applied. |
|  | Boot tampering for GVNP of type 2 | Different threats. See detail in clause 5.2.4.3.2.5.3. |
|  | Log Tampering | Threats can be applied. |
|  | OAM traffic Tampering | Threats can be applied. |
|  | File Write Permissions Abuse | Threats can be applied. |
|  | User Session Tampering | Threats can be applied. |
| Repudiation | Lack of User Activity Trace | Threats can be applied. |
| Information disclosure | - | All threats can be applied. |
| Denial of Service | - | Different threats. See detail in clause 5.2.4.3.2.8. |
| Elevation of privilege | - | All threats can be applied. |

*******************************End of 4th change *********************************************

******************************    5th change ***********************************************

- 5.2.4.4.2.10 Summary of threats for GVNP of type 3

The threats for GVNP of type 3 can be compared to TR 33.926[3] and summarized as following:

| Threat Category | Detailed threat | Comparison to TR33.926[3] |
| --- | --- | --- |
| Threats relating to 3GPP-defined interfaces | - | All threats can be applied. |
| Threats relating to ETSI-defined interfaces | - | New threats: - The threats on interface between 3GPP VNF and VNFM - The threats on interface between 3GPP VNF and virtualisation layer - The threats on interface between virtualisation layer and VNF - The threats on interface between virtualisation layer and hardware - The threats on interface between hardware and virtualisation layer - The threats on interface between virtualisation layer and VIM - The threats on interface between hardware and Virtualised Infrastructure Manager (VIM) |
| Spoofing identity | Default Accounts | Threats can be applied with difference that access through VNC instead of physical console interface. |
|  | Weak Password Policies | Same as above. |
|  | Password peek | Same as above. |
|  | Direct Root Access | Threats can be applied. |
|  | IP Spoofing | Threats can be applied with difference that objective is VNF and virtualisation layer in addition to computer. |
|  | Malware | Threats can be applied. |
|  | Eavesdropping | Threats can be applied. |
| Tampering | Software Tampering | Threats can be applied. |
|  | Ownership File Misuse | Threats can be applied. |
|  | Boot tempering for GVNP of type 3 | Different threats. See detail in clause 5.2.4.4.2.5.3. |
|  | Log Tampering | Threats can be applied. |
|  | OAM traffic Tampering | Threats can be applied. |
|  | File Write Permissions Abuse | Threats can be applied. |
|  | User Session Tampering | Threats can be applied. |
| Repudiation | Lack of User Activity Trace | Threats can be applied. |
| Information disclosure | - | All threats can be applied. |
| Denial of Service | - | Different threats. See detail in clause 5.2.4.4.2.8. |
| Elevation of privilege | - | All threats can be applied. |

*******************************End of 5th change *********************************************

******************************    6th change ***********************************************

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

VNF	Virtualized Network Function

VNFM	Virtualized Network Function Manager

VNFC	Virtualized Network Function Component

*************************************** End of  6tht Change    *************************************