3GPP TSG-SA3 Meeting #99e 	S3-201166r1 (merging 1023 and 0988)

- e-meeting, 11 -15 May 2020

Title:	[DRAFT] LS on False Base Station Detection

Response to:	R2-1914224

Release:	Rel-16

Work Item:	FS_5GFBS

Source:	SA3

To:	RAN2

Cc:	RAN3

Contact Person:

#### Name:	Zander Lei

Tel. Number:

E-mail Address:	lei.zhongding@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 would like to thank RAN2 for the analysis and considerations. With respect to the questions raised, SA3’s answers are as follows.

Q1: How frequently the network will request UE to report the hashes of system information which will bring impact on UE power consumption?

A1:

SA3 does not have any requirement on the frequency of reporting the hashes of system information. SA3 assumes that reporting the hashes of system information is performed as part of a normal measurement report procedure and no additional reading of MIB/SIBs by the UE should happen (see answer 2 for details).

Q2: For the hash of MIB/SIBs, does SA3 assume this is only for MIB and SIB1, or also for other SIBs? Does SA3 assume this mechanism applies for both idle/inactive mode and connected mode?

A2:

SA3 assumes that the mechanism is supported in general for all RRC states. Specifically,

- Idle/Inactive mode: UE

1) monitors the camped cell the following information: MIB, SIB1 and any other SIBs that UE is interested in;

2) stores in the logged MR the hashes of the monitored MIB, SIB1 and any other SIBs if read

- Connected mode: UE

1) reports the logged MR;

2) reports, as part of measurement report, the hashes of MIB and SIB1 if they are already available in the UE.

Q3: For the additional information to be included in the logged MDT results, what additional information is required by SA3, compared with the existing parameters defined in NR measurement reporting (including SON/MDT)?

A3:

SA3 has not concluded any additional information to be included in the logged MDT.

SA3 would like to inform RAN2 that SA3 has not concluded the key issue on the enriched measurement report.

2. Actions:

To RAN2 group.

ACTION: SA3 kindly requests RAN2 to take the above answers into account.

3. Date of Next TSG-SA WG3 Meetings:

SA3#100e	6 -10 July 2020	e-meeting

SA3#100e-Bis	17 – 21 August 2020	e-meeting