- 3GPP TSG-SA3 Meeting #99e	S3-201167

- e-meeting, 11 -15 May 2020	revision of S3-20xxxx

- Source:	Huawei, HiSilicon

- Title:	Update solution 4 for clarification

- Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

This pCR adds the details of the MIB/SIBs Hashes reporting in solution #4 for clarification

# 2	References

# 3	Rationale

To clarify the solution 4, more details are provided on UE behaviour when in the idle/connected mode.

# 4	Detailed proposal

**** Start of Changes ****

## 6.4	Solution #4: Enriched measurement reports

### 6.4.1	Introduction

This solution addresses the first security requirement in the following key issue:

-	Key issue #3: network detection of false base stations (first requirement).

The solution provides a mechanism for enhancing the detection of false base stations by enriching the measurement reports from the UE. The solution is applicable to UEs in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states. Specifically,

- When UE is in the Idle/Inactive mode, UE

1) monitors the camped cell the following information: MIB, SIB1 and any other SIBs (if available) that UE is interested in;

2) stores in the logged MR the hashes of the monitored MIB, SIB1 and other SIBs (if available) read

- When UE is in the Connected mode, UE

1) reports the logged MR;

2) measures neighbouring cells’ MIB and SIB1 (if CGI info is requested), and reports , as part of measurement report, the hashes of the measurement MIB and SIB1 if they are already available in the UE.

### 6.4.2	Solution details

#### 6.4.2.1	Enrichment of measurement report

The UE measurement reports specified in 3GPP TR 38.331 [2] already contain several information relevant for the detection of false base stations, e.g., identifier and received-signal strength information of the cell. Additionally, 3GPP TS 38.331 provides support for reporting of Cell Group Info (CGI_info) which contains information broadcasted in MIB and SIB1.

In addition to the existing information, the system shall support the following new information about camped and neighboring cells to be included in the measurement report:

-	mib_info = hash of the MIB, which helps in detection of DoS attempts, e.g., cellBarred=barred;

-	sib_info = list of {SIB number, hash of the SIB}, which helps in detection of DoS, fraud, and subscription identification attempts, e.g., ims-EmergencySupport=false, tampered SI-SchedulingInfo, and useFullResumeID=true;

NOTE 1:	The sib_info could contain at least SIB1 which the UE currently obtains to generate CGI_info.

NOTE 2:	The mib_info and sib_info could contain their corresponding recorded time.

NOTE 3:	The network can set the requesting frequency for CGI info, similar to the ANR frequency configuration. That is implementation specific.

It’s suggested to reuse the algorithm of SHA256 to calculate the hashes of MIB/SIBs.

In addition to the information above which can be transmitted by UEs in CONNECTED mode, it is also possible to extend the logged measurements, currently discussed in TS 38.331 for Rel-16, to include the following information that can help in detecting false base stations:

-	reject_info = information about REJECTs that the UE had received earlier, which helps in detection of DoS attempts, e.g., presence of rogue REJECTs;

-	signal_info = information about signal as below, which helps in detection of DoS attempts, e.g., presence of erratic radio signals:

-	just power: signal is not associated with any normal pilots or reference signals.

-	power with just pilots: signal is associated with normal pilots or reference signals, but those signals do not provide any readable system information.

-	power with pilots and system info: signal is associated with normal pilots or reference signals and those signals provide system information, but the system information is wrong (e.g., inconsistent information, not possible to access the network according to the information).

The UE shall send the above information to the network only after a successfull AS security mode command procedure.

The network shall verify and compare the above information with what is expected, e.g., comparing hashes of MIB/SIBs for reported cells with those of genuine cells using the same hash algorithm used in the UE.

To verify the hashes of MIB/SIBs reported by the UE in the MR, the gNB should store the hashes of all broadcasted MIB/SIBs of all cells that belong to the gNB and records the changed time when the MIB/SIBs are modified.

#### 6.4.2.2	Verification of the MIB/SIBs Hashes

When the serving cell receives a MR/logged MR with hashes of neighbouring cell MIB/SIBs, the serving cell can report the MR/logged MR to OAM, then the OAM communicates with the corresponding neighbouring cells to do the MIB/SIBs hashes verification; the serving cells also can communicate with the OAM to get the corresponding neighbouring cell’s MIB/SIBs hashes, and verify the reported MIB/SIBs hashes in local. The communication between OAM and cells are implementation specific.

If verification fails, e.g., the hashes do not match, the network may consider it as a factor to detect the presence of the false base station.

During handovers, the serving cell may use the above information, i.e., the MIB/SIBs hashes verification result, to decide whether or not to attempt handover of the UE to the reported neighbouring cell.

### 6.4.3	Evaluation

Editor's Note:	Impacts on UE power consumption is FFS.

Enriched measurement reports certainly help in enhancing the detection of false base stations.

NOTE:	It is not in the merit of SA3 alone to define new information to be included in measurement reports. SA3 could give directions to RAN groups about information helpful for false base station detection. Then, RAN groups would design solutions. Hence, RAN groups must be liaised.

**** End of Changes ****