3GPP TSG-SA3 Meeting #99e 	S3-201172

- e-meeting, 11 -15 May 2020

- Source:	KPN

- Title:	Key issue on Protection of the Milenage OP value during storage in UDR

- Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is requested to approve the key issue on protection of the Milenage OP value during storage in UDR

# 2	References

[1]	3GPP TS 35.205: " Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*;Document 1: General"

# 3	Rationale

In case of separation of storage and functionality in, respectively, UDR and UDM/ARPF, there is a need to investigate solutions for the protection of sensitive credential data in the UDR. This contribution introduces a key issue focussing on the protection of the Milenage OP value (see [1]) during storage in the UDR.

# 4	Detailed proposal

START OF CHANGE 1

## 6.x	Key Issue #<Y>: protection of Milenage OP value during storage in UDR

### 6.x.1	Key issue details

In case the Milenage OP value, which can be used to generate the OPc value that is part of the authentication subscription data needed to generate authentication vectors in the UDM/ARPF – in case Milenage [3] is used – is stored in the UDR separate from the UDM/ARPF, then this value needs to be protected. This key issue addresses this need.

### 6.x.2	Security threats

If the OP value can be obtained by an unauthorized network element or an unauthorized person, it can be used – in combination with long-term keys – to create subscriber specific Milenage OPc values. These OPc values are one of the data items needed to populate USIMs.

If the stored OP value can be modified in the UDR, this can cause a DOS attack by invalidating regular authentication subscription data.

### 6.x.3	Potential security requirements

If the OP value is stored in the UDR, then the OP value shall be protected against retrieval by unauthorized network elements and by unauthorized persons.

If the OP value is stored in the UDR, then the OP value shall be protected against unauthorized modification after storage in the UDR.

END OF CHANGE 1