3GPP TSG-SA3 Meeting #99e 	S3-201188

- e-meeting, 11 – 15 May 2020															merge of S3-201188 and S3-201003

- Source:	Huawei, Hisilicon, Apple

- Title:	Add a new clause on KAF refresh

- Document for:	Approval

Agenda Item:	4.10

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

NA

# 3	Rationale

During the collecting of open issues, some companies prefers to specify a Kaf refresh procedure, while some companies saying the AKMA should be similar to the GBA. But in GBA, the KNAF refresh is based on the GBA authentication which shall be called rekeying. The different between GBA and AKMA is on the life time of keys. AKMA introduces the independent lifetimes of KAKMA and KAF, but in GBA, the lifetime of Ks may be dependent on the KNAF. The difference between AKMA and GBA is very clear, i.e. when the lifetime of KAF is expired, the KAKMA is not changed. Thus, the first reason of introducing the KAF refresh is because the independent lifetime of KAKMA and KAF.

The other reason to introduce the KAF refresh is that the explicit lifetime has been introduced. If we do not define a 3GPP specific KAF refresh procedure, we can only relay on the design on Ua* interface which is very dangerous. Because whether the AF refresh the KAF is not guaranteed.

The third reason is that a 3GPP specific KAF refresh procedure is that the use of explicit lifetime is clear to the AKMA end user, otherwise the AKMA end user will confuse on the meaning of introducing the explicit lifetime.

The solution is a bit extended than the previous versions, because a new KAKMA and a new A-KID can be generated before the KAF lifetime expires. In this case, the AAnF will return a failure to the AF on KAF refresh.

# 4	Detailed proposal

## 6.4	AKMA key change

6.4.1	KAKMA re-keying

KAKMA shall be re-keyed by running a primary authentication as described in clause 6.1.

### 6.4.2	KAF re-keying

The KAF refresh depends on the lifetime of the KAF and may be trigged by the AF, which means when a new KAKMA is derived, the KAF will not be re-keyed automatically.

### 6.4.X KAF refresh

#### 6.4.X.xx AKMA specific KAF refresh

When the KAF is about to expire, the AF shall send the stored A-KID at the AF corresponding to the currently used KAF to the AAnF in KAF update request message. If the A-KID is found in the AAnF, then the AAnF shall derive a new KAF corresponding to the A-KID. If the A-KID is not found in the AAnF due to re-authentication happened, the AAnF shall response a failure to the AF.

NOTE: Whether to use KAF refresh and how to derive a new KAF is up to Ua* protocol

#### 6.4.X.yy KAF refresh on Ua* protocol

UE and AF may rely on the Ua* protocol to update the KAF.