3GPP TSG-SA3 Meeting #99e 	S3-201199

- e-meeting, 11 – 15 May 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0013 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update testcase in gNB SCAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  |  | Date: |  | 2020-5-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The clause 4.2.2.1.18 is to test whether the gNB support the key refresh when it is acting as an SN. The testcases and the requirement are the same as when gNB as a MN which has already been defined in 4.2.2.1.13. The trequirement and estcases of 4.2.2.1.18 are the same as those are defined in 4.2.2.1.13, so there is no need to duplicated the requirement and testcases. |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the content in clause 4.2.2.1.18 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicated testcases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF CHANGE **********

- 4.2.2.1.18	Key update at the gNB on dual connectivity

Requirement Name: Key update at the gNB on dual connectivity

Requirement Reference: TS 33.501 [2], clause 6.10.2.1; clause 6.10.2.2.1.

Requirement Description: "When executing the procedure for adding subsequent radio bearer(s) to the same SN, the MN shall, for each new radio bearer, assign a radio bearer identity that has not previously been used since the last KSN change. If the MN cannot allocate an unused radio bearer identity for a new radio bearer in the SN, due to radio bearer identity space exhaustion, the MN shall increment the SN Counter and compute a fresh KSN, and then shall perform a SN Modification procedure to update the KSN" as specified in TS 33.501 [2], clause 6.10.2.1.

"The SN shall request the Master Node to update the KSN over the Xn-C, when uplink and/or downlink PDCP COUNTs are about to wrap around for any of the SCG DRBs or SCG SRB" as specified in TS 33.501 [2], clause 6.10.2.2.1.

NOTE: The following testcases are only tested when the NR-NR DC, NE-DC and EN-DC scenarios are deploied.

Threat References: TR 33.926 [5], clause D.2.2.7 Key Reuse

Test Case 1:

Test Name: TC_GNB_DC_KEY_UPDATE_DRB_ID

Purpose:

Verify that the gNB under test acting as a Master Node (MN) performs KSN update when DRB-IDs are about to be reused.

Pre-Conditions:

-	Test environment with a gNB or ng-eNB acting as the Secondary Node (SN), which may be simulated

-	Test environment with a UE, SMF and AMF, which may be simulated

Execution Steps

1.	The gNB under test establishes RRC connection and AS security context with the UE.

2.	The gNB under test establishes security context between the UE and the SN for the given AS security context shared between the gNB under test and the UE; and generates a KSN sent to the SN.

3.	A SCG bearer is set up between the UE and the SN.

4.	The gNB under test is triggered to execute the SN Modification procedure to provide additional available DRB IDs to be used for SN terminated bearers (e.g. by the UE making multiple IMS calls, or by the SMF requesting PDU session modification and deactivation via the AMF), until the DRB IDs are reused,

Expected Results:

-	Before DRB ID reuse, the gNB under test generates a new KSN and sends it via the SN Modification Request message to the SN.

Expected format of evidence:

Evidence suitable for the interface, e.g. text representation of the captured SN Modification Request message.

Test Case 2:

Test Name: TC_GNB_DC_KEY_UPDATE_ PDCP_COUNT

Purpose:

Verify that the gNB under test acting as a Secondary Node (SN) requests KSN update when PDCP COUNTs are about to wrap around.

Pre-Conditions:

-	Test environment with an ng-eNB or gNB acting as the Master Node (MN), which may be simulated.

-	Test environment with a UE which may be simulated.

Execution Steps

1.	The MN establishes RRC connection and AS security context with the UE.

2.	The MN establishes security context between the UE and the gNB under test for the given AS security context shared between the MN and the UE; and generates a KSN sent to the gNB under test.

3.	A SCG bearer is set up between the UE and the gNB under test.

4.	The UE sends UP data in the SCG bearer to the gNB under test until the PDCP COUNT value wraps around.

Expected Results:

-	Before PDCP COUNT wraps around, the gNB under test sends a SN Modification Required message including a KSN update indication to the MN.

Expected format of evidence:

Evidence suitable for the interfaces. Protocol traces of the Uu interface showing the last packets on Uu including PDCP COUNT reset due to rekeying, rather than a PDCP COUNT wrap-around. Protocol traces of the Xn interface including the captured SN Modification Required message. The protocol traces need to be synchronized.

********** END OF  CHANGE **********