3GPP TSG-SA3 Meeting #99e 	S3-201200

- e-meeting, 11 – 15 May 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.216 | CR | 0015 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update eNB SCAS testcase |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_eNB |  |  |  |  |  |  | Date: |  | 2020-5-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update a testcase and delete Editor’s Note |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the testcase of clause 4.2.2.1.3.The testcase is aligned with testcases designed for gNB which was agreed in SA3#97 meeting. In clause 4.2.2.1.14, excuption 1) is a special case of 3). Because X2 interface is also used between MeNB and SeNB. Therefore, the step 1) can be deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase in clause 4.2.2.1.3 is wrong. Testcase in clause 4.2.2.1.14 is duplicated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.3, 4.2.2.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

- 4.2.2.1.3	User plane data ciphering and deciphering at the eNB

Requirement Name: User plane data ciphering and deciphering at eNB

Requirement Reference: TS 33.401 [3], clause 5.3.4

Requirement Description: "The eNB shall cipher and decipher user plane packets between the Uu reference point and the S1/X2 reference points." as specified in TS 33.401 [3], clause 5.3.4.

Threat References: TR 33.926 [4], clause C.2.2.3 – User plane data ciphering and deciphering at eNB.

Test Case:

Test Name: TC-DATA-CIP-eNB-Uu

Purpose: To verify that the user data packets are confidentiality protected over the air interface.

Pre-Condition:

-	The eNB network product shall be connected in emulated/real network environments. UE and the MME may be simulated,

-	The tester can capture the messages via the air interface.

-	The tester shall enable the user plane ciphering protection and ensure EEA0 is not used.

Execution Steps:

1. The UE sends an attach request to the MME.

2. The MME sends a KgNB and the UE security capability to the eNB.

3. eNB selects an algorithm and sends AS SMC to the UE,

4. eNB receive AS SMP from the UE.

Expected Results:

User plane packets sent by the eNB after eNB sending AS SMC is ciphered.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

Test Name: TC-DATA-CIP-eNB-S1/X2

The requirement mentioned in this clause is tested in accordance to the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [2].

********** END OF 1st CHANGE **********

********** START OF 2nd  CHANGE **********

- 4.2.2.1.14	UE NR security capability is only sent to a SgNB

Requirement Name: UE NR security capability is only sent to a SgNB

Requirement Reference: TS 33.401 [3], clause E.3.4.3

Requirement Description: "When adding SgNB while establishing an EN-DC connection, the MeNB shall send these created UE NR security capabilities to the SgNB. Other than for adding an SgNB, the created UE NR security capabilities shall not be sent from the MeNB." as specified in TS 33.401 [3], clause E.3.4.3.

Threat References: TBA

Test Case:

Test Name: TC_NR_SEC_CAP_SENT

Purpose: To verify that the UE NR security capabilities are only sent to a SgNB.

Pre-Condition:

-	The UE, gNB and eNB network products are connected in the test environment. UE and gNB may be simulated.

-	The Tester has method to trigger dual connective to a SeNB and to a SgNB.

-	The tester shall have access to the X2 interface.

Execution Steps:

1) The Tester triggers MeNB to send SN addition Request message to a SeNB.

2) The Tester triggers MeNB to send MN addition Request message to a SgNB.

3) The Tester checks if the UE NR security capabilities were sent in the X2 interface.

Expected Results:

The UE NR security capabilities are only sent to the SgNB.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot contains the operation results.

********** END OF CHANGE **********