- 3GPP TSG SA WG3 Meeting #99e                            	S3-201218

- e-meeting, 11-15 May 2020	                                    revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	A solution to protect SQN during a re-synchronisation procedure in AKA

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

This contribution proposes a solution for the key issue 4.1 of TR 33.846.

# 2	References

# 3	Rationale

The basic idea is similar with solution 4.1. The difference is that the proposed solution does not requires change to f5* itself. The evulatution in solution 4.1 is also added here accordingly.

# 4	Detailed proposal

*************** Start of Change ****************

# 6.4.x	Solution #4.x: A solution to protect SQN during a re-synchronisation procedure in AKA

- 6.4.x.1	Introduction

This solution addresses the key issue #4.1.

- 6.4.x.2  Solution details

In this solution, xoring K with RAND and MAC-S are added as input parameters to calculate AS in the case of synchronisation failure for AKA.  The calculation of the AUTS parameter for re-synchronistaion is described in clauses 6.3.3 and 6.3.5 of TS 33.102 [3].  The figure in clause 6.3.3 is modified to the figure below.

TEMP

Figure 6.4.x.2-1: Modified construction of the parameter AUTS

For re-synchronisation procedure, when the HE/AuC receives an authentication data request with a "synchronisation failure indication" and AUTS, the detailed procedure is defined as follows:

- MAC-S and SQNMS AK are retrieved from AUTS.

- AK is calculated using f5 function with MAC-S as one input.

- SQNMS is retreived by computing SQNMS AK AK.

- MAC-S is verified with MAC-S’, where MAC-S’ = f1*K(SQNMS || RAND || AMF).

- It means MAC-S is one of the inputs to retrieve SQNMS. SQNMS is one of the inputs to verify MAC-S.

- 6.4.x.3	Evaluation

TBD

*************** End of Change ****************