- 3GPP TSG SA WG3 Meeting #99e                            	S3-201223

- e-meeting, 11 - 15 May 2020	                         revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	Clarification on procedures without security protection

- Document for:	Approval

Agenda Item:	4.16

# Decision/action requested

Approve this pCR to complement the content of no security protection.

# References

[1]	Draft TS 33.536 on eV2X

# Rationale

This contribution proposes to clarify on the procedures without security protection. The intention of this contribution is to show the impaction on introducing the on-demand signalling security protection. A summary was givin in S3-201222. Huawei prefares to delete the signalling security policy as proposed in S3-201222.

Background of this contribution:

Based on the email discussion among SA3 eV2X experts, the on-demand security policy negotiations for both signalling and user plane were required during connection set-up due to service flexibility requirements, for example, availability requirements which are prior to security, potential unprotected service in the future development, or distinct requirements caused by different purposes and regions. Due to the above reasons, the necessity of security policy negotiation may not be treated as an open issue any more.

TS 33.536 [1] defines that UE shall only accept to establish a connection without security if the signalling integrity protection policy is set to ‘NOT NEEDED’. However the current version of SA3 eV2X TS [1] still lacks sufficient description regarding the scenario without security protection, thus procedures with no security protection applied need to be clarified in the SA3 eV2X TS, otherwise leading to unclear and incomplete specification.

# Detailed proposal

It is proposed that SA3 approves the below pCR for adding the content of no security protection.

*************** Start of 1st Change ****************

### 5.3.2	Requirements

#### 5.3.2.1	Requirements for securing the PC5 unicast link

The initiating UE shall establish a different security context for each receiving UE during the PC5 unicast link establishment if the security is activated.

PC5 unicast link security establishment between the initiating UE and each receiving UE shall be protected from man-in-the-middle attacks.

The system shall support confidentiality protection, integrity protection and replay protection of the user plane data of PC5 unicast.

The system shall support confidentiality protection, integrity protection and replay protection of signalling for PC5 unicast link.

The system shall support means of configuring the signalling and user plane security policies to UEs for a particular PC5 unicast link.

Signalling plane protection of the PC5 unicast link for a V2X service shall align with the PC5 signalling security policies of the communicating UEs.

User plane protection of the PC5 unicast link for a V2X service shall align with the PC5 user plane security policies of the communicating UEs.

#### 5.3.2.2	Identity privacy requirements for the PC5 unicast link

The 5G System shall provide means for mitigating trackability attacks on a UE during PC5 unicast communications.

The 5G System shall provide means for mitigating linkability attacks on a UE during PC5 unicast communications.

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

###### 5.3.3.1.2.1 	Key hierarchy

PC5 unicast link uses 4 different layers of keying material as shown in figure 5.3.3.1.2.1-1.

Figure 5.3.3.1.2.1-1: Key Hierarchy for PC5 unicast link

The different layers of keys are the following:

Long term credentials: These are the credentials that are provisioned into the UE(s) and form the root of the security of the PC5 unicast link. The credentials may include symmetric key(s) or public/private key pair depending on the particular use case. Authentication signalling (see clause 5.3.3.1.3.2) is exchanged between the UEs to derive the KNRP.

KNRP: This is a 256-bit root key that is shared between the two entities that communicating using NR PC5 unicast link. It may be refreshed by re-running the authentication signalling using the long-term credentials. In order to generate a KNRP-sess (the next layer of keys), nonces are exchanged between the UEs. KNRP may be kept even when the UEs have no active unicast communication session between them. The KNRP ID is used to identify KNRP.

KNRP-sess: This is the 256-bit key that is the root of the actual security context that is being used (or at least in the process of being established) to protect the transfer of data between the UEs. During activated unicast communication session between the UEs, the KNRP-sess may be refreshed by running the rekeying procedure. The actual keys (see next bullet) that are used in the confidentiality and integrity algorithms are derived directly from KNRP-sess. The 16-bit KNRP-sess ID identifies the KNRP-sess.

NOTE 1: A KNRP-sess ID with a zero value indicates no security is used and hence the UEs do not assign an all zero value of KNRP-sess ID when creating a security context.

NRPEK and NRPIK: The NR PC5 Encryption Key (NRPEK) and NR PC5 Integrity Key (NRPIK) are used in the chosen confidentiality and integrity algorithms respectively for protecting PC5-S signalling, PC5 RRC signalling, and PC5 user plane data. They are derived from KNRP-sess and are refreshed automatically every time KNRP-sess is changed.

NOTE 2: Whether the above keys (i.e. KNRP, KNRP-sess, NRPEK and NRPIK) are derived is based on the security activation result of the signalling and user plane security.

###### 5.3.3.1.2.2 	Security states

A UE may be in one of the three different security states with respect to another UE as follows:

Provisioned-security: This is where a UE just has its own long term keys.

Partial-security: This is where a UE has recently communicated with another UE and still has the KNRP that it used with the other UE, but no other derived keys.

Full-security: This is where a UE is actually communicating with another UE and has KNRP, KNRP-sess, NRPEK and NRPIK (as needed), the chosen confidentiality and integrity algorithms and PDCP counters used with each bearer.

Once a UE ends its unicast communication session with another UE, it shall delete KNRP-sess, NRPEK, and NRPIK, the choice of algorithms and the counters. It may also delete KNRP.

###### 5.3.3.1.2.3 	High level flows for the security establishment

Figure 5.3.3.1.2.3-1 provides a high-level flow of a UE establishing a connection with other UE(s).

Figure 5.3.3.1.2.3-1: High-level flow of connection establishment

The flow proceeds as follow:

1. UE_1 sends a Direct Communication Request. This message may be received by multiple UEs.

2a/3a/4a. UE_2a choose to respond to the message and initiates the Direct Auth and Key Establishment procedure (if needed based on the credentials used to establish security – see clause 5.3.3.1.3) to generate the key KNRP. UE_2a then runs the Direct Security Mode Command procedure with UE_1 to establish the security based on that key KNRP. If this is successful, UE_2a sends the Direct Communication Accept message.

2b. UE_2b chooses not to respond the UE_1

2c/3c/4c. UE_2c responds to UE_1 using the same sequence of messages as UE_2a.

When each responder decides to activate signalling integrity protection and/or signalling confidentiality protection, each responder establishes a different security context with UE_1 that is not known to the other UEs, i.e. the security context used between UE_1 and UE_2a is not known to UE_2b and UE_2c.

The Direct Communication Request is always sent unprotected and only contains enough information for a secure connection to be established with the other UE. Any information UE_1 needs to send to the other UEs in order to establish the connection is included in the Direct Security Mode Complete message (sent as part of the Direct Security Mode procedure, see TS 23.287 [2]) from UE_1 as this message is both confidentiality and integrity protected under the condition of activated non-NULL signalling confidentiality protection of the link.

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

##### 5.3.3.1.3	Key establishment procedures

###### 5.3.3.1.3.1 	General

Clause 5.3.3.1.3 provides the details on the establishment of KNRP. The key establishment procedures in this clause shall be skipped if signalling integrity protection is not activated based on the decision of receiving UE of this PC5 unicast link. The long-term credentials and associated authentication method that are used to establish the keys used to protect the PC5 unicast link may either be specified in 3GPP specification or be a method described outside of 3GPP specifications. In the latter case, it is not practical for all cases to specify the signalling in individual IEs on the NR PC5 interface for all these applications, hence all the authentication is specified to be carried in a generic container (called Key_Est_Info in the following clause) on the NR PC5 interface. This allows, for example, an application to change the authentication method without affecting the NR PC5 interface.

*************** End of 3rd Change ****************

*************** Start of 3rd Change ****************

##### 5.3.3.1.4	Security establishment procedures

###### 5.3.3.1.4.1 	General

Clause 5.3.3.1.4.2 describes the security policy and how the UEs handle the policy. There are two different cases when an overall security context may be established; to set up a new connection and to re-key an ongoing connection. These cases are described in clauses 5.3.3.1.4.3 and 5.3.3.1.4.4 respectively. Clause 5.3.3.1.4.5 describes the establishment of security for a user plane bearer.

###### 5.3.3.1.4.2 	Security policy

5.3.3.1.4.2.1	General

The PC5 unicast link shall support activation or deactivation of security based on the security policy similar to Uu, as defined in TS 33.501[6]. The security policy shall be provisioned for PC5 unicast link as well, as detailed in clause 5.3.3.1.4.2.2 of this document and handled as detailed in clause 5.3.3.1.4.2.3 of this document.

5.3.3.1.4.2.2		Procedure for security policy provisioning for PC5 unicast link

For selectively activating or deactivation the security of the PC5 unicast link, the PCF may provision the security policy per V2X service, during service authorization and information provisioning procedure as defined in TS 23.287 [2].

5.3.3.1.4.2.3		Security policy handling

For NR PC5 Unicast the UE shall be provisioned with the following security policy:

The list of V2X services, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s) and their security policy which indicates the following:

•	Signalling integrity protection: REQUIRED/PREFERRED/NOT NEEDED

•	Signalling confidentiality protection: REQUIRED/PREFERRED/NOT NEEDED

•	User plane integrity protection: REQUIRED/PREFERRED/NOT NEEDED

•	User plane confidentiality protection: REQUIRED/PREFERRED/NOT NEEDED

NOTE 1: No integrity protection on signalling traffic enables services that do not require security.

NOTE 2: While some V2X applications are similar to Emergency Services and may require similar security policies handling, such V2X applications are outside of the scope of 3GPP.

The signalling integrity protection security policy being NOT NEEDED means that the UE shall only establish a connection with no security. The signalling integrity protection security policy being PREFFERED means that the UE may try to establish security but will accept the connection with no security. With the integrity protection security policy set to REQUIRED, the UE may only accept the connection if a non-NULL integrity algorithm is used for protection of the signalling traffic.

For the other cases under the condition of integrity protected signalling traffic, a setting of NOT NEEDED means that the UE shall only use NULL algorithm (i.e. applies no protection) for that particular protection, while a REQUIRED setting means that the UE shall only use a non-NULL algorithm. If the security policy is PREFERRED, then the UE may accept any algorithm for that particular protection. One use of PREFERRED is to enable a security policy to be changed without updating all UEs at once.

At initial connection, the UE includes its signalling security policy in the Direct Communication Request message. The UE(s) responding to this takes this into account when choosing the algorithms in the Direct Security Mode Command message. The chosen algorithms shall include the selected integrity and confidentiality protection algorithms. The initiating UE can reject the Direct Security Mode Command if the algorithm choice does not match its policy.

When adding a V2X service to an existing connection, the UE responding to the request shall reject the request if signalling security in use does not match the policy for the new application.

The combination of security policies for UP Integrity Protection will result in the following activation of integrity protection:

Case 1: Both UP security policies indicate UP Integrity Protection "REQUIRED", or one UP security policy indicates “REQUIRED” and the other indicates “PREFERRED”:

Activation of UP integrity protection for each user plane bearer individually of the service type when the PC5 unicast is established.

Case 2: Both UP security policies indicate UP Integrity Protection "PREFERRED":

Activate or deactivate of UP integrity protection for each user plane bearer individually of the service type when the PC5 unicast is established based on local policy.

Case 3: For the other scenarios besides Case 1 and Case 2:

Deactivation of UP integrity protection for each user plane bearer individually of the service type when the PC5 unicast is established.

For UP Ciphering Protection, the resulting activation is the same as the UP integrity protection activation.

The V2X layer of the UE shall pass the security configurations to its AS layer. The security configurations are mutually agreed by both sides’ UEs, including the configuration of confidentiality and integrity protection.

###### 5.3.3.1.4.3 	Security establishment during connection set-up

The clause describes how security is established during connection set-up. The signalling flow is shown in figure 5.3.3.1.4.3-1.

Figure 5.3.3.1.4.3-1: Security establishment at connection set-up

1.	UE_1 has sent a Direct Communication Request to UE_2. This message shall include UE_1’s security capabilities (the list of algorithms that UE_1 will accept for this connection) and UE_1’s signalling security policy. The UE_1 shall also include Nonce_1 (for session key KNRP-sess generation), and the most significant 8-bits of the KNRP-sess ID in this message if UE_1’s signalling integrity protection policy is either “REQUIRED” or “PREFERRED”.D. The most significant 8-bits of the KNRP-sess ID shall be chosen such that UE_1 will be able to locally identify a security context that is created by this procedure. The message may also include a KNRP ID if the UE_1 has an existing KNRP for the UE that it is trying to communicate with. The absence of the KNRP ID parameter indicates that UE_1 does not have a KNRP for UE_2. The message also contains Key_Est_Info (see subclause 5.3.3.1.3.2).

2.	UE_2 shall reject the Direct Communication Request if UE_1's signalling security policy is "NOT NEEDED" while UE_2's security policy is "REQUIRED". UE_2 shall also reject the Direct Communication Request if UE_1's signalling security policy is "REQUIRED" while UE_2's security policy is "NOT NEEDED". UE_2 may initiate a Direct Auth and Key Establish procedure with UE_1. This is mandatory if the UE_2 does not have the KNRP and KNRP ID pair indicated in step 1, and signalling is needed to establish the keys for the particular use case.

3.	UE_2 shall send the Direct Security Mode Command message to UE_1. This message shall only contain the MSB of KNRP ID and optionally Key_Est_Info if a fresh KNRP is to be generated (see clause 5.3.3.1.3).  UE_2 shall includethe Chosen_algs parameter to indicate which security algorithms the UEs will use to protect the data in the message. The Chosen-algs may only indicate the use of the NULL integrity algorithm if UE_2’s signalling integrity security policy is either NOT NEEDED or PREFERRED. UE_2 shall also return the UE_1’s security capabilities and UE_1’s signalling security policy to provide protection against bidding down attacks. In the case that the NULL integrity algorithm is chosen, the NULL confidentiality algorithm shall also be chosen and UE_2 shall set the KNPR-sess ID of this security context to the all zero value.

The following procedures in step 3 shall only be executed if the UE_2 deides to at least activate the integrity security protection for this connection: UE_2 shall also include Nonce_2 to allow a session key to be calculated, as well as the least significant 8-bits of KNRP-sess ID in the messages. These bits are chosen so that UE_2 will be able to locally identify a security context that is created by this procedure. UE_2 shall calculate KNRP-Sess from KNRP and both Nonce_1 and Nonce_2 (see Annex A.3) and then derive the confidentiality (if needed) and integrity keys based on the chosen algorithms (Annex A.2). UE_2 shall integrity protect the Direct Security Mode Command before sending it to UE_1. UE_2 is then ready to receive both signalling and user plane traffic protected with the new security context. UE_2 shall form the KNRP-sess ID from the most significant bits it received in message 1 and least significant bits it sent in message 3.

4. 	On receiving the Direct Security Mode Command, the UE_1 shall first check the Chosen_algs and shall accept the NULL integrity algorithm only if its security policy for signalling integrity protection is either NOT NEEDED or PREFERRED. Then UE_1 shall check the returned UE_1’s security capabilities and UE_1’s signalling security to avoid bidding down attacks if NULL integrity algorithm is selected for signalling integrity protection. If the above check passes, UE_1 shall send unprotected Direct Security Mode Complete message to UE_2. UE_1 shall set the KNPR-sess ID of this security context to the all zero value.

Under the condition of non-NULL integrity algorithm indicated in the Chosen_algs, UE_1 shall first check that the received LSB of KNPR-sess ID is unique, i.e. has not been sent by another UE responding to this Direct Commuication Request. If the LSB of KNPR-sess ID is not unique, then UE_1 shall respond with a Direct Security Mode Reject message including a cause value to specify that the LSB of KNPR-sess ID is not unique. The peer UE-2 receiving a Direct Security Mode Reject message shall inspect the cause value and, if the cause is related to the session identifier uniqueness then, the UE-2 shall generate a new LSB of KNPR-sess ID and reply to UE-1 again (i.e., UE-2 shall send a Direct Security Mode Command message with the new LSB of KNPR-sess ID). UE-2 shall erase the former LSB of KNPR-sess ID from its memory. On receiving this new Direct Security Mode Command, UE_1 shall process the message from the start of step 4.

If the LSB of KNPR-sess ID is unique, UE_1 shall calculate KNRP-sess and the confidentiality and integrity keys in the same way as UE_2. UE_1 shall check that the returned UE_1 security capabilities and UE_1’s signalling security policy are the same as those it sent in step 1. UE_1 shall also check the integrity protection on the message. If both these checks pass, then UE_1 is ready to send and receive signalling and user plane traffic with the new security context. UE_1 shall send integrity protected and confidentiality protected Direct Security Mode Complete message to UE_2. UE_1 shall form the KNRP-sess ID from the most significant bits it sent in message 1 and least significant bits it received in message 3.

5.	If the Chosen_algs in step 3 includes non-NULL integrity algorithm, UE_2 checks the integrity protection on the received Direct Security Mode Complete. If this passes, UE_2 is now ready to send user plane data and control signalling protected with the new security context. UE_2 deletes any old security context it has for UE_1.

###### 5.3.3.1.4.4 	Security establishment during re-keying

The rekeying specified in this sub-clause shall only be executed under the condition that signalling integrity protection is activated for this connection, as keys are not established when security protection of this connection is not needed. By rekeying, the UEs ensure fresh session keys KNRP-sess are used. Optionally the rekeying can also enforce refresh of KNRP. Either UE may rekey the connection at any time. This shall be done before the counter for a PDCP bearer repeats with the current keys. A rekeying operation shall refresh the KNRP-sess  and NRPEK and NRPIK, and may refresh KNRP. A rekeying operation follows the flows given in figure 5.3.3.1.4.4-1.

Figure 5.3.3.1.4.4-1: Security establishment during rekeying

1.	UE_1 sends a Direct Rekey Request to UE_2. This message shall include Nonce_1 (for session key generation), UE_1 security capabilities (the list of algorithms that UE_1 will accept for this connection) and the most significant 8-bits of the KNRP-sess ID. These bits are chosen such that UE_1 will be able to locally identify a security context that is created by this procedure. The message may also include a Re-auth Flag if UE_1 wants to rekey KNRP. The message also contains Key_Est_Info (see subclause 6.5.4).

2.	UE_2 may initiate a Direct Auth Key Establish procedure with UE_1. This is mandatory if UE_1 included the Re-auth Flag and signalling is needed to establish KNRP.

3.	This step is the same as step 3 in 5.3.3.1.4.3 except that the chosen algorithm shall only be NULL if the NULL algorithm is currently in use, the chosen algorithm shall only be non-NULL if the non-NULL confidentiality algorithm is currently in use and UE_1’s signalling security policy is not included in this message.

4.	This step is the same as step 4 in 5.3.3.1.4.3 except that UE_1 shall only accept the NULL algorithm if the NULL algorithm is currently in use, UE_1 shall only accept the non-NULL algorithm if the non-NULL algorithm is currently in use, and UE_1 does not check the returned siganlling security policy (as it is not sent in this case).

5.	This step is the same as step 5 in 5.3.3.1.4.3.

6. When UE_1 receives message integrity protected with the new security context, it shall delete any old security context it has still stored for UE_2.

###### 5.3.3.1.4.5 	Security establishment for user plane bearers

At initial connection or adding a V2X service, the initiating UE includes its user plane security policy in the Direct Security Mode Complete or Link Modification request message respectively. The receiving UE shall reject the connection setup or Link Modification Request if the received user plane security policy had either confidentiality/integrity set to NOT NEEDED and its own corresponding policy is set to REQUIRED or if the received user plane security policy had either confidentiality/integrity set to REQUIRED and its own corresponding policy is set to NOT NEEDED. Otherwise, the receiving UE may accept the connection setup or Link Modification Request.

Editor’s note: If is FFS whether the receiving UE sends its UP security policy or a choice of security back to the initiating when accepting the connection setup or Link Modification Request.

The UE initiating the establishment of a user plane bearer shall select an LCID whose associated value of Bearer for input to the security algorithms (see clauses 5.3.3.1.5.2 and 5.3.3.1.5.3) has not been used with the current keys, NRPEK and NRPIK. If this is not possible the UE shall initiate a re-keying (see clause 5.3.3.1.4.4) before establishing the user plane bearer.

When establishing the user plane bearer, the initiating UE shall indicate the configuration of confidentiality and integrity protection in the PC5-RRC message. The confidentiality and/or integrity protection algorithms are same as those selected for protecting the signalling bearers if confidentiality and/or integrity protection are required for both signalling and user plane.

Both UEs shall ensure that the user plane for each V2X service is only sent or received (e.g. dropped if received on a bearer with incorrect security) on user plane bearers with the necessary security if security protection of this link is activated.

*************** End of 3rd Change ****************

*************** Start of 4th Change ****************

#### 5.3.3.2 	Identity privacy for the PC5 unicast link

##### 5.3.3.2.1	General

The link identifier update procedure given in TS 23.287 [2] is used to provide privacy for the identities in the unicast link. These messages in this procedure are always sent protected and shall be executed under the condition that security protection is established for this connection, hence the new identities agreed by the UEs are only known to the involved UEs. A three-way message exchange procedure is required with this procedure since both UEs need to change their identifiers during the same procedure and to allow these new values to be acknowledged before them being used.

##### 5.3.3.2.2	Procedures

Figure 5.3.3.2.2-1 shows the flows for changing the identities of the UEs involved in PC5 unicast link. The figure and the following steps only show the security parameters (KNRP-sess ID) that are changed and the Layer-2 ID but not the other parameters described in TS 23.287 [2].

Figure 5.3.3.2.2-1: Link identifier update procedure

The procedure proceeds with the following steps and provides additional handling on top of what is provided in TS 23.287 [2].

0. UE_1 and UE_2 are communicating via a unicast link and have established the security for the link.

1. UE_1 decides to change the identifiers and sends a Link Identifier Update Request message to UE_2 (see TS 23.287 [2]). UE_1 shall include the MSB of KNRP-sess ID in the Link Identifier Update Request message. These bits shall be chosen so that they uniquely identify KNRP-sess at UE_1.

2. UE_2 shall choose the LSB of KNRP-sess ID so that they uniquely identify KNRP-sess at UE_2. UE_2 shall form the new KNRP-sess ID from the MSB received from UE_1 and the LSB that UE_2 chose. UE_2 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_2 shall send the LSB of KNRP-sess ID to UE_1 along with the received MSB of KNRP-sess ID in the Link Identifier Update Response message. UE_1 shall check that the returned MSB of KNRP-sess ID is identical to the one sent in step 1.

3. UE_1 shall form the new KNRP-sess ID from the LSB received from UE_2 and the MSB chosen by UE_1 (in step 1). UE_1 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_1 shall send the Link Identifier Update Ack message to UE_2 including the LSB of KNRP-sess ID. UE_2 shall check that the returned LSB of KNRP-sess ID are identical to the one sent in step 2.

*************** End of 4th Change ****************