3GPP TSG-SA3 Meeting #99e 	S3-201287

- e-meeting, 11-15 May 2020													Revision of S3-20xxxx

- Source:	Samsung, Huawei, Hisilicon, CATT

- Title:	Authorization of AF in AKMA

- Document for:	Approval

Agenda Item:	4.10

# 1	Decision/action requested

It is proposed to approve this contribution for AF authorization in AKMA (TS 33.535).

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]                        3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

# 3	Rationale

This contribution proposes to resolve the following Editor’s Note:

Editor’s Note: It is FFS how the AAnF knows whether it can provide the service to a specific AF

The AAnF shall authorize the AF based on the configured local policy or authorization information or policy provided by NEF.

# 4	Detailed proposal

*****START OF CHANGES*****

## 6.2	Deriving AKMA Application key for a specific AF

Figure 6.2-1 AF Key generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF needs to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF.

Editor’s Note: It is FFS whether the capability to negotiate the use of AKMA between the UE application and the AKMA AF also needs to be supported.

1.	When the UE initiates communication with the AKMA AF, it shall include the derived A-KID in the Application Session Establishment request message (cf. clause 6.1).

2.	If the AF does not have an active context associated with the A-KID, then the AF sends a Naanf_key_create request to AAnF with the A-KID to request application function-specific AKMA keys for the UE.  The AF also includes its identity (AF Id) in the request. The AAnF shall authorize AF. The AAnF shall check whether the AAnF can provide the service to the authorized AF based on the configured local policy or based on the authorization information or policy provided by the NEF/NRF using the AF Id. If succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

The AAnF can check whether the subscriber is authorized to use AKMA by the presence of the AKMA anchor key K_AKMA that has been received from the AUSF.

If the AAnF is in possession of the AF specific key (KAF), it responds to the AF with the KAF key. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the A-KID. The Step 3 to step 5 are skipped.

If KAKMA is available in AAnF, it shall derive the AF specific AKMA key (KAF) from KAKMA and respond to the AF with KAF and lifetime. The Step 3 and step 4 are skipped.

If KAKMA is not available, the procedure continued.

3.	The AAnF shall send a Nausf_AKMAkey_Create request to the AUSF to obtain the KAKMA key specific to the UE. It includes the A-KID in the request.

4.	The AUSF shall respond Nausf_AKMAkey_Create with the KAKMA key identified by the A-KID.

5.	The AAnF derives the AF specific key (KAF) from KAKMA.

6.	The AAnF sends Naanf_key_Create response to the AF with KAF and lifetime.

7.	The AF response the Application Session Establishment request to the UE.

Editor’s Note: Derivation of the AF specific KAF is FFS.

*****END OF CHANGES*****