3GPP TSG-SA3 Meeting #99e 	Draft v3 S3-201307

- e-meeting, 11 -15 May 2020

Title:	[DRAFT] LS on Updated User Plane Integrity Protection advice

Response to:

Release:	16

Work Item:

Company Sources:	Vodafone

Source:	SA3

To:	CT1, SA2

Cc:	SA, RAN

Contact Person:

#### Name:	Tim Evans

Tel. Number:	+447920871635

E-mail Address:	tim.evans1@vodafone.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-201392, S3-201391 and S3-201296

1. Overall Description:

SA3 has reviewed the information provided by the GSMA regarding user plane integrity protection (UPIP) related attacks on 3GPP 4G and 5G networks.  Whilst these attacks are very difficult, require very skilled attackers to exploit and are currently localised attacks based on false base stations, SA3 is conscious that once such attacks are known, refinements and the development of tools for less skilled attacks usually follow.

SA3 advises that it has agreed the following 2 CR's:

- S3-201392 (attached) - CR to 33.501 (5G security) - Adds an informative annex describing how to secure DNS and ICMP traffic

- S3-201391 (attached) - CR to TS 33.401 (LTE security) - Adds and informative reference to the informative annex added into TS 33.501 describing how to secure DNS and ICMP traffic

Additionally, SA3 discussed S3-201296 (attached), but several delegates raised concerns about the impacts of this solution on SA2 specifications.  SA2 are invited to comment as to whether S3-201296 has any adverse impacts.

2. Actions:

To CT1, SA2 groups.

ACTIONS:

- SA3 kindly asks CT1 to review this information and update their specifications from release 16 accordingly.

- SA3 kindly asks SA2 to review and comment as to whether S3-201296 has any adverse impacts on their specifications.

3. Date of Next TSG-SA WG3 Meetings:

SA3#100e	6 -10 July 2020	e-meeting

SA3#100e-Bis	17 – 21 August 2020	e-meeting