3GPP TSG-SA3-LI Meeting #55	SA3LI14_151r3

# Portland, Oregon, USA, 28-30 October 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of LI for GCSE with HI2 & HI3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2014-11-05 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Current text misses some important IRI information (e.g., location of the target) when reporting GCS AS events. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Includes changes to align event information for GCS reported events with messages reported for LI for GCSE over HI2 & HI3 and correct instances of GCSE AS to GCS AS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.2.0, 16.2.2.1, 16.2.2.2, 16.2.2.3, 16.2.2.4, 16.2.2.5, 16.2.2.6, 16.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

##### 16.2.2.2.0	General

Intercept Related Information events to be reported by the GCS AS include:

-	When GCSE communications group involving a target of interception is activated (enabled for communications)

-	When GCSE communications group involving a target of interception is deactivated (no longer enabled for communications)

-	When a User is added to an active GCSE communications group

-	When a User is dropped from an active GCSE communications group

-	Start of Interception with an Active GCSE communications Group

-	End of Interception with an Active GCSE communications Group

-	Modification of Target Connection to GCS AS.

A set of possible elements as shown in Table 16.4 are used to generate the events.

Table 16.4: Information Events for GCS AS Event Records

Element

Observed IMSI IMSI of the target.

Observed IMEI IMEI of the target.

Observed ProSe UE ID ProSe UE ID of the target.

Observed Other Identity Other Identity of the target

Event type Description which type of event is delivered: Group Activated, Group Deactivated. Group Add Member, Group Drop Member, Start of Intercept with Active Group, End of Intercept with Active Group, Modification of Active Group.

Event date Date of the event generation in the GCS AS.

Event time Time of the event generation in the GCS AS. Timestamp shall be generated relative to the GCS AS internal clock.

Observed Communications Group ID Identifies the GCSE communications group at the GCS AS.

GCSE Group Communication Characteristics Details of the Group Communications Service to which the Target is a member including such characteristics such as voice, video, and data communications.

GCSE Communications Group Membership List List of all users that are membersof the GCSE communications group. Not all members may be participants in a group communications.

GCSE Communications Group Participants List of all users that are participating in the GCSE communications group.

Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records.

Network Element Identifier Unique identifier for the element reporting the ICE.

Added User ID Identity of the party successfully added to an active GCSE Communications Group.

Dropped User ID Identity of the party successfully dropped from an active GCSE Communications Group.

Target Connection Method Identifies the current target connection method with the GCS AS including whether the target is connected at all.

Modified Target Connection Method Identifies the modified target connection method with the GCS AS when the target connection method changes including whether the target is connected at all.

Identity of Visited Network Identifies the visited network from which the target is connecting to the GCS AS.

Length of TMGI Reservation Time Identifies the length of time reserved for use of a TMGI for a GCSE Communications Group.

Reserved TMGI Identifies the TMGI reserved for use by a GCSE Communications Group.

Location information Location information of the target, e.g., Cell ID as known by the GCS AS.

NOTE: Generation of Correlation Number is FFS.

*** NEXT MODIFICATION ***

##### 16.2.2.2.1	Activation of GCSE Communications Group

When a GCSE communications group is activated at the GCS AS (i.e., enabled for communications), an Activation of GCSE Communications Group event is generated in the following cases:

-	When the GCS AS successfully activates a GCSE communications group in which a member is a target of interception.

The fields, shown in Table 16.5, will be delivered to the DF2, if available, by the GCS AS.

Table 16.5 Activation of GCSE Communications Group

Observed IMSI

Observed IMEI

Observed ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Target Connection Method

GCSE communications group membership list

GCSE communications group participants

Group Communications Characteristics

Observed Communications Group ID

Reserved TMGI

Length of TMGI reservation time (if known)

Identity of Visited Network (if known)

Location Information

*** NEXT MODIFICATION ***

##### 16.2.2.2.2	Deactivation of GCSE Communications Group

When a GCSE communications group is deactivated (not enabled for communications) at the GCS AS, a Deactivation of GCSE Communications Group event is generated in the following cases:

-	When the GCS AS successfully releases a GCSE communications group in which a member is a target of interception.

The fields, shown in Table 16.6, will be delivered to the DF2, if available, by the GCS AS.

Table 16.6 Deactivation of GCSE Communications Group

Observed IMSI

Observed IMEI

ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

GCSE communications group membership list

Observed Communications Group ID

Reserved TMGI

Length of TMGI reservation time (if known)

Identity of Visited Network (if known)

Location Information

*** NEXT MODIFICATION ***

##### 16.2.2.2.3	User Added

A User Added-event is generated in the following cases:

-	When a user is successfully added to a GCSE communications group in which a member is a target of interception.

The fields, shown in Table 16.7, will be delivered to the DF2, if available, by the GCS AS.

Table 16.7 User Added

Observed IMSI

Observed IMEI

Observed ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Added User ID

GCSE communications group membership list

Observed Communications Group ID

GCSE communications group participants

Reserved TMGI

Identity of Visited Network (if known)

*** NEXT MODIFICATION ***

##### 16.2.2.2.4	User Dropped

A User Dropped-event is generated in the following cases:

-	When a user is successfully dropped from a GCSE communications group in which a member is a target of interception.

The fields, shown in Table 16.8, will be delivered to the DF2, if available, by the GCS AS.

Table 16.8 User Dropped

Observed IMSI

Observed IMEI

Observed ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Dropped User ID

GCSE communications group membership list

Observed Communications Group ID

GCSE communications group participants

Reserved TMGI

Identity of Visited Network (if known)

*** NEXT MODIFICATION ***

##### 16.2.2.2.5	Start of Intercept with an Active GCSE Communications Group

When an intercept is started with an active GCSE communications group, a Start of Intercept for GCSE Communications Group event is generated in the following cases:

-	When a target of interception is successfully added to an active GCSE communications group.

-	When interception is activated for a target of interception who is already a member of an active GCSE communications group.

The fields, shown in Table 16.9, will be delivered to the DF2, if available, by the GCS AS.

Table 16.9 Start of Intercept with an Active GCS Communications Group

Observed IMSI

Observed IMEI

Observed ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Target Connection Method

GCSE communications group membership list

Group Communications Characteristics

Observed Communications Group ID

GCSE communications group participants

Reserved TMGI

Length of TMGI reservation time (if known)

Identity of Visited Network (if known)

Location Information

*** NEXT MODIFICATION ***

##### 16.2.2.2.6	End of Intercept with an Active GCSE Communications Group

When an intercept is ended with an active GCSE communications group, an End of Intercept for GCSE Communications Group event is generated in the following cases:

-	When a target of interception is successfully dropped from an active GCSE communications group.

The fields, shown in Table 16.10, will be delivered to the DF2, if available, by the GCS AS.

Table 16.10 End of Intercept with an Active GCS Communications Group

Observed IMSI

Observed IMEI

Observed ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

GCSE communications group membership list

Observed Communications Group ID

GCSE communications group participants

Reserved TMGI

Length of TMGI reservation time (if known)

Identity of Visited Network (if known)

Location Information

*** NEXT MODIFICATION ***

##### 16.2.2.2.7	Modification of Target Connection to GCS AS

When a modification to a target connection to the GCS AS occurs, a Modification of Target Connection event is generated in the following cases:

-	When a target of interception changes the current downlink communications reception method to now receive only via a unicast link, only via a multicast link, or via both unicast and multicast links.

-	When the target of interception changes the uplink and downlink connection method from not connected to one of the connected connection methods, and vice versa.

The fields, shown in Table 16.11, will be delivered to the DF2, if available, by the GCS AS.

Table 16.11 Modification of Target Connection to GCS AS

Observed IMSI

Observed IMEI

Observed ProSe UE ID

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Correlation Number

Modified Target Connection Method

GCSE communications group membership list

Group Communications Characteristics

Observed Communications Group ID

GCSE communications group participants

Reserved TMGI

Length of TMGI reservation time (if known)

Identity of Visited Network (if known)

Location Information

*** END OF MODIFICATIONS ***