3GPP TSG-SA3-LI Meeting #56	S3i150050

# Sophia Antipolis, France, 20-22 January 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | WebRTC Interworking LI Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2015-01-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | The current specification for 33.107 does not address the LI Stage 2 X2 & X3 changes needed to support WebRTC Interworking. This needs to be addressed to meet Regulatory requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution incorporates changes to the LI Stage 2 X2 & X3 to support WebRTC Interworking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no LI solution for WebRTC Interworking resulting in Regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7A.2.1, 15.2, E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Revision of S3i150038 |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	ETSI ES 201 158 (V1.2.1 April 2002): "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671 (V3.1.1 May 2007): "Handover Interface for the lawful interception of telecommunications traffic".

[4]	Void

[5]	Void

[6]	Void

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A (April 2003): "Lawfully Authorised Electronic Surveillance".

[9]	 VOID

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	 Void

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122 (October 1989): "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123 (October 1989): "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	Void

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: " Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2"

[31]	3GPP TS 22.220: " Service Requirements for Home NodeBs and Home eNodeBs".

[32]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2".

[33]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2"

[34]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB) ".

[35]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[36]	IETF RFC 3966 (December 2004): "The Tel URILs for Telephone Numbers ".

[37]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[38]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[39]	IETF RFC 791: "Internet Protocol".

[40]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[41]	IEFT RFC 3697: "IPv6 Flow Label Specification"

[42]	3GPP TS 29.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW); Iq Interface (Stage 3)"

[43]	3GPP TS 23.228: "IP Multimedia Subsystem; Stage 2"

[44]	3GPP TS 23.203: "Policy Charging and Control Architecture"

[45]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2"

[46]	3GPP TS 29.162: "Interworking between IM CN subsystem and IP Networks"

[47]	3GPP TS 29.163:"Interworking between IP Multimedia Core Network (CN) subsystem and Circuit Switched (CS) Networks"

[48]	3GPP TS 23.334: "IP Multimedia Subsystem (IMS) Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) interface: Procedures descriptions"

[49]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3"

[50]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)"

[51]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)"

[52]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2"

[53]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2"

[54]	3GPP TS 23.303: "Proximity based Services; Stage 2".

[55]	3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[56]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[xx]	IETF RFC 6714: “Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)”.

*** NEXT MODIFICATION ***

### 7A.2.1	Provision of IRI with SIP messaging

SIP messaging is reported as Intercept Related Information for the interception of multi-media service. As shown in figure 22 below, all SIP messages executed on behalf of a target are subject to intercept at the S-CSCF and Optionally P-CSCF. Based upon network configuration, the ADMF shall provision P-CSCFs, or S-CSCFs, or both P-CSCFs and S-CSCFs with SIP URI, TEL URI, or IMEI target identifiers. These resulting intercepted SIP messages shall be sent to DF2 for mediation prior to transmittal across the HI2 interface.  For the support of WebRTC Interworking as specified in [43], the eP-CSCF shall adhere to all LI requirements pertaining to the P-CSCF. Any additional requirements that only apply to the eP-CSCF are described distinctly.

WebRTC Web Server Function (WWSF), if provided by the CSP, is an ICE that is used to copy and transmit via the DF to the LEMF the outer IP address and port used by the target in the internet or any public IP network.

WebRTC Authorisation Function (WAF), if provided by the CSP, is an ICE that creates a time-stamped authentication event associated with the target including relevant information such as the user’s identity provided to the WAF.

Further details of the WWSF and WAF are FFS.

For roaming scenarios, interception at the P-CSCF shall be Mandatory, in order to provide IRI Interception in the visited network, where the P-CSCF is located in the Visited Network. Where the P-CSCF is located in the Home Network, interception at the P-CSCF shall be Optional, subject to national regulation.

ADMF

P

-

CSCF

S-CSCF

DF2

LEMF

Intercepted

Subscriber

Other
party

Figure 22: Provision of Intercept Related Information for multi-media

*** NEXT MODIFICATION ***

## 15.2	Provision of Content of Communications

As the interception of CC needs to be done at a network node that has access to the voice media, and that interception of CC is required for all targeted calls, including forwarded calls and transferred calls, the CSP needs to support the capability to dynamically trigger CC interception for a call at a network node that has access to the voice media. Depending on the CSP’s network configuration and the call scenario, different network elements will intercept the CC.  For WebRTC Interworking as described in [43], the eIMS-AGW re-writes the original Message Session Relay Protocol (MSRP) message which contain IP address information when the both endpoints don’t support MSRP Connection Establishment for Media Anchoring (CEMA) extension [xx], the eIMS-AGW shall be able to copy and report this information. WWSF, if provided by a Third Party Provider, is outside the scope of this specification.

The interception and delivery of CC for VoIP may be done at the following functional element:

1)	PDN-GW/GGSN;

2)	IMS-AGW;

3)	TrGW;

4)	IM-MGW;

5)	MRF.

Note 2:  Other functional elements may also be applicable in specific deployment scenarios.

Note 3:  The redirection of target communications to a specific network element purely for LI purposes is undesirable.

The functional elements that provide the signalling to generate the trigger for the CC interception may be any of the following functional elements:

-	P-CSCF, for PDN-GW/GGSN and IMS-AGW;

-	IBCF for TrGW;

-	MGCF for IM-MGW

-	S-CSCF or AS for MRF.

At any given time, for a specific target and for any given call, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the call scenario.

Annex E shows scenarios where the use of the functional elements listed above are applicable.

*** NEXT MODIFICATION ***

# E.2	Background

One of the use-cases of IMS-based VoIP service is VoLTE.  The term "VoLTE" is used to refer to an IMS-based VoIP service when EPS (see 3GPP TS 23.401 [22]) happens to be the access network. When EPS is not the access network, the lawful interception capabilities defined in this informative annex applies for any IMS-based VoIP service with the presumption that in those cases the media on the target’s access goes through an IMS-AGW (see 3GPP TS 29.334 [42]) or a PDN-GW (see 3GPP TS 23.401 [22] and 3GPP TS 402 [23]) or a GGSN (UMTS network).  For the support of WebRTC Interworking as specified in [43], the P-CSCF and IMS-AGW shall be interpreted as the eP-CSCF and the eIMS-AWG, respectively, when handling WebRTC Interworking.  That is, all LI requirements pertaining to the P-CSCF and IMS-AGW shall apply to the eP-CSCF and eIMS-AGW, respectively, and wherever a P-CSCF or IMS-AGW appears in this specification, it shall be considered to be the eP-CSCF for the WebRTC Interworking cases. Any additional requirements that only apply to the eP-CSCF or eIMS-AGW are described distinctly.

Note 1: Even with the EPS-based access network, the media may still go through the IMS-AGW. And, in this case, a VoLTE shall be treated very similar to any other VoIP service.

Furthermore, it is presumed that an inter-CSP call enters or leaves an IMS network via an IBCF/TrGW or an MGCF/IM-MGW depending on whether the inter-working CSP network is an IMS-based network or a CS-based network (see 3GPP TS 23.228 [43]). Also, for an IMS roaming scenario, it is presumed that the signalling and media enters or leaves the home CSP through the IBCF/TrGW.

The figure E.1 illustrates the VoIP configuration considered for the lawful interception capabilities defined in this clause:

Figure E.1: IMS-based VoIP Configuration

Note 2: The configuration scenario where the media does not go through the GGSN, or PDN-GW or the IMS-AGW is outside the scope of this document.

Note 3: In the remaining part of this informative annex, the PDN-GW/GGSN are shown in one box and should read as either the GGSN (UMTS) or the PDN-GW (EPS).

In figure E.1, the term "media node" is used to denote the network node present on media path and the term "signalling node" is used to denote the network node present on the signalling path.

The general concepts of VoIP LI is shown in figure E.2

Figure E.2: General Principles of VoIP Interception

In clause 15, the SIP signalling nodes provide signalling information to the CC Interception Triggering Function.  The CC Interception Triggering Function triggers the CC interception at a media node that implements the CC Intercept Function.

The following functional elements provide the CC Intercept Function in the example call scenarios:

-	PDN-GW/GGSN;

-	IMS-AGW;

-	TrGW;

-	IM-MGW

-	MRF.

The following functional elements provide the signalling to the CC Intercept Triggering Function:

-	P-CSCF, for PDN-GW/GGSN and IMS-AGW;

-	IBCF for TrGW;

-	MGCF for IM-MGW

-	S-CSCF for MRF.

At any given time, for a specific target and for any given call, only one functional element is required to provide the CC interception. The functional element that provides the CC interception may vary, primarily, based on the CSP network implementation and the call scenario.

*** END OF MODIFICATIONS ***