3GPP TSG-SA3 Meeting #74-BIS LI	s3i190538

# Basking Ridge (NJ), United States, 4th Sep 2019 - 6th Sep 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.126 | CR | 0042 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Separate Service Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2019-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The document does not capture the full architectural requirement regarding the LI architeture being able to intercept multiple services for a single target as well as to separately intercept and deliver services for a single target to the exclusion of other services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR updates the delivery requirement to support separation of CSP provided services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure to fulfill regulatory requirements for being able to intercept and deliver only the service(s) required to be intercepted by a lawful authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of First MODIFICATION ***

## 6.4	Delivery

R6.4 – 10	LI Service Scope - The CSP shall only deliver Interception Product relating to specific CSP services which are specified implicitly or explicitly in the warrant. . The system should be able to support the ability of the CSP to perform interception and delivery of multiple CSP subscription based services (e.g., CSP provided voice, messaging services, internet access) for a single target. The system should be able to support the ability of the CSP to perform delivery of an intercepted service separate from the delivery of all other intercepted services for a single target (e.g., only perform intercept of messaging services).

R6.4 - 20	Context Correlation - The CSP shall be able to deliver information such that the LEA can correlate all CC and IRI to the Context of Communications.

R6.4 - 30	IRI to IRI Correlation - The CSP shall be able to deliver information such that all the IRI can be correlated with related IRI of the same Target Communication.

R6.4 - 40	CC to CC Correlation - The CSP shall be able to deliver information such that all the CC can be correlated with related CC of the same Target Communication.

R6.4 - 50	IRI and CC Correlation - The CSP shall be able to deliver information such that the related IRI and CC of the same Target Communication can be correlated.

R6.4 - 60	POI Identification - The CSP shall be able to report to the LEA the POI source(s) of the Interception Product.

R6.4 - 70	Delivery Reliability - The CSP shall be able to employ mechanisms (e.g. buffering) to limit the effect of delivery network failures or limitations to prevent loss of Interception Product.

R6.4 - 80	Delivery Latency - The CSP shall ensure that the Interception Product is delivered to the LEA without undue delay as defined by mutual agreement between the CSP and the LEA.

R6.4 - 90	Timestamping at Capture - The CSP shall timestamp the Interception Product (both IRI and CC) at capture (at the POI) with a timestamp of precision, resolution, and accuracy commensurate with the performance of the intercepted service.

R6.4 - 100	Timestamping at Delivery - The CSP shall provide, where required for correlation purposes, the timestamp of the Interception Product (both IRI and CC) at the Mediation and Delivery Function (MDF) as sent to the LEMF, with a timestamp of precision, resolution, and accuracy commensurate with the performance of the intercepted service.

R6.4 - 110	UTC - The CSP shall provide all timestamps in UTC (including local offset).

R6.4 - 120	Trusted Time - The CSP shall utilise a trusted time source for all LI related functions.

R6.4 - 130	Separate delivery of services - The CSP shall be able to support delivering Interception Product for a particular service separately from other services' Interception Product (e.g. delivering SMS Interception Product independent of CS Voice Interception Product).

R6.4 - 140	Ordering - The CSP shall provide a means to enable the LEA to order the events of an intercepted service.

R6.4 - 150	Duplication - The CSP shall endeavour to limit duplicate delivery of Interception Product.

R6.4 - 160	Encryption - The CSP shall remove any encryption it provides or manages before delivery of the Interception Product to the LEA, or shall provide the LEA the information necessary to decrypt the intercepted communications (e.g. keys, algorithms, parameters) included with the Interception Product.

R6.4 - 170	CSP provided Encryption Keys - If the CSP provides encryption keys to the target, but is not involved in the encryption service, the CSP shall provide the keys to the LEA.

R6.4 – 175  	CSP provided cryptographic parameters in roaming – When a home CSP’s subscriber is roaming, independently of whether or not the subscriber is an LI Target in the VPLMN, the home CSP shall provide to the visited CSP the means to decrypt user services which are encrypted between the ME and an entity outside the visited CSP and using cryptographic parameters established in the home CSP.

R6.4 - 180	Retroactive Decryption - The CSP shall ensure that the crypto keys, algorithm and parameters delivered to the LEA enable the LEA to decrypt encrypted Target Communications retroactively.

R6.4 - 190	Mid Communication Interception - The CSP shall retain sufficient key material for the duration of any communications such that it is possible to decrypt already on going communications, when using CSP provided or managed encryption.

R6.4 - 200	Encryption Key Material Lifecycle - Destruction – Once key material specifically retained for LI purposes is no longer required, the CSP shall securely delete this key material.

R6.4 - 210	Encoding - The CSP shall be able to remove any specific CSP-controlled encoding before delivery to the LEA, or provide the LEA the information necessary to decode the intercepted communications concurrently with delivery of LI product.

R6.4 - 220	Compression - The CSP shall be able to remove any specific CSP-controlled compression before delivery to the LEA, or provide the LEA the information necessary to decompress the intercepted communications concurrently with delivery of LI product.

R6.4 -230	Target Identifier Provenance – The CSP shall be able to indicate, for each target identifier provided to the LEA in the Interception Product, the provenance of the identifier, specifically, whether the identifier was provided to the CSP by the LEA (in the initial warrant), whether it was observed in the intercepted communications, whether it was matched on by the function performing the isolation of communications, and whether it was associated with the target.

*** End of All MODIFICATIONS ***