3GPP TSG-SA3 Meeting #80-LI-e-a	s3i210006

# Online, , 25th Jan 2021 - 29th Jan 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0153 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GUTI allocation procedure reporting correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (BT plc) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI16 |  |  |  |  |  | Date: |  |  | 2021-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The specification does not correctly describe the necessary IEF behaviour to generate IEF to ICF assocation events especially in error scenarios when the 5G-GUTI allocation procecures from AMF to UE do not complete successfully. In addition the ICF stage 3 description does not correctly describe IEF assocation handling at the ICF and existing solution may lead to premature deletion of associations in some scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IEF event generation timing is more precisely specified to always occur when a 5G-GUTI allocation procedure is triggered in the AMF regardless of whether it completes succesfully. In addition, ICF assocation and cache deletion description text is improved. The CR also provides alignment of functionality with stage 2 in 33.127. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LI functionality as defined in the present version of the TS will continue not to fully meet regulatory requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.2.2A.2.1, 7.6.2.3, 7.6.2.4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**********************START OF CHANGES********************

##### 6.2.2A.2.1	Events

The IEF in the AMF shall generate an IEFIdentifierAssociation record whenever the IEF present in the AMF detects a change in association between a SUPI and a 5G-GUTI for any UE registered with the AMF. The IEF shall send the IEFIdentifierAssociation records to the ICF over LI_XER as defined in clause 5.9.

Accordingly, the IEF in the AMF generates IEFIdentifierAssociation records when any of the following events are detected:

-	IEFAssociationRecord:  Association of a 5G-GUTI to a SUPI, (this may also include SUCI to SUPI association).

-	IEFDeassociationRecord: De-association of a 5G-GUTI from a SUPI.

NOTE1:	The de-association of 5G-GUTI from a SUPI event record is only generated if a new 5G-GUTI is not allocated to a SUPI to update a previous association (e.g. at inter-AMF handover).

NOTE 2:	As SUCIs are single use and only valid for a single authentication, they are only be valid at the single point in time when the association event is detected and reported to the ICF by the IEF.

In addition, when an IEF is activated as per clause 6.2.2A.1, the IEF shall generate associations event for all SUPIs which are registered in the AMF, where those identifier associations allocated prior to IEF activation remain current and are still available in the AMF (See NOTE 2).

NOTE 3:	Only identifier associations which have been maintained by the AMF as part of normal network operations will be available.

In the case where the IEF in the AMF detects that a REGISTRATION ACCEPT message or a CONFIGURATION UPDATE (5G-GUTI) message as defined in TS 24.501 [13] has been sent by the AMF towards a UE, the IEF shall immediately generate an IEFIdentifierAssociation record. This record shall be generated regardless of whether the CONFIGURATION UPDATE (5G-GUTI) procedure is subsequently successfully completed or not.

#### ********************************NEXT CHANGE*******************************

#### 7.6.2.3	ICF Query and Response over LI_XQR

When the ICF receives an identifier association query request from the IQF, the ICF shall search the cached identifier associations to establish a match, based on RequestValues received in the request (see clause 5.8), subject to clause 7.6.2.4.

Upon successful matching of one or more identifier associations which were valid at the observed time specified in the query, the ICF shall provide a response to the IQF using the IdentityAssociationResponse message as defined in clause 5.8. Where the ICF is not able to provide a single identifier association based on the RequestValues, the IQF is responsible for any subsequent handling of multiple identifier associations in terms of whether to provide all associations to the LEA over LI_HIQR.

#### ********************************NEXT CHANGE*******************************

#### 7.6.2.4	ICF Identifier Association Event Handling

Upon receipt of an IEFIdentifierAssociation record as defined in clause 6.2.2A.2, the ICF shall cache the identifier association(s) contained within the record as followings:

-	Where the IEFIdentifierAssociation record contains a new SUPI to SUCI association, the association shall be cached until the maximum cache duration is reached.

-	Where no current SUPI to 5G-GUTI association is held in the ICF cache, the identifier association shall be held until a subsequent SUPI to 5G-GUTI association update is received or the ICF maximum active association lifetime is exceeded (See NOTE 1).

-	Where a previous SUPI to 5G-GUTI association is held in the cache, the previous association shall be subsequently cached until the maximum cache duration is reached and the new identifier association shall be held until a subsequent SUPI to 5G-GUTI association update is received or the ICF maximum active association lifetime is exceeded (See NOTE 1).

-	Where a SUPI to 5G_GUTI association is indicated as de-associated by an IEF but no new 5G-GUTI has been allocated, the association shall be subsequently cached until the maximum cache duration is reached.

NOTE 1:	The ICF maximum active association lifetime ensures that assocations which have not been correctly disassociated (e.g. due to message loss or UE error conditions) do not remain in the ICF longer than they would remain valid for a UE in idle mode, under normal network conditions.While the exact value of the maximum active association lifetime is outside the scope of the present document, selection of this value by the CSP needs to take into consideration, among other aspects, the network default UE re-authentication time in the AMF and the maximum short-term cache time for disassociated identifier associations.

It is recommended that the maximum active association lifetime is at least as long as the AMF network default UE re-authentication time plus the maximum  short-term cache time (i.e. the maximum time an assocation would remain in the ICF for a UE on its home network in idle mode, under normal network conditions).

Where a new SUPI to 5G-GUTI association replaces a previous association, the ICF shall store the time of disassociation for that previous association in the cache.

Where an IEFIdentifierAssociation record contains a PEI or a TAI list, the ICF shall store the received values and associate them both the current received SUPI to 5G-GUTI association (including SUPI to SUCI association where also present in the record) and any future association until:

-	A subsequent IEFIdentifierAssociation record is receive which updates the PEI or TAI list values.

-	The old PEI / TAI list shall be retained in association with previous SUPI to 5G-GUTI or SUCI associations until those associations are deleted from cache.

-	New PEI / TAI list shall be used in association with both the association(s) with which it was received and any subsequent associations until another update is received.

-	All SUPI associations for which the PEI / TAI list is valid are deleted from the cache.

When the ICF receives a query request from the IQF as defined in clause 7.6.2.3, the ICF shall search available identifier associations (both active associations and those marked for deletion in the cache) for a match. The ICF shall be able to use both time and TAI (as a single TAI and in relation to a TAI list) to identity the correct SUPI to 5G-GUTI or SUCI association(s). For associations which have been disassociated (and will be deleted once the maximum cache duration is reached), the time of disassociation is used by the ICF to identify the correct association match (based on observed time in LEA request), where multiple associations are held in the cache.

NOTE 2:	Use of nCGI to match associations based on physical location for SUCI / 5G-S-TMSI to SUPI requests, is out of scope of the present document.

As the LEA and CSP are unlikely to have synchronised the time of identifier observation / association provided by the LEA in the query request, with NF time of the IEFs, the ICF shall search the cached identifier associations using a short window time duration both before and after (subject to maximum cache duration) the observed time provided by the LEA in the RequestValues over LI_XQR.

NOTE 3:	The search window duration before and after the LEA provided observed time value is outside the scope of the present document. The use of such a window is constrained by the availability of associations in the cache at any given point in time.

NOTE 4:	While the value of the maximum short-term caching time is outside the scope of the present document, selection of this value by the CSP needs to take into consideration, among other aspects, the duration of potential period of recovery from a 5G-GUTI update error, in order to prevent previous assocations being deleted before they have been fully disassociated by both the UE and AMF.

**********************END OF CHANGES********************