3GPP TSG-SA3 Meeting #80-LI-e-a	s3i210036

# Online, , 25th Jan 2021 - 29th Jan 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | 0424 | rev | - | Current version: | 14.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS33.108 Annex B.3 reference clauses correction of OMA MLP TS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ofcom (CH), Rogers Communications Canada) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI14 |  |  |  |  |  | Date: |  |  | 2021-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The OMA MLP TS reference clauses in the ASN.1 ExtendedLocParameters structure are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The OMA MLP TS reference clauses used in the ASN.1 ExtendedLocParameters structure of the UmtsHI2Operations module are not aligned with the specified OMA MLP TS version. It is proposed to update the OMA MLP TS version and to align the corresponding comments with the correct reference clauses in the ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The OMA MLP TS reference clauses will remain incorrect and lead to wrong implementation by the CSP and wrong decoding by the LEMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[3]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[4]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[5A]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[5B]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[5C]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[5D]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[6]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

NOTE 1:	It is recommended that for [5A], [5B], [5C], [5D] and [6] the 2002 specific versions should be used.

[7]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[8]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI realizations - Remote Operations Service Element (ROSE) protocol specification".

NOTE 2:	It is recommended that for [8] the 1994 specific versions should be used.

[9]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[10] - [12]	Void.

[13]	IETF STD 9 (RFC 0959): "File Transfer Protocol (FTP)".

[14]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[15]	IETF STD0005 (RFC 0791: "Internet Protocol".

[16]	IETF STD0007 (RFC 0793): "Transmission Control Protocol".

[17]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[18]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[19]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[20]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[21] – [22]	Void.

[23]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[24]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[25]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[26]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[27]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[28]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[29]	ITU-T Recommendation Q.763: "Signalling System No. 7 - ISDN User Part formats and codes".

[30]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[31]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

NOTE 3:	Reference [31] is specific, because ASN.1 parameter "release-Reason-Of-Intercepted-Call" has the following comment: "Release cause coded in [31] format". In case later version than the given one indicated for ISDN specification ETSI EN 300 403-1 has modified format of the "release cause", keeping the reference version specific allows to take proper actions in later versions of this specification.

[32] - [33]	Void

[34]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[35]	Void.

[36]	Void.

[37]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[38]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[39]	ISO 3166-1: "Codes for the representation of names of countries and their subdivisions - Part 1: Country codes".

[40]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[41]	3GPP TS 29.234: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals: 3GPP System to Wireless Local Area Network (WLAN) interworking; Stage 3".

[42]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[43]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[44]	3GPP TS 23.401: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[45]	3GPP TS 23.402: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for non-3GPP accesses".

[46]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Access (GPRS) Tunneling Protocol for Control Plane (GTPv2-C); Stage 3".

[47]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[48]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunneling protocols; Stage 3".

[49]	3GPP TS 24.303: "Mobility management based on Dual-Stack Mobile IPv6; Stage 3".

[50]	(void)

[51]	(void)

[52]	3GPP TS 24.147: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[53]	3GPP TS 29.273: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[54]	3GPP TS 33.328: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) media plane security".

[55]	ATIS-0700005 "Lawfully Authorized Electronic Surveillance (LAES) for 3GPP IMS-based VoIP and other Multimedia Services".

[56]	3GPP TS 29.212: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Policy and Charging Control(PCC); Reference points".

[57]	Void.

[58]	IETF RFC 4217: "Securing FTP with TLS".

[59]	3GPP TS 29.272: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[60]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[61]	IETF RFC 6043: "MIKEY-TICKET: Ticket-Based Modes of Key Distribution in Multimedia Internet KEYing (MIKEY)", available at www.ietf.org

[62]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[63]	3GPP TS 29.279: "Mobile IPv4 (MIPv4) based mobility protocols; Stage 3".

[64]	3GPP TS 29.118: "Mobility Management Entity (MME) –Visitor Location Register (VLR) SGs interface specification"

[65]	ANSI/J-STD-025-B: "Lawfully Authorized Electronic Surveillance", July 17, 2006.

[66]	3GPP TS 24.007: "Mobile Radio Interface Signalling Layer 3; General Aspects".

[67]	IETF RFC 3966: "The Tel URIs for Telephone Numbers", December, 2004.

[68]	IETF RFC 791: "Internet Protocol"

[69]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[70]	IEFT RFC 3697: "IPv6 Flow Label Specification".

[71]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[72]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[73]	ISO.3166-2: International Organization for Standardization, "Codes for the representation of names of countries and their subdivisions - Part 2: Country subdivision code".

[74]	UPS SB42-4: Universal Postal Union (UPU), "International Postal Address Components and Templates".

[75]	ISO 639-1:2002: "Codes for the representation of names of languages -- Part 1: Alpha-2 code".

[76]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[77]	3GPP TS 24.623: "Technical Specification Group Core Network and Terminals; Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[78]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[79]	3GPP TS 24.109: "Universal Mobile Telecommunications System (UMTS); Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[80]	IETF RFC 4825:"The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[81]	IETF RFC 7254: "A Uniform Resource Name Namespace for the Global System for Mobile Communications Association (GSMA) and the International Mobile station Equipment Identity (IMEI)"

[82]	IETF RFC 7255: "Using the International Mobile station Equipment Identity (IMEI) Uniform Resource Name (URN) as an Instance ID".

[83]	3GPP TS 22.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[84]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[85]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[86]	3GPP TS 24.371: " Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3".

[87]	ITU-T Recommendation E.212: "The international identification plan for public networks and subscriptions".

[88]	OMA MLP TS: "Mobile Location Protocol", OMA-TS-MLP-V3_5-20181211-C.

[89]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[90]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[91]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[92]	IETF RFC 2822: "Internet Message Format".

[93]	IETF RFC 3551: "RTP Profile for Audio and Video Conferences with Minimal Control".

[94]	IETF RFC 4566: "Session Description Protocol".

[95]	IETF RFC 3550: "Realtime Transport Protocol".

[96]				3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details".

END OF FIRST CHANGE

START OF SECOND CHANGE

# B.3	Intercept related information (HI2 PS and IMS)

Declaration of ROSE operation umts-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data UmtsIRIsContent must be considered.

# ASN1 description of IRI (HI2 interface)

UmtsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2(1)  r14 (14) version-3 (3)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version18(18)}; -- Imported from TS 101 671v3.14.1

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2DomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2(1)  r14 (14) version-3 (3)}

umts-sending-of-IRI  OPERATION ::=

{

ARGUMENT	UmtsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2(1) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

UmtsIRIsContent		::= CHOICE

{

umtsiRIContent			UmtsIRIContent,

umtsIRISequence			UmtsIRISequence

}

UmtsIRISequence		::= SEQUENCE OF UmtsIRIContent

-- Aggregation of UmtsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- UmtsIRIContent needs to be chosen.

UmtsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- This values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2DomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 domain

iRIversion				[23] ENUMERATED

{

version2 (2),

...,

version3 (3),

version4 (4),

-- note that version5 (5) cannot be used as it was missed in the version 5 of this

-- ASN.1 module.

version6 (6),

-- vesion7(7) was ommited to align with ETSI TS 101 671.

lastVersion (8)	} OPTIONAL,

-- Optional parameter "iRIversion" (tag 23) was always redundant in 33.108, because

-- the object identifier "hi2DomainId" was introduced into "IRI Parameters" in the

-- initial version of 33.108v5.0.0. In order to keep backward compatibility, even when

-- the version of the "hi2DomainId" parameter will be incremented it is recommended

-- to always send to LEMF the same: enumeration value "lastVersion(8)".

-- if not present, it means version 1 is handled

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

gPRSCorrelationNumber	[18] GPRSCorrelationNumber OPTIONAL,

gPRSevent 				[20] GPRSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

-- Coded according to 3GPP TS 29.002 [4] and 3GPP TS 23.003 25].

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

mediaDecryption-info	[36] MediaDecryption-info OPTIONAL,

servingS4-SGSN-address	[37] OCTET STRING OPTIONAL,

-- Diameter Origin-Host and Origin-Realm of the S4-SGSN based on the TS 29.272 [59].

-- Only the data fields from the Diameter AVPs are provided concatenated

-- with a semicolon to populate this field.

sipMessageHeaderOffer   [38] OCTET STRING OPTIONAL,

sipMessageHeaderAnswer  [39] OCTET STRING OPTIONAL,

sdpOffer                [40] OCTET STRING OPTIONAL,

sdpAnswer               [41] OCTET STRING OPTIONAL,

uLITimestamp            [42] OCTET STRING (SIZE (8)) OPTIONAL,

-- Coded according to 3GPP TS 29.060 [17]; Only the ULI Timestamp value is reported.

packetDataHeaderInformation		 [43] PacketDataHeaderInformation	OPTIONAL,

mediaSecFailureIndication       [44] MediaSecFailureIndication OPTIONAL,

pANI-Header-Info		[45] SEQUENCE OF PANI-Header-Info		OPTIONAL,

-- information extracted from P-Access-Network-Info headers of SIP message;

-- described in TS 24.229 §7.2A.4 [76]

imsVoIP					[46] IMS-VoIP-Correlation	OPTIONAL,

xCAPmessage				[47] OCTET STRING OPTIONAL,

-- The entire HTTP contents of any of the target's IMS supplementary service setting

-- management or manipulation XCAP messages, mainly made through the Ut

-- interface defined in the 3GPP TS 24 623 [77].

ccUnavailableReason		[48] PrintableString	OPTIONAL,

carrierSpecificData					[49] OCTET STRING OPTIONAL,

-- Copy of raw data specified by the CSP or his vendor related to HSS.

current-Previous-Systems			[50] Current-Previous-Systems OPTIONAL,

change-Of-Target-Identity				[51] Change-Of-Target-Identity OPTIONAL,

requesting-Network-Identifier		[52] OCTET STRING OPTIONAL,

-- the requesting network identifier PLMN id (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

requesting-Node-Type					[53] Requesting-Node-Type OPTIONAL,

serving-System-Identifier				[54] OCTET STRING OPTIONAL,

-- the requesting network identifier (Mobile Country Code and Mobile Network Country,

-- defined in E212 [87]).

extendedLocParameters 	[55] 	ExtendedLocParameters OPTIONAL, -- LALS extended parameters

locationErrorCode		[56]	LocationErrorCode OPTIONAL,	-- LALS error code

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules

-- PARAMETERS FORMATS

PANI-Header-Info::= SEQUENCE

{

access-Type 		[1] OCTET STRING 		OPTIONAL,

-- ASCII chain '3GPP-GERAN',... : see TS 24.229 §7.2A.4 [76]

access-Class  		[2] OCTET STRING		OPTIONAL,

-- ASCII chain'3GPP-GERAN',... : see TS 24.229 §7.2A.4 [76]

network-Provided	[3] NULL				OPTIONAL,

-- present if provided by the network

pANI-Location		[4] PANI-Location 		OPTIONAL,

...

}

PANI-Location  ::= SEQUENCE

{

raw-Location     [1] OCTET STRING	OPTIONAL,

-- raw copy of the location string from the P-Access-Network-Info header

location          [2] Location     	OPTIONAL,

...

}

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

gPRS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei					[1] OCTET STRING (SIZE (8)) OPTIONAL,

-- See MAP format [4]

imsi					[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

msISDN					[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

e164-Format				[7] OCTET STRING    (SIZE (1 .. 25)) OPTIONAL,

-- E164 address of the node in international format. Coded in the same format as

-- the calling party number  parameter of the ISUP (parameter part:[29])

sip-uri					[8] OCTET STRING 	OPTIONAL,

-- See [26]

...,

tel-uri					[9] OCTET STRING 	OPTIONAL,

-- See [67]

x-3GPP-Asserted-Identity [10] OCTET STRING 	OPTIONAL,

-- X-3GPP-Asserted-Identity header (3GPP TS 24.109 [79]) of the target, used in

-- some XCAP transactions. This information complement SIP URI or Tel URI of the target.

xUI						[11] OCTET STRING 	OPTIONAL

-- XCAP User Identifier (XUI)is a string, valid as a path element in an XCAP URI, that

-- may be associated with each user served by a XCAP resource server. Defined in IETF

-- RFC 4825[80]. This information may complement SIP URI or Tel URI of the target.

},

services-Data-Information 	[4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the

-- complementary information associated to the basic data call

...

}

Location	::= SEQUENCE

{

e164-Number         [1] OCTET STRING (SIZE (1..25)) OPTIONAL,
        -- Coded in the same format as the ISUP location number (parameter
        -- field) of the ISUP (see EN 300 356 [30]).    
	globalCellID		[2] GlobalCellID	OPTIONAL,

--see MAP format (see [4])

rAI					[4] Rai		OPTIONAL,

-- the Routeing Area Identifier in the current  SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used)

gsmLocation			[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] Sai	OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 – 3)

--			LAC		2 octets (no. 4 – 5)

--			SAC		2 octets (no. 6 – 7)

--			(according to 3GPP TS 25.413 [62])

...,

oldRAI				[8] Rai		OPTIONAL,

-- the Routeing Area Identifier in the old SGSN is coded in accordance with the

-- § 10.5.5.15 of document [9] without the Routing Area Identification IEI

-- (only the last 6 octets are used).

tAI					[9] OCTET STRING (SIZE (6))	OPTIONAL,

-- The TAI is coded according to the TS 29.118 [64] without the TAI IEI.

-- The tAI parameter is applicable only to the CS traffic cases where

-- the available location information is the one received from the the MME.

eCGI				[10] OCTET STRING (SIZE (8)) OPTIONAL,

-- the ECGI is coded according to the TS 29.118 [64] without the ECGI IEI.

-- The eCGI parameter is applicable only to the CS traffic cases where

-- the available location information is the one received from the the MME.

civicAddress		[11] CivicAddress OPTIONAL

-- Every elements that describe civicAddress are based on IETF RFC 4776 or IETF

-- 5139, ISO.3166-1 and –2, ISO 639-1, UPU SB42-4 ([71]to [75]) Such element is to

-- enrich IRI

-- Messages to LEMF by civic elements on the location of a H(e)NodeB or a WLAN hotspot,

-- instead of geographical location  of the target or any geo-coordinates. Please, look

-- at the §5.11 location information of TS 33.106 and §4 functional architecture of TS

-- 33.107 on how such element can be used.

}

GlobalCellID	::= OCTET STRING  (SIZE (5..7))

Rai				::= OCTET STRING (SIZE (6))

Sai				::=	OCTET STRING (SIZE (7))

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude		[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude		[2]	PrintableString (SIZE(8..11)),

-- format : 	XDDDMMSS.SS

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 		[1] PrintableString (SIZE(10)),

utm-North 		[2] PrintableString (SIZE(7)),

-- example	utm-East	32U0439955

--			utm-North	5540736

mapDatum		[3]	MapDatum DEFAULT wGS84,

...,

azimuth			[4] INTEGER (0..359) OPTIONAL

-- The azimuth is the bearing, relative to true north.

},

utmRefCoordinates  	[3] SEQUENCE

{

utmref-string		PrintableString (SIZE(13)),

mapDatum			MapDatum DEFAULT wGS84,

...

},

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING

-- format is as defined in [37].

}

MapDatum ::= ENUMERATED

{

wGS84,

wGS72,

eD50, 	-- European Datum 50

...

}

UMTSLocation ::= CHOICE {

point					[1]	GA-Point,

pointWithUnCertainty	[2]	GA-PointWithUnCertainty,

polygon					[3]	GA-Polygon

}

GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints						INTEGER ::= 15

GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

CivicAddress ::= CHOICE {

detailedCivicAddress        SET OF DetailedCivicAddress,

xmlCivicAddress				XmlCivicAddress,

...

}

XmlCivicAddress ::= UTF8String

-- Must conform to the February 2008 version of the XML format on the representation of

-- civic location described in IETF RFC 5139[72].

DetailedCivicAddress ::= SEQUENCE {

building			[1] UTF8String OPTIONAL,

-- Building (structure), for example Hope Theatre

room				[2] UTF8String OPTIONAL,

-- Unit (apartment, suite), for example 12a

placeType			[3] UTF8String OPTIONAL,

-- Place-type, for example office

postalCommunityName	[4] UTF8String OPTIONAL,

-- Postal Community Name, for example Leonia

additionalCode	[5] UTF8String OPTIONAL,

-- Additional Code, for example 13203000003

seat				[6] UTF8String OPTIONAL,

-- Seat, desk, or cubicle, workstation, for example WS 181

primaryRoad		[7] UTF8String OPTIONAL,

-- RD is the primary road name, for example Broadway

primaryRoadDirection   [8] UTF8String OPTIONAL,

-- PRD is the leading road direction, for example N or North

trailingStreetSuffix [9] UTF8String OPTIONAL,

-- POD or trailing street suffix, for example SW or South West

streetSuffix		[10] UTF8String OPTIONAL,

-- Street suffix or type, for example Avenue or Platz or Road

houseNumber			[11] UTF8String OPTIONAL,

-- House number, for example 123

houseNumberSuffix	[12] UTF8String OPTIONAL,

-- House number suffix, for example A or Ter

landmarkAddress		[13] UTF8String OPTIONAL,

-- Landmark or vanity address, for example Columbia University

additionalLocation	[114] UTF8String OPTIONAL,

-- Additional location, for example South Wing

name 			[15] UTF8String OPTIONAL,

-- Residence and office occupant, for example Joe's Barbershop

floor 			[16] UTF8String OPTIONAL,

-- Floor, for example 4th floor

primaryStreet	[17] UTF8String OPTIONAL,

-- Primary street name, for example Broadway

primaryStreetDirection   [18] UTF8String OPTIONAL,

-- PSD is the leading street direction, for example N or North

roadSection		[19] UTF8String OPTIONAL,

-- Road section, for example 14

roadBranch    	[20] UTF8String OPTIONAL,

-- Road branch, for example Lane 7

roadSubBranch  	[21] UTF8String OPTIONAL,

-- Road sub-branch, for example Alley 8

roadPreModifier [22] UTF8String OPTIONAL,

-- Road pre-modifier, for example Old

roadPostModifier [23] UTF8String OPTIONAL,

-- Road post-modifier, for example Extended

postalCode		[24]UTF8String OPTIONAL,

-- Postal/zip code, for example 10027-1234

town			[25] UTF8String OPTIONAL,

county			[26] UTF8String OPTIONAL,

-- An administrative sub-section, often defined in ISO.3166-2[74]  International

-- Organization for Standardization, "Codes for the representation of names of

-- countries and their subdivisions –  Part 2: Country subdivision code"

country			[27] UTF8String,

-- Defined in ISO.3166-1 [39] International Organization for Standardization, "Codes for

-- the representation of names of countries and their subdivisions – Part 1: Country

-- codes". Such definition is not optional in case of civic address. It is the

-- minimum information needed to qualify and describe a civic address, when a

-- regulation of a specific country requires such information

language		[28] UTF8String,

-- Language defined in the IANA registry according to the assignments found

-- in the standard ISO 639 Part 1, "ISO 639-1:2002[75], Codes for the representation of

-- names of languages – Part 1: Alpha-2 code" or using assignments subsequently made

-- by the ISO 639 Part 1 maintenance agency

...

}

SMS-report		::= SEQUENCE

{

sMS-Contents	[3] SEQUENCE

{

sms-initiator		[1] ENUMERATED 	-- party which sent the  SMS

{

target			(0),

server			(1),

undefined-party	(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer	(0), 		-- the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined			(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED 	-- in case of terminating call, indicates if

-- the server will send other SMS

{

yes			(0),

no			(1),

undefined	(2),

...

} OPTIONAL,

content				[4] OCTET STRING (SIZE (1 .. 270)) OPTIONAL,

-- Encoded in the format defined for the SMS mobile

...

}

}

GPRSCorrelationNumber ::= OCTET STRING (SIZE(8..20))

CorrelationValues ::= CHOICE {

iri-to-CC 	[0]		IRI-to-CC-Correlation, -- correlates IRI to Content(s)

iri-to-iri	[1]		IRI-to-IRI-Correlation, -- correlates IRI to IRI

both-IRI-CC	[2]		SEQUENCE { -- correlates IRI to IRI and IRI to Content(s)

iri-CC	[0]	IRI-to-CC-Correlation,

iri-IRI	[1]	IRI-to-IRI-Correlation}

}

IMS-VoIP-Correlation ::= SET OF SEQUENCE {

ims-iri		[0]		IRI-to-IRI-Correlation,

ims-cc		[1]		IRI-to-CC-Correlation 		OPTIONAL

}

IRI-to-CC-Correlation ::= SEQUENCE { -- correlates IRI to Content

cc 		[0] SET OF 	OCTET STRING,-- correlates IRI to multiple CCs

iri 	[1] 		OCTET STRING OPTIONAL

-- correlates IRI to CC with signaling

}

IRI-to-IRI-Correlation ::= OCTET STRING -- correlates IRI to IRI

GPRSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

pDPContextModification					(13),

servingSystem							(14),

... ,

startOfInterceptionWithMSAttached		(15),

packetDataHeaderInformation				(16) ,	hSS-Subscriber-Record-Change			(17),

registration-Termination				(18),

-- FFS

location-Up-Date						(19),

-- FFS

cancel-Location							(20),

register-Location						(21),

location-Information-Request			(22)

}

-- see [19]

IMSevent ::= ENUMERATED

{

unfilteredSIPmessage (1),

-- This value indicates to LEMF that the whole SIP message is sent , i.e. without filtering

-- CC; location information is removed by the DF2/MF if not required to be sent.

...,

sIPheaderOnly (2),

-- If warrant requires only IRI then specific content in a 'sIPMessage'

-- (e.g. 'Message', etc.) has been deleted before sending it to LEMF.

decryptionKeysAvailable (3) ,

-- This value indicates to LEMF that the IRI carries CC decryption keys for the session

-- under interception.

startOfInterceptionForIMSEstablishedSession  (4) ,

-- This value indicates to LEMF that the IRI carries information related to

-- interception started on an already established IMS session.

xCAPRequest (5),

-- This value indicates to LEMF that the XCAP request is sent.

xCAPResponse (6) ,

-- This value indicates to LEMF that the XCAP response is sent.

ccUnavailable (7)

-- This value indicates to LEMF that the media is not available for interception for intercept

-- orders that requires media interception.

}

Current-Previous-Systems ::= SEQUENCE

{

serving-System-Identifier			[1] OCTET STRING OPTIONAL,

-- VPLMN id (Mobile Country Code and Mobile Network Country, E. 212 number [87]).

current-Serving-SGSN-Number			[2] OCTET STRING OPTIONAL,

-- E.164 number of the serving SGSN.

current-Serving-SGSN-Address		[3] OCTET STRING OPTIONAL,

-- The IP address of the serving SGSN or the Diameter Origin-Host and Origin-Realm of the

-- serving S4 SGSN.

current-Serving-S4-SGSN-Address			[4]OCTET STRING OPTIONAL,

-- The Diameter Origin-Host and Origin-Realm of the previous serving S4 SGSN.

previous-Serving-System-Identifier		[5] OCTET STRING OPTIONAL,

-- VPLMN id (Mobile Country Code and Mobile Network Country, defined in E212 [87]).

previous-Serving-SGSN-Number				[6] OCTET STRING OPTIONAL,

-- The E.164 number of the previous serving MSC.

previous-Serving-SGSN-Address			[7] OCTET STRING OPTIONAL,

-- The IP address of the previous serving MSC or its Diameter Origin-Host and Origin-Realm.

previous-Serving-S4-SGSN-Address			[8]OCTET STRING OPTIONAL,

-- The Diameter Origin-Host and Origin-Realm of the previous serving S4 SGSN.

...

}

Change-Of-Target-Identity ::= SEQUENCE

{

new-MSISDN								[1] PartyInformation OPTIONAL,

-- new MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

old-MSISDN				[2] PartyInformation OPTIONAL,

-- new MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document TS 29.002 [4]

new-IMSI 								[3] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

old-IMSI 								[4] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Station Identity E.212 number beginning with Mobile Country Code

new-IMEI 								[5] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Equipement Identity defined in MAP format document TS 29.002 [4]

old-IMEI 								[6] PartyInformation OPTIONAL,

-- See MAP format [4] International Mobile

-- Equipement Identity defined in MAP format document TS 29.002 [4]

...,

new-IMPI							[7] PartyInformation OPTIONAL,

old-IMPI							[8] PartyInformation OPTIONAL,

new-SIP-URI							[9]	PartyInformation OPTIONAL,

old-SIP-URI							[10] PartyInformation OPTIONAL,

new-TEL-URI							[11] PartyInformation OPTIONAL,

old-TEL-URI							[12] PartyInformation OPTIONAL

}

Requesting-Node-Type ::= ENUMERATED

{

mSC				(1),

sMS-Centre		(2),

gMLC			(3),

mME				(4),

sGSN			(5),

...

}

Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

-- The Access Point Name (APN) is coded in accordance with

-- 3GPP TS 24.008 [9] without the APN IEI (only the last 100 octets are used).

-- Octets are coded according to 3GPP TS 23.003 [25].

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

-- Include either Octets 3 and 4 of the Packet Data Protocol Address information element of

-- 3GPP TS 24.008 [9]or Octets 4 and 5 of the End User Address IE of 3GPP TS 29.060 [17].

-- when PDP-type is IPv4 or IPv6, the IP address is carried by parameter

-- pDP-address-allocated-to-the-target

-- when PDP-type is IPv4v6, the additional IP address is carried by parameter

-- additionalIPaddress

...,

nSAPI					[4] OCTET STRING (SIZE (1)) OPTIONAL,

-- Include either Octet 2 of the NSAPI IE of 3GPP TS 24.008 [9] or Octet 2 of the NSAPI IE of

-- 3GPP TS 29.060 [17].

additionalIPaddress		[5] DataNodeAddress OPTIONAL

}

GPRSOperationErrorCode ::= OCTET STRING

-- The parameter shall carry the GMM cause value or the SM cause value, as defined in the

-- standard [9], without the IEI.

LDIevent ::= ENUMERATED

{

targetEntersIA 			(1),

targetLeavesIA			(2),

...

}

UmtsQos ::= CHOICE

{

qosMobileRadio [1] OCTET STRING,

-- The qosMobileRadio parameter shall be coded in accordance with the § 10.5.6.5 of

-- document [9] without the Quality of service IEI and Length of

-- quality of service IE (. That is, first

-- two octets carrying 'Quality of service IEI' and 'Length of quality of service

-- IE' shall be excluded).

qosGn [2] OCTET STRING

-- qosGn parameter shall be coded in accordance with § 7.7.34 of document [17]

}

MediaDecryption-info ::= SEQUENCE OF CCKeyInfo

-- One or more key can be available for decryption, one for each media streams of the

-- intercepted session.

CCKeyInfo ::= SEQUENCE

{

cCCSID	 [1]	OCTET STRING,

-- the parameter uniquely mapping the key to the encrypted stream.

cCDecKey [2]	OCTET STRING,

cCSalt   [3]    OCTET STRING OPTIONAL,

-- The field reports the value from the CS_ID field in the ticket exchange headers as
        -- defined in IETF RFC 6043 [61].

...

}

MediaSecFailureIndication  ::= ENUMERATED

{

genericFailure  (0),

...

}

PacketDataHeaderInformation ::= CHOICE

{

packetDataHeader		[1]	PacketDataHeaderReport,

packetDataSummary	[2]	PacketDataSummaryReport,

...

}

PacketDataHeaderReport ::= CHOICE

{

packetDataHeaderMapped	[1]	PacketDataHeaderMapped,

packetDataHeaderCopy	[2]	PacketDataHeaderCopy,

...

}

PacketDataHeaderMapped ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the "Protocol" field and for IPv6 report "Next Header" field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

packetsize				[6] INTEGER OPTIONAL,

flowLabel				[7] INTEGER OPTIONAL,

packetCount				[8] INTEGER OPTIONAL,

direction				[9] TPDU-direction,

...

}

TPDU-direction ::= ENUMERATED

{

from-target 	(1),

to-target 		(2),

unknown 		(3)

}

PacketDataHeaderCopy ::= SEQUENCE

{

direction				[1] TPDU-direction,

headerCopy				[2] OCTET STRING,	-- includes a copy of the packet header at the IP

-- network layer and above including extension headers, but excluding contents.

...

}

PacketDataSummaryReport ::= SEQUENCE OF PacketFlowSummary

PacketFlowSummary ::= SEQUENCE

{

sourceIPAddress			[1] IPAddress,

sourcePortNumber		[2] INTEGER (0..65535) OPTIONAL,

destinationIPAddress	[3] IPAddress,

destinationPortNumber	[4] INTEGER (0..65535) OPTIONAL,

transportProtocol		[5] INTEGER,

-- For IPv4, report the "Protocol" field and for IPv6 report "Next Header" field.

-- Assigned Internet Protocol Numbers can be found at

-- http://www.iana.org/assignments/protocol-numbers/protocol-numbers.xml

flowLabel				[6] INTEGER OPTIONAL,

summaryPeriod			[7] ReportInterval,

packetCount				[8] INTEGER,

sumOfPacketSizes		[9] INTEGER,

packetDataSummaryReason	[10] ReportReason,

...

}

ReportReason ::= ENUMERATED

{

timerExpired			(0),

countThresholdHit		(1),

pDPComtextDeactivated   (2),

pDPContextModification	(3),

otherOrUnknown		(4),

...

}

ReportInterval ::= SEQUENCE

{

firstPacketTimeStamp	[0] TimeStamp,

lastPacketTimeStamp		[1] TimeStamp,

...

}

-- LALS extended location parameters are mapped from the MLP pos element parameters

-- and attributes defined in [88]. For details see specific [88] clauses refered below.

ExtendedLocParameters ::= SEQUENCE

{

posMethod		[0]		PrintableString OPTIONAL, -- clause 5.3.93.1

mapData			[1] 								-- clause 5.2.2.3

CHOICE 		{base64Map 	[0] PrintableString, 	-- clause 5.3.12

url 		[1] PrintableString 	-- clause 5.3.138

} OPTIONAL,

altitude		[2]

SEQUENCE 	{alt PrintableString, -- clause 5.3.5

alt-uncertainty PrintableString OPTIONAL -- clause 5.3.7

} OPTIONAL,

speed			[3] 	PrintableString OPTIONAL, -- clause 5.3.119

direction		[4]		PrintableString OPTIONAL, -- clause 5.3.26

level-conf		[5]		PrintableString OPTIONAL, -- clause 5.3.24

qOS-not-met		[6] 	BOOLEAN OPTIONAL, -- clause 5.3.97

motionStateList [7] 								-- clause 5.2.2.3

SEQUENCE	{primaryMotionState 	[0] PrintableString, -- clause 5.3.69

secondaryMotionState 	[1] SEQUENCE OF PrintableString OPTIONAL,

confidence 				[2] PrintableString -- clause 5.3.24

} OPTIONAL,

floor    		[8]

SEQUENCE 	{floor-number PrintableString, -- clause 5.3.39

floor-number-uncertainty PrintableString OPTIONAL -- clause 5.3.40

} OPTIONAL,

additional-info	[9] PrintableString OPTIONAL, -- clause 5.3.1

-- The following parameter contains a copy of the unparsed XML code of

-- MLP response message, i.e. the entire XML document containing

-- a <slia> (described in [88], clause 5.2.3.2.2) or

-- a <slirep> (described in [88], clause 5.2.3.2.3) MLP message.

-- This parameter is present when the LI-LCS client cannot fully map

-- the MLP response message into an ASN.1 Location object.

lALS-rawMLPPosData	[10] UTF8String OPTIONAL,

...

}

LocationErrorCode ::= INTEGER (1..699)

-- LALS location error codes are the OMA MLP result identifiers defined in [88], Clause 5.4

END -- OF UmtsHI2Operations

END OF SECOND CHANGE