3GPP SA3LI#81-e-a	S3i210244

- eMeeting, 12-16 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0189 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LIPF logic: Part III (data - EPC) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is the third set of the LIPF logic flow-charts that illustrate the LIPF logic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides the LIPF logic for the service type of Data (EPC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementers may not know the inherrent logic to be used and consequence can be either not meeting the requirements or delivery of multiple copies of the same Interception Product to the LEAs. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i210244 |  |  |  |  |  |  |  |  |

##### *** First change  **

Annex X (normative):
LIPF logic

## X.1	Background

## X.2	Governing scenarios

## X.3	Top-level LIPF provisioning logic

## X.4	Data

### X.4.1	Scope of interception

### X.4.2	Top level view

### X.4.3	5GC

### X.4.4	EPC

#### X.4.4.1		The flow-chart

The figure X.4-3 shows the LIPF logic in determining the host NFs in EPC that have the LI functions for the service type of Data.

Figure X.4-3: LIPF logic for the service type Data  in EPC

For the delivery type of IRI + CC, the IRI-POIs and the CC-TFs are provisioned. For the delivery type of IRI, the IRI-POIs and the IRI-TFs are provisioned. For the delivery type of CC, the CC-TFs are provisioned.

For the LI within the EPC, the CSP may deploy either an SGW-based interception or a PGW-based interception. The LIPF logic in supporting the two deployment options is illustrated in figure X.4-4.

Figure X.4-4: Two deployment options in EPC

The LIPF includes a parameter while provisioning the SGW/SGW-C and PGW/PGW-C.

To PGW/PGW-C, with the SGW based deployment option:

- Provide LI functions only for the targets that are outbound roaming with HR.

SGW/SGW-C, with the PGW based deployment option:

- Provide LI functions only for the targets that are inbound roaming with HR.

The above parameter is needed to  avoid both SGW/SGW-C and PGW/PGW-C end up performing the interceptions for the same packet data session.

#### X.4.4.2	Interception

##### X.4.4.2.1	PDHR

There are two deployment options for the packet data header reporting (option 1 and option 2). It is expected that the CSP implements one of the two deployment options.

In option 1, the IRI-POI present in the SGW-U/PGW-U (based on a trigger from IRI-TF present in the SGW-C/PGW-C) or the IRI-POI present in the SGW/PGW generates the xIRI. Accordingly, the IRI-TFs in SGW-C/PGW-C  and IRI-POI in SGW/PGW are provisioned.

In option-2, the CC-POI present in the SGW-U/PGW-U (based on a trigger from CC-TF present in the SGW-C/PGW-C) or the CC-POI present in the SGW/PGW delivers the UP packets as xCC to the MDF3, which in turn, forward the same to the MDF2, and MDF2 would then would generate the IRI messages from the received xCC. Accordingly, the  CC-TF in SGW-C/PGW-C, CC-POI in SGW/PGW and the PDHR handling function in MDF3 are provisioned.

##### X.4.4.2.2	LALS Triggering

There are two deployment options for LALS triggering. It is expected that the CSP implements one of the two deployment options.

In option 1, the LTF present in the host NF that has the associated IRI-POI triggers the LI-LCS client. Accordingly, the LTFs in MME, SGW, PGW, SGW-C and PGW-C  are provisioned.

In option 2, the LTF present in the MDF2 triggers the LI-LCS Client. Accordingly,  the LTF in MDF2 is provisioned.

##### X.4.4.2.3	SGW/PGW deployment options

For the IRI/CC generation, two deployment options are considered:

- SGW-based

- PGW-based.

The CSP expected to implement one of the two options.

When SGW/SGW-C is provisioned, if the deployment option is PGW-based approach, then the SGW/SGW-C does not  perform an interception unless the target is inbound roaming with HR. Similarly, when the PGW/PGW-C is provisioned, if the deployment option is SGW-based approach, then the PGW/PGW-does not perform the interception unless the target is outbound roaming with HR.

##### X.4.4.2.4	HSS

The HSS is expected to provide the IRI-POI functions in the HPLMN only.

##### X.4.4.2.5	Summary

The table X-4-2 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Data in the EPC for an SGW-based deployment option.

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MME |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW |  |  | CC-POI | CC-POI | n/a | CC-POI | n/a |
| PGW |  |  | n/a | n/a | n/a | n/a | IRI-POI |
| PGW |  |  | n/a | n/a | n/a | n/a | CC-POI |
| SGW-C |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW-C |  |  | CC-TF | CC-TF | n/a | CC-TF | n/a |
| SGW-U |  |  | CC-POI | CC-POI | n/a | CC-POI | n/a |
| PGW-C |  |  | n/a | n/a | n/a | n/a | IRI-POI |
| PGW-C |  |  | n/a | n/a | n/a | n/a | CC-TF |
| PGW-U |  |  | n/a | n/a | n/a | n/a | CC-POI |
| MDF3 |  |  | MDF3 | MDF3 | n/a | MDF3 | MDF3 |
| MDF2 |  |  | MDF2 | MDF2 | n/a | MDF2 | MDF2 |
| PDHR | Option 1 | SGW | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
|  |  | SGW-C | IRI-TF | IRI-TF | n/a | IRI-TF | n/a |
|  |  | SGW-U | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
|  |  | PGW | n/a | n/a | n/a | n/a | IRI-POI |
|  |  | PGW-C | n/a | n/a | n/a | n/a | IRI-TF |
|  |  | PGW-U | n/a | n/a | n/a | n/a | IRI-POI |
|  | Option 2 | SGW | CC-POI | CC-POI | n/a | CC-POI | n/a |
|  |  | SGW-C | CC-TF | CC-TF | n/a | CC-TF | n/a |
|  |  | SGW-U | CC-POI | CC-POI | n/a | CC-POI | n/a |
|  |  | PGW | n/a | n/a | n/a | n/a | CC-POI |
|  |  | PGW-C | n/a | n/a | n/a | n/a | CC-TF |
|  |  | PGW-U | n/a | n/a | n/a | n/a | CC-POI |
|  |  | MDF3 | PDHR | PDHR | n/a | PDHR | PDHR |
| LALS triggering | Option 1 | SGW | LTF | LTF | n/a | LTF | n/a |
|  |  | SGW-C | LTF | LTF | n/a | LTF | n/a |
|  |  | PGW | n/a | n/a | n/a | n/a | LTF |
|  |  | PGW-C | n/a | n/a | n/a | n/a | LTF |
|  | Option 2 | MDF2 | LTF | LTF | n/a | LTF | LTF |

The table X.4-3 provides the scope of NF domain that provides the IRI-POI/CC-TF/CC-POI functions for the service type of Data in the EPC for a PGW-based deployment option.

Table X.4-3: Scope of NF domain in EPC providing the LI functions (PGW-based deployment)

| NFs with LI function |  |  | Non-roaming | Roaming with LBO |  | Roaming with HR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPLMN | HPLMN | VPLMN | HPLMN |
| HSS |  |  | IRI-POI | n/a | IRI-POI | n/a | IRI-POI |
| MME |  |  | IRI-POI | IRI-POI | n/a | IRI-POI | n/a |
| SGW |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| SGW |  |  | n/a | n/a | n/a | CC-POI | n/a |
| PGW |  |  | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
| PGW |  |  | CC-POI | CC-POI | n/a | n/a | CC-POI |
| SGW-C |  |  | n/a | n/a | n/a | IRI-POI | n/a |
| SGW-C |  |  | n/a | n/a | n/a | CC-TF | n/a |
| SGW-U |  |  | n/a | n/a | n/a | CC-POI | n/a |
| PGW-C |  |  | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
| PGW-C |  |  | CC-TF | CC-TF | n/a | n/a | CC-TF |
| PGW-U |  |  | CC-POI | CC-POI | n/a | n/a | CC-POI |
| MDF3 |  |  | MDF3 | MDF3 | n/a | MDF3 | MDF3 |
| MDF2 |  |  | MDF2 | MDF2 | n/a | MDF2 | MDF2 |
| PDHR | Option 1 | SGW | n/a | n/a | n/a | IRI-POI | n/a |
|  |  | SGW-C | n/a | n/a | n/a | IRI-TF | n/a |
|  |  | SGW-U | n/a | n/a | n/a | IRI-POI | n/a |
|  |  | PGW | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
|  |  | PGW-C | IRI-TF | IRF-TF | n/a | n/a | IRI-TF |
|  |  | PGW-U | IRI-POI | IRI-POI | n/a | n/a | IRI-POI |
|  | Option 2 | SGW | n/a | n/a | n/a | CC-POI | n/a |
|  |  | SGW-C | n/a | n/a | n/a | CC-TF | n/a |
|  |  | SGW-U | n/a | n/a | n/a | CC-POI | n/a |
|  |  | PGW | CC-POI | CC-POI | n/a | n/a | CC-POI |
|  |  | PGW-C | CC-TF | CC-TF | n/a | n/a | CC-TF |
|  |  | PGW-U | CC-POI | CC-POI | n/a | n/a | CC-POI |
|  |  | MDF3 | PDHR | PDHR | n/a | PDHR | PDHR |
| LALS triggering | Option 1 | SGW | n/a | n/a | n/a | LTF | n/a |
|  |  | SGW-C | n/a | n/a | n/a | LTF | n/a |
|  |  | PGW | LTF | LTF | n/a | n/a | LTF |
|  |  | PGW-C | LTF | LTF | n/a | n/a | LTF |
|  | Option 2 | MDF2 | LTF | LTF | n/a | LTF | LTF |

NOTE 1:	The use of “n/a” in the above table implies that the LI function is not applicable to the NF for the indicated scenario.

NOTE 2:	The LIPF is not aware of the above role played by the host NFs in providing the LI functions.

##### ** End of all changes **