3GPP TSG-T1/SIG Meeting #17 	T1S-010094

- Melbourne, Australia, 14-16 May 2001

|  | 34.123-3 |  |  |  | CR |  | CR-Num |  |  |  rev |  |  |  |  |  |  Current version: |  |  |  |  | 1.0.1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  | X |  | Core Network |  |  |
| Title:  |  | Inclusion of Logical Channel Type in primitive definitions in TS 34.123-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Motorola India Electronics Limited |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 8 April 2001 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | R99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | 1. To include Logical Channel Type in the RLC and MAC control primitives. 2. Addition of Transport Channel Type in CPHY_TrCh_Config_REQ primitive on lines of CMAC_Config_REQ primitive. 3. Defining 3 missing constant definitions. 4. Explicit statement that a Logical Channel Identity is an “instance number”. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | 1. Information regarding the Logical channel type is missing in the control Primitive CMAC_Config_REQ. If hard coded channel mapping is not to be used, logical channel type needs to be known by the MAC Layer in SS, to encode the TCTF (Target Channel Type Field) in the MAC header, in the Downlink direction, (i.e. from SS to UE). This may not required in uplink direction as the SS receives the MAC PDU's containing the TCTF from the UE, and as RB Id to Transport channel mapping is 1:1, routing of incoming PDU's in SS towards RLC and higher layers is not a problem. However, For testing of MAC multiplexing where the UE and UTRAN have dissimilar multiplexing information, a need to include logical channel type can be foreseen. Further for consistency, similar changes need to be reflected in CRLC_Config_REQ. 2. In the CPHY_TRCH_Config_REQ primitive the Transport Channel Type is not available. The Transport Channel Type is available in CMAC_Config_REQ. To avoid the ”hard coding” of the Transport Channel Identities Transport Channel Type is to be added in primitive CPHY_TRCH_Config_REQ. With this change, CPHY_TRCH_CONFIG_REQ can be processed, without waiting for a follow up CMAC-CONFIG-REQ, in the underlying implementation. 3. Three constants "maxulTrCH", "maxdlTrCH" & "maxLogChperTrch" are used in the 34.123-3 but are defined neither in 34.123-3 nor in 25.331. (These definitions are defined in ETSI ITEX files, but not documented in 34.123-3) 4. In sec 8.2 of TS 34.123-3 it has been explicitly specified that Physical Channel Identity, RB Identity and Logical Channel Identity are unique. Nothing explicitly is said about Logical Channel Identity, whether it is a unique number or an "instance number' (i.e. number differentiating it from other Logical Channels mapped onto one transport Channel). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | 1. Incomplete information in the Primitive. 2. Inconsistency across Primitives will exist. 3. Results in ITEX ANALYSE errors. 4. Leads to confusion. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 1. Section 7.3.2.2.13 & 7.3.2.2.8 of TS 34.123-3 version 1.0.1 2. Section 7.3.2.2.1 of TS 34.123-3 version 1.0.1 3. Section 7.3.2.2 of TS 34.123-3 version 1.0.1 4. Section 8.2 of TS 34.123-3 version 1.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

1.

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_LogicalChannel_Mapping |
| Comment |  |
| Type Definition |  |
| SEQUENCE { ul_TransportChannelType SS_UL_TransportChannel_Type, logicalChannelIdentity LogicalChannelIdentity, logicalChannelType LogicalChannelType OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DL_LogicalChannel_Mapping |
| Comment |  |
| Type Definition |  |
| SEQUENCE { dlTransportChannelType SS_DL_TransportChannel_Type, logicalChannelIdentity LogicalChannelIdentity, logicalChannelType LogicalChannelType OPTIONAL, mac_LogicalChannelPriority MAC_LogicalChannelPriority OPTIONAL } |  |

The following enumeration needs to be added to support Logical Channel Type.

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | LogicalChannelType |
| Comment |  |
| Type Definition |  |
| ENUMERATED { BCCH, PCCH, CCCH, CTCH, DCCH, DTCH, SHCCH } |  |

Changes to be reflected in the CRLC_Config_Req for consistency.

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | RB_LogCH_Mapping |
| Comment | Provide mapping information between RB, logical channel and CN domain. |
| Type Definition |  |
| SEQUENCE { uLlogicalChannelIdentity LogicalChannelIdentity OPTIONAL, dLlogicalChannelIdentity LogicalChannelIdentity OPTIONAL, logicalChannelType LogicalChannelType OPTIONAL, cn-DomainIdentity CN-DomainIdentity OPTIONAL } |  |

2.

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyTrchConfigReq |
| Comment | To request to configure the transport channel |
| Type Definition |  |
| SEQUENCE { ulconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, ul_TransportChannelType SS_UL_TransportChannel_Type, TransportChannelInfo CHOICE { ul_CommonTrCHInfo CommonTransChTFS, ul_DedTrCHInfo DedicatedTransChTFS}, } OPTIONAL, ulctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023), gainFactorInformation PowerOffsetInformation } OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, dl_TransportChannelType SS_DL_TransportChannel_Type, TransportChannelInfo CHOICE { dl_CommonTrCHInfo CommonTransChTFS, dl_DedTrCHInfo DedicatedTransChTFS}, } OPTIONAL, dlctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023) } OPTIONAL } |  |

3.    Constant definitions to be declared in section 7.3.2.2

maxulTrCH						INTEGER	::= 16

maxdlTrCH						INTEGER	::= 16

maxLogCHperTrCH					INTEGER	::= 8

4.  The following text to be added to section 8.2

Requirements on identification of Logical Channel in a test case:

It is an instance number of the individual logical channel and uniquely identifies among all the Logical Channel mapped onto one Transport Channel.

Same or different identities may be assigned for UL and DL of the same Logical Channel type.