3GPP TSG T1/SIG#20

26 – 28 Oct 2001

Cancun, Mexico

- Title:	Essential modifications required to CPHY_TrCH_Config_REQ ASP

- Source:	Anritsu

- Date:	10 Nov 2001

- Document for:	Discussion / Agreement

# Summary

This document presents a problem found in the current definition of the CPHY_TrCH_Config_REQ ASP defined in TS 34.123-3, and proposes a solution.

# Introduction

The present definition of CPHY_TrCH_Config_REQ includes the following definition of the Transport Format Set.

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyTrchConfigReq |
| Comment | To request to configure the transport channel |
| Type Definition |  |
| SEQUENCE { modificationInfo ModificationInfo, ulconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, ul_TransportChannelType SS_UL_TransportChannel_Type, transportChannelInfo CHOICE { ul_CommonTrCHInfo CommonTransChTFS, ul_DedTrCHInfo DedicatedTransChTFS }, } OPTIONAL, ulctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023), gainFactorInformation PowerOffsetInformation } OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, dl_TransportChannelType SS_DL_TransportChannel_Type, transportChannelInfo CHOICE { dl_CommonTrCHInfo CommonTransChTFS, dl_DedTrCHInfo DedicatedTransChTFS}, } OPTIONAL, dlctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023) } OPTIONAL } |  |

1. The element DedicatedTransChTFS eventually breaks down to define the TFS as:

DedicatedDynamicTF-Info ::=			SEQUENCE {

rlc-Size							CHOICE {

bitMode				BitModeRLC-SizeInfo,

octetModeType1			OctetModeRLC-SizeInfoType1

},

numberOfTbSizeList				SEQUENCE (SIZE (1..maxTF)) OF										NumberOfTransportBlocks,

logicalChannelList				LogicalChannelList

}

where rlc-Size is the size of an RLC PDU. It is related to TB size as follows (from [2]):

TB size = RLC size + MAC header size

2. The element CommonTransChTFS similarly breaks down to define the TFS as:

CommonDynamicTF-Info ::=			SEQUENCE {

rlc-Size							CHOICE {

fdd									SEQUENCE {

octetModeRLC-SizeInfoType2			OctetModeRLC-SizeInfoType2

},

…

# Description of the problems

1. 	The Transport channel processing object in the System Simulator has no knowledge of the MAC header size. For a dedicated channel this depends upon whether the MAC is required to multiplex logical channels onto the transport channel. This information is communicated to the MAC layer in a subsequent ASP (CMAC_Config_REQ).

2.	The definition for a common channel only allows the TB size to be specified to an octet boundary. For RRC signalling on the UU interface, there is no need to convey a TB size on a bit boundary. The only common channel to which this can apply is the BCCH, where the size is not signalled.  However, for the purposes of internal configuration it is necessary to convey TB sizes not aligned to octet boundaries.

# Proposed Solutions

The configuration information for the TFS should be simplified in a way that conveys TB sizes irrespective of the channel type:

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CphyTrchConfigReq |
| Comment | To request to configure the transport channel |
| Type Definition |  |
| SEQUENCE { modificationInfo ModificationInfo, ulconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, ul_TransportChannelType SS_UL_TransportChannel_Type, transportChannelInfo CommonOrDedicatedTFS, ulctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023), gainFactorInformation PowerOffsetInformation } OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (0..maxTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, dl_TransportChannelType SS_DL_TransportChannel_Type, transportChannelInfo CommonOrDedicatedTFS, dlctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023) } OPTIONAL } |  |

Where CommonOrDedicatedTFS is defined as:

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTFS |
| Comment | Transport Format Set |
| Type Definition |  |
| CommonOrDedicatedTFS ::= SEQUENCE { tti CHOICE { tti10 CommonOrDedicatedTF-InfoList, tti20 CommonOrDedicatedTF-InfoList, tti40 CommonOrDedicatedTF-InfoList, tti80 CommonOrDedicatedTF-InfoList, dynamic CommonOrDedicatedTF-InfoList -DynamicTTI }, semistaticTF-Information SemistaticTF-Information } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CommonOrDedicatedTF-InfoList |
| Comment | Transport Format Set |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxTF)) OF ConnonOrDedicatedTF-Info |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | ConnonOrDedicatedTF-Info |
| Comment | Transport Format Set |
| Type Definition |  |
| SEQUENCE { tb-Size INTEGER {0..4992}, numberOfTbSizeList SEQUENCE (SIZE (1..maxTF)) OF NumberOfTransportBlocks, logicalChannelList LogicalChannelList } |  |