3GPP TSG T WG2, SWG1 (MExE)	Tdoc T2X99106

New bury, England

11-13 August, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 23.057 |  | Version: |  |  | 1.2.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Vodafone, Nokia | Date: | 3/08/99 |
| --- | --- | --- | --- |

| Subject: | Permissions for User Preferences |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | To ensure that changes to important User Preferences such as the homepage, or bookmarks, can only be made with “explicit” user permission, all changes to User Preferences should only be made with single action user permission. |
| --- | --- |

| Clauses affected: | 8.2 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: | Although this CR makes single action user permission mandatory for all user preference changes, it is recognised that in the future some benign user preference fields may be identified. These could be changed with blanket user permission. |
| --- | --- |

## 8.2	MExE executable permissions

The following table 3 specifies the permissions of operator, manufacturer and third party security domains.

The actions listed in the security table 3 are generic actions. Application programming interfaces (APIs) which are intrinsically part of the MExE implementation may directly or indirectly cause these actions to be performed. When these actions occur, whether directly or indirectly called by such an API, the security restrictions shall apply.

Explicit user permission is required for all actions by MExE executables in all domains.  Types of user permission are defined in clause 8.3

File access is not permitted for untrusted MExE executables, except that untrusted Java applications can access files only in the application’s own directory. Untrusted MExE executables are not permitted access to any actions which access the phone functionality (phone functionality includes all the actions in the table 3) except for this one qualified exception, and as further identified in 8.2.1.

Actions available using interfaces giving access to the phone functionality (either in existence at the time of approval of this specification or not) that are not listed or not of the same category as a group in the security table 3 can be accessed by all trusted MExE executables.

Table 3: Security domains and actions

|  |  | MExE Security Domains |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Actions |  |  | Operator | Manufacturer |  | Third Party |  |
| User private data access 1 - Read - Write - Get properties - Delete - Get Location Information - Read stored SMS - Delete stored SMS |  | Yes 2 |  |  |  |  |
| Network access - Initiate a voice/data connection 3 - Accept a voice/data connection 3 - Call forward 4 - Multiparty call 4 - Call deflection 4 - Explicit call transfer 4 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) 4 - Generate DTMF - Query network status - Get signal level - Get call list |  | Yes |  |  |  | Yes 6 |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable |  | Yes |  |  |  |  |
| Miscellaneous functions - Get manufacturer software version |  | Yes |  |  |  |  |
| Certificate management - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain |  | Yes 5 |  |  |  |  |
| Audio access - Sound generation to speaker (e.g. via stream) - Select Melody - Set speaker volume - Get melody list |  | Yes |  |  |  |  |
| Power management - Monitor the power state - Change the power state |  | Yes |  |  |  |  |
| Network selection/operator data access - Get IMSI - Get home network - Select network | Yes |  |  | No |  |  |

Table 3 (concluded): Security domains and actions

|  |  | MExE Security Domains |  |  |
| --- | --- | --- | --- | --- |
| Actions |  |  | Operator | Manufacturer | Third Party |
| Device core function access - Start/stop radio - Turn on/off device | No |  |  |  |
| User profile management - Activate a user profile - Modify a user profile | No |  |  |  |
| User preference management Modify user preferences | Yes7 |  |  |  |
| Legend for above footnotes:- 1 – User private data includes user files, phonebook, etc located on the MS. 2 – The user shall be able to specify data access permissions within the capabilities of the device. 3 – A network connection may be via any supported bearer service 4 – Multiparty, deflection, and explicit call transfer shall be permitted only to numbers explicitly supplied by the user to the MExE Executable. Modification of call forward numbers stored in the network shall only be permitted to numbers explicitly supplied by the user to the operator. 5 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. 6– The Third Party domain's permission to access the networking action depends on the provisioning mechanism as described in section 8.15 7 – Trusted applications only have permission to modify user preferences, and not to activate or de-activate them. The user shall be able to specify for each domain, the preferences that applications in that domain can access. All other preferences shall not be accessible to that domain. The default shall be that there is no access. Single action user permission is the only type of user permission that shall be possible for changes to User Preferences. |  |  |  |  |