3GPP TSG-T WG2-SWG3 #7 	T2M010071

Brunswick, Germany
27-29 June 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 23.140 |  |  |  | CR |  |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | Priority field in notification message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Siemens AG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MMS |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 25 June 2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The sender of a message is enabled to assign a value that expresses the priority of the message to that message itself. This information may be conveyed within the submit request (MM1_submit.REQ) and the retrieve response (MM1_retrieve.RES). In consequence the priority of the message will be unknown to the recipient until the retrieval of the message. In order to enable the recipient to take the priority (importance) of the message into account when making his decision whether to download a message or not, the priority information should be added to the notification message (MM1_notification.REQ) optionally. This CR proposes to add the information element called “priority” to the MM1_notification.REQ message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Introduction of a new information element containg the priority of a message to the notification. The introduction of this new feature requires changes in the technical realisation chapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | The recipient will only be provided with some basic information about the message to be retrieved within the notification. The priority, assigned to the message by the sender, will not be known to the recipient of the message until he has retrieved the message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 8.1.2 Multimedia Message Notification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

# …

### 8.1.2	Multimedia Message Notification

This part of the MMS service covers the notification about MM from the recipient MMS Relay/Server to the corresponding recipient MMS User Agent and involving abstract messages  are outlined in Table 4 from type, and direction points of view.

Table 4: abstract messages for notification of MM in MMS

| Abstract message | Type | Direction |
| --- | --- | --- |
| MM1_notification.REQ | Request | MMS Relay/Server -> MMS UA |
| MM1_notification.RES | Response | MMS UA -> MMS Relay/Server |

#### 8.1.2.1	Normal Operation

Upon receiving the MM1_notification.REQ, the recipient MMS User Agent shall respond with the MM1_notification.RES to the recipient MMS Relay/Server to acknowledge the successful reception of the MM1_notification.REQ.

The MM1_notification.RES shall unambiguously refer to the corresponding MM1_notification.REQ.

#### 8.1.2.2	Abnormal Operation

In this case the MMS UA shall respond with a MM1_notification.RES encapsulating a status which indicates the reason the notification could not be processed. If the MMS UA does not provide the MM1_notification.RES the MMS Relay/Server should be able to retransmit the notification at a later state.

#### 8.1.2.3	Features

Addressing: The MM originator address may be provided to recipient MMS User Agent in the MM1_notification.REQ.

Time constraints: The recipient MMS User Agent shall be provided a time of expiry of the MM. In case of reply-charging the deadline for the latest time of submission of a reply-MM should be conveyed within the MM1_notification.REQ.

Reply-Charging: In case of reply-charging the MMS Relay/Server may indicate in the MM1_notification.REQ that a reply to the notified original MM is free of charge.

Message class, message size, priority and subject: The MM shall be qualified further by adding a message class and an approximate size to the MM in the MM1_notification.REQ. The MM may be qualified further by adding a priority and/or subject to the MM. Additional qualifiers may be added.

Reporting: If the originator MMS User Agent has requested to have a delivery report, the recipient MMS Relay/Server may convey this information to the recipient MMS User Agent in the MM1_notification.REQ. The recipient MMS User Agent may indicate in the MM1_notification.RES that it would not wish a delivery report to be created.

Identification: In case of reply-charging when a reply-MM is notified within the MM1_notification.REQ the MMS Relay/Server should convey the identification of the original MM replied to within the same MM1_notification.REQ.

Message Reference: The recipient MMS Relay/Server shall always provide a reference, e.g., URI, for the MM in the MM1_notification.REQ.

MM Status: The recipient MMS User Agent may indicate in the MM1_notification.RES how it intends the MM to be handled, e.g. the immediate rejection of the MM.

#### 8.1.2.4	Information Elements

Table 5: Information elements in the MM1_notification.REQ.

| Information element | Presence | Description |
| --- | --- | --- |
| Message class | Mandatory | The class of the MM (e.g., personal, advertisement, information service; default = personal) |
| Message size | Mandatory | The approximate size of the MM |
| Time of expiry | Mandatory | The time of expiry for the MM. |
| Message Reference | Mandatory | a reference, e.g., URI, for the MM |
| Priority | Optional | The priority (importance) of the message. |
| Subject | Optional | The title of the whole MM. |
| Sender address | Optional | The address of the MM originator. |
| Delivery report | Optional | Request for delivery report |
| Reply-Charging | Optional | Information that a reply to this particular original MM is free of charge. |
| Reply-Deadline | Optional | In case of reply-charging the latest time of submission of a reply granted to the recipient. |
| Reply-Charging-ID | Optional | The identification of the original MM replied to if this notification indicates a reply-MM. |

Table 6: Information elements in the MM1_notification.RES.

| Information element | Presence | Description |
| --- | --- | --- |
| MM Status | Optional | The status of the MM’s retrieval |
| Report allowed | Optional | Request to allow or disallow the sending of a delivery report to the MM originator |

…