ETSI SMG4	TSGT2#2(99)060

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.07 |  | Version: |  |  | 7.2.0 |  |  |  |
| Submitted to SMG | 29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: Error: Reference source not found |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | Ericsson | Date: | 990312 |
| --- | --- | --- | --- |

| Subject: | Alarm functionality |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Added functionality to set up single alarm for a specific date and for recurrent alarms. Added unsolicited result code for the alarm event. Also, added “snooze” function for the alarms. |
| --- | --- |

| Clauses affected: | 8.xx, Annex B |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

8.16	Alarm +CALA

Table 1: +CALA parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CALA=<time>[,<n>[,<type> [,<text>[,<recurr>[,<silent>]]]]] | +CME ERROR: <err> |
| +CALA? | [+CALA: <time>,<n1>,<type>,[<text>],[<recurr>],<silent> [<CR><LF>+CALA: <time>,<n2>,<type>,[<text>],[<recurr>],<silent> [...]]] +CME ERROR: <err> |
| +CALA=? | +CALA: (list of supported <n>s),(list of supported <type>s),<tlength>,<rlength>,(list of supported <silent>s) +CME ERROR: <err> |

Description

Set command sets an alarm time in the ME. There can be an array of different types of alarms, and each alarm may cause different text to be displayed in the ME display. If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

To set up a recurrent alarm for one or more days in the week, the <recurr>-parameter may be used.

When an alarm is timed out and executed, the unsolicited result code +CALV: <n> is always returned, even if the alarm is set up to be silent.

Read command returns the list of current active alarm settings in the ME.

Test command returns supported array index values, alarm types, and maximum length of the text to be displayed.

Defined values

<time>: refer +CCLK
Note! If the <recurr>-parameter is used, the <time>-parameter must not contain a date.

<n>, <n1>, <n2>: integer type value indicating the index of the alarm; default is manufacturer specific

<type>: integer type value indicating the type of the alarm (e.g. sound, volume, LED); values and default are manufacturer specific

<text>: string type value indicating the text to be displayed when alarm time is reached; maximum length <tlength>

<tlength>: integer type value indicating the maximum length of <text>

<recurr>: string type value indicating day of week for the alarm in one of the following formats:

“<1..7>[,<1..7>[…]]” – Sets a recurrent alarm for one or more days in the week. The digits 1 to 7 corresponds to the days in the week, Monday (1), …, Sunday (7).
Example: The string “1,2,3,4,5” may be used to set an alarm for all weekdays.

“0” – Sets a recurrent alarm for all days in the week.

<rlength>: integer type value indicating the maximum length of <recurr>

<silent>: Integer type value indicating if the alarm is silent or not. If set to 1 the alarm will be silent and the only result from the alarm is the unsolicited result code +CALV. If set to 0 the alarm will not be silent.

Implementation

Optional.

8.x	Delete alarm +CALD

Table 2: +CALD action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CALD=<n> | +CME ERROR: <err> |
| +CALD=? | +CALD: (list of supported <n>s) +CME ERROR: <err> |

Description

Action command deletes an alarm in the ME. If the command fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Test command returns supported array index values.

Defined values

<n>: integer type value indicating the index of the alarm; default is manufacturer specific

Implementation

Optional.

8.x+1	Postpone or dismiss an alarm +CAPD

Table 3: +CAPD action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CAPD=[<sec>] | +CME ERROR: <err> |
| +CAPD=? | +CAPD: (list of supported <sec>s) +CME ERROR: <err> |

Description

Set command postpones or dismisses a currently active alarm. If the command fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Test command returns supported <sec>-parameter values.

Defined values

<sec>: integer type value indicating the number of seconds to postpone the alarm. If <sec> is set to 0 (default), the alarm is dismissed.

Implementation

Optional.

Annex B (normative):
Summary of result codes

V.25ter [14] result codes which can be used in GSM and codes defined in this ETS:

Table B.1: Result codes

| Verbose result code (V.25ter command V1 set) | Numeric (V0 set) | Type | Description |
| --- | --- | --- | --- |
| +CALV | as verbose | unsolicited | refer subclause 8.16 |
| +CCCM: <ccm> | as verbose | unsolicited | Refer subclause 7.15 $(AT R97)$ |
| +CCWA: <number>,<type> ,<class>[,<alpha>] | as verbose | unsolicited | refer subclause 7.11 |
| +CCWV | as verbose | unsolicited | refer subclause 8.28 |
| +CDEV: <elem>,<text> | as verbose | unsolicited | refer subclause 8.10 |
| +CIEV: <ind>,<value> | as verbose | unsolicited | refer subclause 8.10 |
| +CKEV: <key>,<press> | as verbose | unsolicited | refer subclause 8.10 |
| +CLAV: <code> | as verbose | unsolicited | refer subclause 8. |
| +CLIP: <number> ,<type>[,<subaddr> ,<satype>[,<alpha>]] | as verbose | unsolicited | refer subclause 7.6 |
| +CME ERROR: <err> | as verbose | final | refer subclause 9.2 |
| +COLP: <number> ,<type>[,<subaddr> ,<satype>[,<alpha>]] | as verbose | intermediate | refer subclause 7.8 |
| +CR: <type> | as verbose | intermediate | refer subclause 6.8 |
| +CREG: <stat>[,<lac> ,<ci>] | as verbose | unsolicited | refer subclause 7.2 |
| +CRING: <type> | as verbose | unsolicited | refer subclause 6.11 |
| +CSSI: <code1> [,<index>] | as verbose | intermediate | refer subclause 7.16 |
| +CSSU: <code2> [,<index>[,<number>, <type>[,<subaddr>, <satype>]]] | as verbose | unsolicited | refer subclause 7.16 |
| +CUSD: <m>[,<str>,<dcs>] | as verbose | unsolicited | refer subclause 7.14 |
| +DR: <type> | as verbose | intermediate | refer subclause 6.13 |
| +ILRR: <rate> | as verbose | intermediate | refer subclause 4.3 |
| BUSY | 6 | final | busy signal detected |
| CONNECT | 1 | intermediate | connection has been established |
| CONNECT <text> | manufacturer specific | intermediate | as CONNECT but manufacturer specific <text> gives additional information (e.g. connection data rate) |
| ERROR | 4 | final | command not accepted |
| NO ANSWER | 7 | final | connection completion timeout |
| NO CARRIER | 3 | final | connection terminated |
| NO DIALTONE | 5 | final | no dialtone detected |
| OK | 0 | final | acknowledges execution of a command line |
| RING | 2 | unsolicited | incoming call signal from network |