ETSI SMG4	TSGT2#2(99)061

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.07 |  | Version: |  |  | 7.2.0 |  |  |  |
| Submitted to SMG | 29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | SMG4 | Date: | 990312 |
| --- | --- | --- | --- |

| Subject: | Phonebook storage |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | x |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | To make it easier to choose a PIN2-locked phone book memory storage by sending the PIN2 as an optional parameter. |
| --- | --- |

| Clauses affected: | 8.11 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

8.11	Select phonebook memory storage +CPBS

Table 1: +CPBS parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CPBS=<storage> [,<password>] | +CME ERROR: <err> |
| +CPBS? | +CPBS: <storage>[,<used>,<total>] +CME ERROR: <err> |
| +CPBS=? | +CPBS: (list of supported <storage>s) |

Description

Set command selects phonebook memory storage <storage>, which is used by other phonebook commands. If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Read command returns currently selected memory, and when supported by manufacturer, number of used locations and total number of locations in the memory.

Test command returns supported storages as compound value.

Defined values

<storage> values reserved by this ETS:

"DC"	ME dialled calls list (+CPBW may not be applicable for this storage) $(AT R97)$

"EN"	SIM (or ME) emergency number (+CPBW is not be applicable for this storage) $(AT R97)$

"FD"	SIM fixdialling-phonebook

"LD"	SIM last-dialling-phonebook

"MC"	ME missed (unanswered received) calls list (+CPBW may not be applicable for this storage)

"ME"	ME phonebook

"MT"	combined ME and SIM phonebook

"ON"	SIM (or ME) own numbers (MSISDNs) list (reading of this storage may be available through +CNUM also) $(AT R97)$

"RC"	ME received calls list (+CPBW may not be applicable for this storage) $(AT R97)$

"SM"	SIM phonebook

"TA"	TA phonebook

<password>: string type value representing the PIN2-code required when selecting PIN2-code locked <storage>s above, e.g. “FD”.

<used>: integer type value indicating the number of used locations in selected memory

<total>: integer type value indicating the total number of locations in selected memory

Implementation

Mandatory when phonebook read, find or write command, or direct dialling (refer subclause "Direct dialling from phonebooks") is implemented.