ETSI SMG4	Tdoc SMG4 99e005

Oslo, Norway

December, 17-18, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 08.60 |  | Version |  | 5.1.1 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Circuit switched EDGE |
| --- | --- |

| Source: | Nokia | Date: | 28.02.99 |
| --- | --- | --- | --- |

| Subject: | Improvement of user-initiated service level up and downgrading |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Introduction of changes necessary for support of EDGE channels. |
| --- | --- |

| Clauses affected: | Several. See the attached CR. |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications | X | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

## 3.5	Coding

In the following sections, the coding of the frames is described. Any spare or not used control bits should be coded binary "1".

For all frame types the octet 0, 1 and the first bit of octets 2, 4, 6, 8, ... 38 are used as frame sync.

### 3.5.1	Coding of Speech Frames

Control bits (C-bits):

| Description | Uplink | Downlink |
| --- | --- | --- |
|  | C1C2C3C4 C5 | C1C2C3C4 C5 |
| Frame type FR | Speech: 0 0 0 1 0. | Speech: 1 1 1 0 0 |
| (Bits C1 - C5). EFR | Speech: 1 1 0 1 0 | Speech: 1 1 0 1 0 |
| Time | Binary number indicating the | Binary number indicating |
| Alignment | required timing adjustment to | the timing adjustment made. |
| (Bits C6 - C11) | be made in steps of 250/500 s. |  |
|  | The following values apply for the coding |  |
|  | C6C7 . . . C11 |  |
|  | 0 0 0 0 0 0 No change in frame timing |  |
|  | 0 0 0 0 0 1 Delay frame 1 x 500 s |  |
|  | 0 0 0 0 1 0 Delay frame 2 x 500 s |  |
|  | . . . .... |  |
|  | . . . .... |  |
|  | 1 0 0 1 1 1 Delay frame 39 x 500 s |  |
|  | 1 0 1 0 0 0 Not used |  |
|  | . . . .... |  |
|  | 1 1 1 1 0 1 Not used |  |
|  | 1 1 1 1 1 0 Delay frame 1 x 250 s |  |
|  | 1 1 1 1 1 1 Advance frame 250 s |  |
| Frame indicators. The definition | C12: BFI | C12 - C15: Spare |
| and coding of these indicators | 0 : BFI = 0 |  |
| are given in GSM 06.31. | 1 : BFI = 1 | IF FR. Speech |
|  | C13 C14: SID | ELSE |
| Bits C12 - C16 | 0 0 :SID = 0 | C12: UFE |
|  | 0 1 :SID = 1 | 0 :UFE=0 bad uplink frame |
|  | 1 0 :SID = 2 | 1 : UFE=1 good up-link frame |
| Downlink |  |  |
| Uplink Frame Error | C15: TAF |  |
| (UFE) C12 | 0 : TAF = 0 |  |
| (see subclause 4.8.3) | 1 : TAF = 1 | C13 - C15: spare |
|  | C16: Spare | C16: SP |
|  |  | 0 : SP = 0 |
|  |  | 1 : SP = 1 |
| DTX indicator | C17: DTXd | C17: Spare |
|  | 0 : DTX not applied |  |
|  | 1 : DTX applied |  |
| Bits C18 - C21 | Spare | Spare |

Data Bits (D-bits):

Bits D1 .. D260:		Speech block transferred in the same order as output from the transcoder (see GSM 06.10).

For Enhanced Full Rate Speech:

The speech block is subdivided in five subsets. The order within a given subset is the same as output from the transcoder (see ETS 300 726, GSM 06.60). Three parity bits are added at the end of each sub-set.

These parity bits are added to the bits of the subset, according to a degenerate (shortened) cyclic code using the generator polynomial:

g(D) = D3  + D + 1

The encoding of the cyclic code is performed in a systematic form which means that, in GF(2), the polynomial:

d(m)Dn + d(m+1)Dn-1 + ......+ d(m + n-3)D3 + p(0)D2 + p(1)D + p(2)

where p(0), p(1), p(2) are the parity bits, when divided by g(D), yields a remainder equal to:

1 + D + D2

For every CRC, the transmission order is p(0) first followed by p(1) and p(2) successively.

Bit D1			:		spare (binary "1").

Bits D2...D39		:		Indexes of the LSF submatrices.

Bits D40...D42		:		CRC over bits D1 to D22, D25 to D27 and D29.

Bits D43...D95		:		Indexes of the parameters of first sub-frame.

Bits D96...D98		:		CRC over bits D43 to D52, D91 and D92.

Bits D99...D148		:		Indexes of the parameters of second sub-frame.

Bits D149...D151	:		CRC over bits D99 to D103, D105, D144 and D145.

Bits D152...D204	:		Indexes of the parameters of third sub-frame.

Bits D205...D207	:		CRC over bits D152 to D161, D200 and D201.

Bits D208...D257	:		Indexes of the parameters of fourth sub-frame.

Bits D258...D260	:		CRC over bits D208 to D212, D214, D253 and D254.

Time Alignment Bits:

Bits T1 .. T4:			Bits positioned at the end of the downlink frames. If the timing of the frame is to be advanced 250 µS, these 4 bits are not transferred in order to reduce the frame length accordingly. When transferred the bits are set to binary "1".

### 3.5.2	Coding of O&M Frames

Control bits (C-bits):

| Description | Uplink | Downlink |
| --- | --- | --- |
|  | C1C2C3C4 C5 | C1C2C3C4 C5 |
| Frame type | 0 0 1 0 1 : O&M | 1 1 0 1 1 : O&M |
| Bits C1 - C5 |  |  |
| Bits C6 - C15 | Spare | Spare |

Data Bits (D-bits):

Bits D1 .. D264:	Bits used for transfer of O&M information. The coding and use of these bits are left to the manufacturer of the BSC/TRAU.

Spare Bits:

Bits S1 .. S6:		Spare

### 3.5.3	Coding of Data Frames

Control bits (C-bits):

| Description | Uplink | Downlink |
| --- | --- | --- |
| Frame type. | C1C2C3C4 C5 | C1C2C3C4 C5 |
| Bits C1 - C5 | 0 1 0 0 0 : Data | 1 0 1 1 0 : Data |
|  | except 14.5 | except 14.5 |
|  | 1 0 1 0 0 : Data14.51) | 1 0 1 0 0 : Data 14.51) |
| Intermediate RA bit | 0: 8 kbit/s | 0: 8 kbit/s |
| rate. | 1: 16 kbit/s | 1: 16 kbit/s |
| Bit C6 |  |  |
| for data services |  |  |
| except 14.5 |  |  |
| Spare | Spare | Spare |
| for Data 14.5 |  |  |
| Bits C7 - C15 | Spare | Spare |

NOTE 1:	The Data frame is in case of data 14.5 kbit/s used only for synchronization purposes. The data bits are in this case set according to subclause 4.5.1.

### 3.5.4	Coding of Extended Data Frames

Control bits (C-bits):

| Description | Uplink | Downlink |
| --- | --- | --- |
| Frame type. | C1C2C3C4 C5 | C1C2C3C4 C5 |
| Bits C1 - C5 | 1 1 1 1 1 : | 1 1 1 1 1 : |
|  | Extended Data | Extended data |
|  | frame 14.5 kbit/s | Frame 14.5 |
|  | 0 0 0 0 0 : | 0 0 0 0 0 : |
|  | Dummy frame 14.5 kbit/s | Dummy frame 14.5 kbit/s |
| Bit C6 |  |  |
| Idle/Data/UFE | Idle/data | UFE |
|  | Frame indication |  |
| Bits C7 - C13 | Spare | Spare |
| Multi Frame Structure | M1, M2 | M1, M2 |
| defined in GSM 04.21 |  |  |
| Bits M1, M2 |  |  |

### 3.5.5	Coding of Idle Speech Frames

Control bits (C-bits):

| Description | Uplink | Downlink |
| --- | --- | --- |
|  | C1C2C3C4 C5 | C1C2C3C4 C5 |
| Frame type. | 1 0 0 0 0 : Idle Speech | 0 1 1 1 0 : Idle Speech |
| Bits C1 - C5 |  |  |
| Bits C6 - C21 | Coding as for | Coding as for |
|  | Speech frames. | Speech frames. |

Time Alignment Bits:

Bits T1 .. T4:		Coding as for Speech frames.

## 3.6.	Order of Bit Transmission

The order of bit transmission is:

The first octet is transferred first with the bit no. 1 first, bit no. 2 next etc.