ETSI SMG4	TSGT2#2(99)227

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.5.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE 03.57 |
| --- | --- |

| Source: | Vodafone/SMG10 | Date: |  |
| --- | --- | --- | --- |

| Subject: | Security section, and other security related items in specification |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | X |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | User profile section has additions and changes to security related details. List of items to be recorded by journal to be inserted Security section must be updated to reflect new agreements in certification architecture. Security requirements reflected in Annex A Text on security of OS list of user files that cannot be accessed by applications inserted in Annex A |
| --- | --- |

| Clauses affected: | 4.5, 4.10, 8, Annex A |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 4.5	User profile

The user profile (which may consist of one or more user profiles for a user) contains the characterisation of the MExE MS as defined by the user and service provider. The user profile is not unique to the MExE MS, and this clause identifies the usage and content of the user profile from a MExE perspective only, and does not identify the generic support of user profiles in general.  Refer to UMTS 22.01 [14] for further details on the user profile.

### Location of, access to, and security of, the user profile

As multiple user profiles may be defined, the user is able to set up or receive calls/connections associated with different user profiles simultaneously by securely activating a user profile (with each user profile being associated with at least one unique address).  Refer to the Security clause for further details on user profile activation.

The user’s characterisation of the MExE MS in the user profile may be modified at any time by the user and the service provider, and changes affected at the earliest possible opportunity. Refer to the Security clause for further details of user and service provider modification of a user profile.

The security clause shall apply to all user profiles, at all times, whether activated or not.

The user profile is securely managed by the MExE MS as defined in the Security clause, and stored in a secure area (i.e the SIM or the ME) of the MExE MS (either SIM or ME).  The service provider may also retain parts of the user profile in the network.

The support of more than one user profile is not mandatory.

### Characteristics of the user profile

The user profile acts as a repository (which is always available in the MExE MS) defining the MExE MS behaviour, identifying one or more of the following:-

|  | ME specific | Location(s) | Description |
| --- | --- | --- | --- |
| user interface |  |  | Refer to sub-clause 4.6 User interface customisation for details of the specific user interface customisation characterisation which may be stored in the user profile |
| background - colour - pattern/image |  |  |  |
| - font - type - colour - size |  |  |  |
| - interfaces - softkey definition - additional menu items - enabled/disabled extended menus - icon settings |  |  |  |
| - country - language - date - time - currency |  |  |  |
| - Modify supplementary services settings (i.e. diversion, times) | - ? |  | - ? |
| ###### Modify phone settings - ringer tone/vibrate - enabling SIM Lock - Keypad Lock |  |  |  |
| - Preferences |  |  | - 03.57, section 4.4.1 - ”Support of the user profile preferences is subject to the capabilities of the device. Generally the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to MSE.” |
| - terminal properties - language in which the user wishes to work - preferred input mode - amount of memory authorised by the user for MExE |  |  |  |
| - data transfer - prioritised list of preferred bearers, e.g. SMS, USSD, GPRS - maximum size of transfer - maximum duration of transfer - transfer of location information - transfer of the user profile or parts of it |  |  | - may, or may not be, sent to the MSE - could be shifted in Security section - could be shifted in Security section |
| - codecs - prioritised list of preferred audio codecs - prioritised list of preferred image codecs - prioritised list of preferred video codecs - prioritised list of preferred multimedia codecs |  |  |  |
| Service Management |  |  | Refer to sub-clauses 4.5, 4.7.1, 4.7.2 controls the ability to determine which services are transferred to, resident, configured or executing on the MS, which may be refined by use of one or more of the following characteristics: identified security level specific permitted services types of permitted services date or times for actions priorities of services and identifies service preferences which may be set globally for all services per security level on a per service basis and may be identified at installation of the service, or be subsequently modified. |
| transfer of services |  |  |  |
| automatic configuration |  |  |  |
| execution of services |  |  |  |
| residence on the MExE MS |  |  |  |
| personalisation of services |  |  | 02.57, section 6.1 for further details see section VHE of this table |
| Security CR author’s note: There will be some overlap with information given under “Service management” on the previous page. However, the new details given here are important security details. Duplicated text should be deleted from the Service management entry if duplication is to be avoided and not from the Security entry. |  |  | For each application installed on the MExE MS: - Application name (for use in identifying applications to the user) - Application identifier (for use in identifying applications to the network operator and for use in determining the permissions applications have been granted within the terminal. A secure identification method is required.) - Security level - Source of application (for levels 1-3 only, given in certificate used to verify the application) - Identifier of certificate used to verify the application (for level 1-3 only) - Specific actions for which the user has given permission, and type of permission (one-shot, session, blanket, see clause 9.1 for details) Whether or not applications can configure automatically (see clause 4.7.1) control of the following permissions maybe refined by use of one or more of the following characteristics: - universal authorisation for all calls/connections/ messages (i.e. yes/no) - authorisation by calls/connections/messages type (i.e. SMS, USSD etc.) - authorisation by identified services - authorisation by service type (i.e. security level) - authorisation by date/time/day of week |
| call authorisation |  |  | 03.57, section 4.5 |
| Implicit permission for the initiation of events (includes responses made by the MS to external events) of transferred applications, applets |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Implicit permission for the transfer of applications, applets and content |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Network event journal - status of the optional network event journal which is used to maintain a record of application initiated network events (i.e. activated/deactivated) |  |  |  |
| Miscellaneous |  |  |  |
| phonebook entry |  |  | 03.57, section 4.6 |
| ###### Redirect - voicemail box number - fax-mail box number |  |  |  |
| - reset call meters |  |  | - 03.57, section 4.6 |
| Classmark 1 specific Entries |  |  |  |
| Classmark 2 specific Entries |  |  |  |
| ###### WAP browser - pre-installed browser overwrite on upgrade - location of user preferred WAP browser |  |  | - applicable to MExE classmark 2 |
| VHE |  |  | TS 22.xx :Provision of Services in UMTS – The Virtual Home Environment (Tdoc SMG1 825/98) |
| - for each VHE profile: - Incoming session routing address |  |  |  |
| - for each service within one VHE profile: - service state - service appearance - service appearance customisation parameters - service functionality customisation parameters - service data |  |  | - e.g. activate/deactivate/not available - e.g. level of functionality complexity - e.g. forwarded-to numbers |

The above list is not exhaustive.  Support of the user profile preferences is subject to the capabilities of the device.

## 4.10	Journalling of network events

To support the user in monitoring and maintaining a record of (potentially chargeable) network events initiated by services in the MExE environment, it shall be possible for the user to request the MExE MS to maintain a record of network events initiated by services on the MExE MS.  Support of such journalling is mandatory.

Network events for the purposes of journalling, are defined as events which result in the origination of voice or data connections by a service in the MExE environment of the MExE MS. Examples of such events are:-

- Sending an SMS message

- Sending an USSD message

- Initiating a circuit switched connection

- Initiating a packet switched connection

- Sending data over a packet switched connection

Editor’s note: the above list requires to be refined to more accurately define the events which shall be recorded in the journal.

The above list is not exhaustive, but includes any (potentially chargeable) network event initiated by services in the MExE environment.

The user shall be able to activate and deactivate the journalling of network events via the user profile.  The length, format and longevity of the journal is undefined and subject to manufacturers’ discretion.

The journal recording the network events shall be securely managed by the MExE MS, and be stored in a secure area of the MExE MS.

### 4.10.1		Journalling Requirements

The terminal shall support the logging of network events. The user shall determine whether logging is in operation or not and the events that are logged.  The size and format of the log is not the subject of specification.

Security

Editor‘s Note: input awaited from SMG10

## 8.1		Generic security

In order to manage the MExE and prevent attack from unfriendly sources or transferred applications unintentionally damaging the MExE device a security system is required. This section defines the MExE security architecture.

The basis of MExE security is

- a framework of permissions which defines the permissions and privileges transferred content, applets or applications have within the MExE.

- the secure storage of these permissions (and permission type as defined in clause 8.3) in the user profile

- conditions within the execution environment that ensure that applications, applets and content can only perform actions for which they have permission.

[CR author’s note:  I have made these additions as a permission framework is no good unless there are mechanisms to ensure it is obeyed.  The requirements for these mechanisms needs to be more explicit.  Further above, I have deleted the term “privileges” as it is redundant.]

The MExE permissions framework is defined in TS GSM 02.57 and is as follows (there is no implied hierarchy):

MExE Security Level 1 (applications, applets and content authorised  by the HPLMN operator);

MExE Security Level 2 (applications, applets and content authorised by the terminal manufacturer);

MExE Security Level 3 (trusted applications, applets and content authorised by third parties);

MExE Security Level 4 (untrusted applications, applets and content);

## 8.2		Application Permissions

The following table specifies the privileges and permissions of each security level.

User Permission is required for all actions by level 1-3 applications, applets and content.

Types of user permission are defined in clause 8.3

|  | MExE Security Levels |  |  |  |
| --- | --- | --- | --- | --- |
| Actions |  | 1 Network Operator | 2 MS Manufacturer | 3 User Trusted Applications | 4 User Untrusted Applications |
| - Read file - Write file - Get file information - Delete file | Yes 1, 2 |  |  | Yes 3, 4 |
| Load native code libraries | No | Yes 1 | No |  |
| - Initiate a voice/data connection 5 - Accept a voice/data connection 5 - Intercept an incoming voice/data connection 5 | Yes 1 |  |  | No |
| Call forward | Yes 1 | No | Yes 1, 6 | No |
| - Terminate an existing connection - Hold an existing connection - Resume an existing connection | Yes 1 |  |  | No |
| Modification of user invoked connection | Yes 1 |  |  | No |
| - Add a phonebook entry - Delete a phonebook entry - Get a phonebook entry - Modify a phonebook entry | Yes 1 |  |  | No |
| Send an SMS | Yes 1 |  |  | No |
| Add an application or applet | Yes 1 |  |  | No |
| Delete a saved application or applet | Yes 1 |  | Yes 1, 7 | No |
| - Add a certificate for a given level - Delete a certificate for a given level - Replace a certificate for a given level | Yes 1, 8 |  |  | No |
| Audio access (e.g. wake-up service) | Yes 1 |  |  | No |
| HTTP Cache manipulation | Yes 1 |  |  | No |
| Get IMEI | Yes 1 |  |  | No |
| Location Information | Yes 1 | No | Yes 1 | No |
| - Activate a user profile - Modify a user profile | Yes 1 |  |  | No |
| Legend for above footnotes:- 1 - With user permission [CR author’s note. As all actions except one in the table required user permission, we might as well make it a default condition and state it outside the table.] 2 - Except in user pre-specified directories. This list can only be modified by the user. 3 - Only in application/applet’s own directory 4 - Restriction may be required on space allocated to write 5 - A network connection may be via any supported bearer service 6 - To pre-defined numbers in the user profile 7 - Only in pre-specified directory 8 - Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate |  |  |  |  |

Table 3: Security levels and actions

## 8.3		User Permission Types

The term “with user permission” is defined to mean that the user can give permission for a specific action in one or more of the following ways defined in Table 4.    Blanket permission and oneshot permission are mandatory, and session permission is optional.

|  | User Permissions |  |  |
| --- | --- | --- | --- |
| Permission Type | Description | Invocation | Revocation |
| blanket permission | The user gives blanket permission to the application for the specified action, and the application subsequently uses the user’s original permission for the identified subsequent actions whenever the application is running. | Typically such permission would be given at application configuration or run time. | The blanket permission may be revoked by the user at any time. The user permission no longer applies once the application has been removed. |
| session permission | The user gives permission to the application for the specified action during a specific run time session of an application, and the application subsequently uses the user’s permission for the identified subsequent actions whilst the application session is still running. | Typically such permission would be given at application run time. | The session permission may be revoked by the user at any time. The user permission no longer applies once the application run time session has terminated. |
| one-shot permission | The user gives a single permission to the application for the specified action; if the application subsequently wishes to repeat the action it must again request the user’s permission for the identified subsequent action. | Typically such permission would be given at application run time. | The user permission no longer applies once the action has terminated. |

Table 4: User Permissions

Editor’s note: the consequences of upgrading of the WAP browser requires to be further elaborated.

## 8.4		Certification and Authorisation Architecture

In order to enforce the MExE security framework a MExE capable MS is required to operate an authentication mechanism for verifying downloaded applications and applets.  A successful authentication will result in the acceptance of the claimed security level.

As the MExE MS may want to authenticate content from many sources, a public key based solution is appropriate.  Before accepting level 1-3 applications, applets or content, the MExE MS will therefore check that the application, applet or content was signed with a private key, for which the MExE MS has the corresponding public key.  The corresponding public key held in the MS must either be a root public key (securely installed in the MS, say at manufacture) or a key provided in a certificate.  The MExE MS must be able to verify this certificate, i.e. have the public key (as a root key or in a certificate) corresponding to the private key used to sign the certificate.  An  certificate chain is therefore envisaged.

The requirements on authorisation and certification are given in clause 8.4.1. An example authorisation and certification process is described in clause 8.4.2.

### 8.4.1		Certification Requirements

A MExE MS cannot verify certified applications, applets or content of a particular level (from levels 1-3) unless it has a root key for that particular level.

Root keys shall be securely installed in the MExE MS, say, at manufacture.

It is recommended that a “disaster recovery” root key be securely installed on the terminal, to be used to install new root keys when all other root keys on the terminal are invalid.

Level 3 root keys will typically be installed along with and integrated into the MExE ME browser, as is done for PC-based browsers.

An application, applet, content can only be verified if the MExE MS contains a valid certified or root public key corresponding to the private key used to sign the applications, applets or content.

A certificates (other than root certificates) shall only be considered valid if it resides below the trust root certificate in its certification tree, or if the certificate's root CA has been cross-certified by the owner of the trust root certificate.

8.4.2		Example Certification Process

The following processes might be followed in order to securely download a level 3 application to a MExE MS.

- Root public keys for a number of Certification Authorities (CA’s) are installed in the ME, along with the ME browser, at manufacture.  These root public keys can be used to verify certificates for level 3 applications, applets or content.

- A third party software developer generates a private and public key pair (or obtains such a pair from a CA).

- The third party software developer obtains a certificate for the public key from a CA.  The certificate contains the developer public key, signed with the private key of the CA.

- The MExE MS download an application, applet or content of the third party software developer.

- If the MExE does not already have a certificate for the public key of third party software developer, it obtains such a certificate from a public key depository.

- The MExE MS verifies the certificate using the root public key, contained in the browser, of the relevant CA, and extracts the third party software developer public key.

- The MExE MS verifies that the application, applet or content was signed using the private key corresponding to the third party software developer public key and installs or rejects the application, applet or content accordingly.

These processes are shown schematically in the diagram below.

Public key repository

(5) Certificate

CA

MS

GSM network

(1) Public key

(3) Certificate

3rd party software developer

(2) Certificate for public key

(4) signed application

(6) MS verifies signed application using certificate

For example, content, applets or applications for Classmark 2 devices are transferred to a MExE device in JAR files. These files are signed with a service provider’s private key and the MExE device determines the MExE security level on the basis of this signature. This signature can be verified using the public key contained in the certificate of the signer. The process is as follows:

The Service provider generates a pair of keys, private and public.

The private key is used to sign the JAR file.

The signed JAR file is transferred to the MExE device.

The MExE device obtains the public key for the service provider from the certification authority and uses this to validate the JAR file.

The MExE device determines the MExE security level of the JAR file on the basis of the certification.

The distribution of MExE authentication keys is described in the figure below.

Editor’s note: [Discovery of the address of the certification authority by the MS is still work-in-progress in the IETF Secure DNS working group, but the current proposal (available as http://www.ietf.org/internet-drafts/draft-ietf-dnssec-certs-02.txt) shows how a domain name (associated with a certificate) can be generated from the certificate itself. When a domain name is known, a proposed CERT RR (resource record) is retrieved via DNS.]

Figure 5: Distribution of Keys

Each MExE MS must be configured with at least two trusted keys – keys for Level 1 and Level 2 trusted applets. Keys for level 3 applets may be used as required by the user.

[CR author’s note: This requirement is not necessary]

## 8.52	WAP security

The WTLS security model provides a flexible framework based on TLS, which is independent of the GSM security model.

Editor’s note: Need to specify the interface with SIM based security under definition in WAP.

## 8.63	Java security

Editor‘s note: input to be provided by Avinash Palaniswamy.

# Annex A (informative): Java API Requirements

Editor’s note: This section captures the working document in which the SMG4 MExE Java API requirements have been captured.  It is intended that this annex will be deleted prior to approval of the specification.

This document provides a first attempt at a table listing the requirements for a MExE ME as mentioned in 02.57, MExE Service Description, Stage 1.

The aim of this table is to provide a requirements framework in order to allow subsequent specification work such as API definition to be performed more efficiently.

The following provides information relating to the table headings:

| Key to Requirements Table Headings |  |
| --- | --- |
| Heading | Explanation |
| i.d. | A unique identifier for each requirement. An identifier can be assigned only once. This heading also contains the [i.d.] of the next requirement to be entered - embedded as a table function (F9 to update). |
| Category | A series of broad categories into which requirements are placed. "J" means this requirement relates directly to the specification of a Java API. "I" means this requirement relates to implementation, and puts the specification of a Java API into context. |
| Requirement | The actual requirement, which should be as unambiguous (and indivisible) as possible |
| Reference | A reference into a document within which the requirement is mentioned, if applicable |
| Priority | Requirements may be mandatory or optional |
| Classmark | An indication as to which classmark the requirement is applicable or in-applicable (in which case the classmark figure is shown in brackets) |
| Release | An indication of whether the requirement is applicable to Release 98, 99 etc. |
| Comment | A general purpose comment field |
| Applicable API | Applies to Java enabled devices. A list of the APIs which satisfy the requirement |
| Chosen API | Applies to Java enabled devices. The API chosen from the "Applicable API" list. |

| i.d. | Category | Requirement | Reference | Priority | Class-mark | Release | Comment | Applicable APIs | Chosen API |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High Level |  |  |  |  |  |  |  |  |
| 1 | J | The MExE ME shall provide a means for applications, applets and content to access the user interface |  |  |  |  |  |  |  |
| 3 | J | The MExE ME shall provide the means to allow the transfer of applications, applets or content either automatically or on demand according to MExE security requirements | 02.57 v7.0.0 6.1.0 |  |  |  |  |  |  |
| 4 | I | The MExE ME shall provide the means to allow the upgrade of existing applications across the GSM/UMTS network subject to MExE security requirements and user authorisation | 02.57 v7.0.0 6.1.0 |  |  |  |  |  |  |
| 5 | I | The MExE ME shall allow the user to personalise services | 02.57 v7.0.0 6.1.0 |  |  |  | FFS – relates to user profile (i.e. the way the information is displayed on the MExE ME , not the type/nature of information). DELETE FROM CLARIFICATION LIST BUT DO NOT CR |  |  |
| 6 | J | The MExE terminal shall allow the user to utilise both internet and intranet applications, applets and content according to MExE security requirements | 02.57 v7.0.0 6.1.0 |  |  |  |  |  |  |
|  |  |  |  |  |  |  | This is ambiguous and does not add value: deleted |  |  |
| 54 | J | The MExE ME shall possess an architecture that is capable of supporting applications, applets and content in a standard execution environment independently of the MExE ME manufacturer | 02.57 v7.0.0 9.1.0 |  |  |  | CR |  |  |
|  | Classmarks |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | CR to 02.57 5.3 |  |  |
| 11 | I | MExE applications and applets conforming to a particular MExE classmark shall be supportable by all MExE MS’s of that classmark regardless of MExE MS manufacturer | 02.57 v7.0.0 5.3 | Mandatory |  |  |  |  |  |
|  | User Interface Configuration Management |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |
| 2 | I | The MExE ME shall allow the user to personalise the user interface within bounds defined by the manufacturer | 02.57 v7.0.0 6.1.0 |  |  |  | The extent of "personalisation" will be limited by the terminal manufacturer. E.g. screen colours, ring patterns, screen saver This needs further clarification – Nash |  |  |
| 15 | I | The user shall be able to manage the user interface configuration of the MExE MS within bounds defined by the manufacturer | 02.57 v7.0.0 6.2.1 |  |  |  | The extent of "personalisation" will be limited by the terminal manufacturer. |  |  |
| 16 | I | The MExE ME shall allow the user to identify the current user interface configuration within bounds defined by the manufacturer | 02.57 v7.0.0 6.2.1 |  |  |  |  |  |  |
| 17 | I | The MExE ME shall allow the user to add, modify or delete a particular user configuration within bounds defined by the manufacturer | 02.57 v7.0.0 6.2.1 |  |  |  |  |  |  |
| 18 | I | It shall be possible to obtain the user interface configuration information, subject to MExE security requirements is resident on the MExE ME, the SIM or the network | 02.57 v7.0.0 6.2.1 |  |  |  | FFS – similar to “.dt”, “.xdefaults” in Unix. [Further contribution expected from Nash]. Change Request |  |  |
|  |  |  |  |  |  |  | Redundant, now covered by Req18 |  |  |
| 20 |  | THe user interface configuration management information may be stored on the MExE ME, the SIM or the network, subject to MExE security requirements. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Now folded into req20 |  |  |
| 22 | J | Any MExE ME shall be able to access and interpret user interface configuration data, according to MExE security requirements |  |  |  |  | Assume “user profile” means personal details i.e. name, e-mail etc. Clarification required on defn of terms “The user profile data shall be associated with a given user” is part of defn (req 23) “User profile data” is a superset of “user interface configuration” – partitioning/access is FFS. NB: “interpret” may need to be clarified! CR – add to end of para3.6.2.1, 02.57 |  |  |
|  |  |  |  |  |  |  | Delete – folded into Req22 GLOBAL: UI CONFIG à USER PROFILE NB: UICONFIG IS SUBSET U PROFILE |  |  |
|  | Service Management |  |  |  |  |  |  |  |  |
| 25 | I | The MExE ME shall allow the user to request the range of MExE services available from the PLMN MExE service provider | 02.57 v7.0.0 6.2.2 |  |  |  | Misgivings about how registration is carried out, notification messages etc. What is the notification mechanism? This is perceived as a common address (e.g. URL. Same address could be used when roaming – use (e.g.) DNS to resolve actual server address). This address is used to provide the first point of access to a MExE network. The user is provided with a list of MexE services available from the PLMN service provider. MExE is allowed to stipulate only access to GSM/GPRS/UMTS MExE SPs – PLMN independent SPs lie outside of the scope of MExE standardisation. When roaming, the user could collect a number of addresses, including HPLMN and VPLMN. |  |  |
| 26 | I | The MExE ME shall allow the user to determine which MExE applications, applets and content are currently resident on the MExE ME | 02.57 v7.0.0 6.2.2 |  |  |  | Resident in terminal only or also in network? |  |  |
| 103 |  | The MExE ME shall be able to determine whether or not the GSM/GPRS/UMTS connection required by an application has been broken |  |  |  |  |  |  |  |
| 102 |  | The MExE ME shall allow the user to control , via the user profile, whether or not an application is automatically executed once it has been downloaded, according to MExE security requirements | 03.57 4.x | Optional |  |  | Execution may in some instance include configuration |  |  |
|  | Subscriber Data |  |  |  |  |  |  |  |  |
|  | Bearer Services |  |  |  |  |  |  |  |  |
| 30 | J | The MexE ME shall provide access to a range of one or more GSM/GPRS/UMTS bearer services, within the capabilities of the ME and network | 02.57 v7.0.0 7.0.0 |  |  |  | Transport protocols shall be defined in other specs (stage2,3?). Use only standard protocols (no proprietary) |  |  |
|  | Protocols |  |  |  |  |  |  |  |  |
| 34 | J | MExE protocols shall be independent of the type of underlying bearer | 02.57 v7.0.0 8.2.0 |  |  |  | What are upper layer MExE protocols? Definition required – suggest all layers above the bearer. Definition accepted – CR to definition section of 02.57 |  |  |
|  |  |  |  |  |  |  | already covered by 34 |  |  |
| 36 | J | Applications and applets shall be able to request specific bearer services and to be notified as to their availability | 02.57 v7.0.0 8.2.0 |  |  |  |  |  |  |
| 38 | J | MExE protocols shall be able to add new bearers | 02.57 v7.0.0 8.3.0 |  |  |  |  |  |  |
| 40 | I | It shall be possible for applications and services to install new protocols | 02.57 v7.0.0 8.4.0 |  |  |  | CR |  |  |
| 41 | I | MExE protocols shall be independent of the network nodes being communicated with | 02.57 v7.0.0 8.5.0 |  |  |  |  |  |  |
| 43 | I | MExE protocols shall support a generic technology independent capability negotiation, notification and enquiry mechanism between the MExE ME and the MExE service environment | 02.57 v7.0.0 8.6.0 |  |  |  |  |  |  |
| 100 | I | The MExE ME shall allow a MExE service provider to determine the MExE ME capabilities, according to MExE security requirements. | 02.57 v7.0.0 6.1.0 |  |  |  | NB: MExE Service Provider can be anybody – hence apply MExE security policy |  |  |
| 101 | I | The MExE ME shall perform capability and content negotiation according to Section 4.4. of 03.57 | 03.57 Section 4.4 |  |  |  | Table reference IN NEW VERSION |  |  |
| 48 | I | The capability negotiation mechanism shall be extensible. | 02.57 v7.0.0 8.6.0 |  |  |  | CR |  |  |
|  |  |  |  |  |  |  | Deleted: Cannot see a difference between “local services” and “services” |  |  |
| 50 | I | MExE protocols shall support the capability to transfer new applications, applets and content to and from the MExE ME as required by the user according to MexE security requirements | 02.57 v7.0.0 8.8.0 |  |  |  | CR required |  |  |
| 51 | I | The MExE protocols shall support both user initiated and MexE server initiated transfer of applications, applets and content as required by the user, according to MExE security requirements | 02.57 v7.0.0 8.8.0 |  |  |  | CR required to section 8.8 of 02.57v7.0.0 |  |  |
|  | MS Application Execution Environment |  |  |  |  |  |  |  |  |
| 55 | J | Ability to cleanly and safely install applications and associated data (e.g. icons) shall be provided. |  |  |  |  |  |  |  |
| 56 | J | Ability to cleanly and safely de-install applications and associated data (e.g. icons) shall be provided. |  |  |  |  |  |  |  |
| 57 | I | The MExE classmark 2 terminal shall be able to implement HTML |  | Mandatory |  |  | WML for classmark 1, HTML for classmark 2 This an an API req, relating to stage 2. |  |  |
| 58 | I | The MExE classmark 2 terminal shall be able to implement WML and WMLScript |  | Mandatory |  |  | This an an API req, relating to stage 2. |  |  |
| 105 | I | The MExE Classmark2 device shall contain a pre-installed WAP browser |  |  |  |  | I.e. browser capable of rendering WML and WMLScript |  |  |
| 106 | I | The MExE Classmark2 device shall allow the pre-installed WAP browser to be updated |  |  |  |  |  |  |  |
| 59 | I | MExE shall define a set of standardised content coding schemes i.e. vCard, vCalendar |  |  |  |  | Is vCard, vCalendar support required? This an an API req, relating to stage 2. |  |  |
| 60 | I | The MExE ME shall be able to support HTML and/or WML |  | Mandatory |  |  |  |  |  |
| 61 | J | MExE ME shall support the 7bit GSM default character set |  | Mandatory |  |  | As defined in 03.38 v6.0.1 Note: User defined 8bit scheme is not mentioned here. Should it be? CR to section 9.2 (last para) of 02.57v7.0.0 – replace “defined” with “identified” |  |  |
| 104 | I | The MExE ME shall be capable of supporting the UCS2 character coding scheme |  | Optional |  |  | Unicode 16bit character coding |  |  |
| 63 | I | The MExE ME shall support a core API, known as the core MExE API | 02.57 v7.0.0 9.3.0 |  |  |  | The core MExE API defines the minimal API set that a MExE ME needs to support |  |  |
| 64 | I | The core MExE API shall consist of both generic and GSM/UMTS specific aspects | 02.57 v7.0.0 9.3.0 |  |  |  |  |  |  |
| 65 | I | The MExE ME shall support standardised MExE API extensions in addition to the core MExE API | 02.57 v7.0.0 9.3.0 | Optional |  |  |  |  |  |
| 66 | J | The standardised MExE API extensions shall include access to location information, according to MExE security | 02.57 v7.0.0 9.3.0 |  |  |  | What is the meaning of "mobility information"? CR Location |  |  |
| 67 | J | The MExE APIs shall provice support for applications and applets to access the GSM/UMTS functionality present in the GSM/UMTS terminal according to MExE security . |  |  |  |  | Functionality includes telephony, e-mail, SMS, fax. |  |  |
| 68 | J | All Java APIs specified by MExE for use on a MExE ME shall be approved by Sun Microsystems. |  |  |  |  |  |  |  |
| 69 | J | The MExE ME shall provide local file system support, according to MExE security | 02.57 v7.0.0 6.1.0 | Optional |  |  |  |  |  |
| 70 | I | The MExE ME shall allow the user to identify the version number of applications, applets and content contained within it | 02.57 v7.0.0 6.1.0 | Optional |  |  |  |  |  |
| 71 | J | A MExE ME shall notify applications of any power state changes. | 02.57 v7.0.0 6.1.0 | Optional |  |  |  |  |  |
| 72 | J | A MExE ME shall support standardised access of a smartcard if a smartcard reader is present on the MExE ME | 02.57 v7.0.0 6.1.0 | Optional |  |  | Support of smartcard reader is not a mandatory requirement. |  |  |
|  | Security |  |  |  |  |  |  |  |  |
| 102 |  | The MExE terminal shall be able to access services behind secure firewalls |  |  |  |  |  |  |  |
| 76 | I | Transferred MExE applications, applets and content shall be secure and verified | 02.57 v7.0.0 11.0.0 |  |  |  | “secure” – received in same state as it was sent i.e. has not been tampered with |  |  |
| 77 | I | The MExE ME shall provide support for four security levels | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | See Table XX in Section 9.1 of 03.57 for an elaboration of security levels. Multiple security levels |  |  |
|  |  |  |  |  |  |  | This is related to requirements 79 (it is a functional solution to req79): deleted |  |  |
| 79 | I | The security level of applications, applets and content shall be verified by the MExE ME | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
|  |  | The MExE MS shall be able to verify the security level, as defined in section 11.3, of applications, applets and content transferred from the MExE service environment. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The verification process in the MS itself shall not compromise the security of the functionality and content in the MS, i.e. there shall be an isolated area within the ME (the “sandbox”) within the which the transferred material shall be verified. The transferred material shall not be able to access any MS functionality or content outside the isolated area until it has been verified in the isolated area that it is authorised to do so. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | Transferred material that fails verification or that cannot be verified due to the absence of required verification information in the MS, shall be considered as level 4 material, as defined in section 11.3. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The events that applications, applets or content are given permission by the user to initiate shall be securely recorded in the user profile. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The events that applications, applets or content are given permission by the user to initiate shall be securely recorded in the user profile. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | There shall be mechanisms within the MExE MS for ensuring that applications cannot have access to MS functionality and content beyond that allowed by their security level, as defined in section 11.3, and as contained in the user profile. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The MExE MS shall be able to detect if applications, applets and content transferred from the MExE service environment have been modified since they were assigned a security level. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | Applications, applets and content shall not be transferred to a MExE MS without the explicit permission of the MS user immediately prior to transfer or implicit permission via the user profile. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | Applications and applets transferred to a MExE MS shall not be able to initiate events without the explicit permission of the MS user immediately prior to event initiation or implicit permission via the user profile. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The user profile data for transfer and event initiation cannot be changed without the explicit agreement of the user. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The user shall be able to request the logging of all network events initiated by MExE MS applications/applets. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | MExE MS applications/applets shall not be able to send command RUN GSM ALGORITHM to the SIM. | 02.57 (V7.1.0 to be) 11.2 | Mandatory |  |  |  |  |  |
|  |  | The MExE MS shall support the three levels of user permission defined in section 8.3 | 03.57, 8.3 | Mandatory |  |  |  |  |  |
|  |  | It shall be possible to securely upgrade untrusted (level 4) applications, applets or content to level 3 applications, applets or content. | 03.57, 8.1 | Mandatory |  |  |  |  |  |
|  |  | The MExE MS shall be able to retrieve public key certificate from public key repositories | 03.57, 8.4.1 | Mandatory |  |  |  |  |  |
|  |  | The list containing the location of the user files cannot be modified by applications or applets can be modified by the user only. | 03.57, 8.4.2 |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Requires discussion with SMG10: deleted |  |  |
| 81 | I | The MExE subscriber shall be able to control whether applications, applets and content of specific security levels may be transferred to the MExE ME | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 82 | J | An application/applet shall specifiy whether or not it can share its own files with other applications/applets | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | More input required here |  |  |
| 83 | I | Within the constraints of the defined security levels, the user shall have ultimate control over the use of bearer services | 02.57 v7.0.0 11.0.0 |  |  |  | CR Incorporate USSD and SS signalling (comment by K Holley at MExE security meeting, SMG4 plen 11 – 14th Dec 1998) |  |  |
| 84 | I | A means to authenticate the MExE subscriber and MExE server shall be provided | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | CR This may be performed by the MExE ME, an intermediate entity or the GSM/UMTS network |  |  |
| 85 | I | MExE shall continue to support the inherent security and integrity of the GSM/GPRS/UMTS network | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 86 | I | The integrity of the SIM and existing GSM/UMTS security mechanisms shall not be compromised by the introduction of MExE services | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 87 | I | All aspects relating to GSM security shall be performed by the SIM | 02.57 v7.0.0 6.1.0 | Mandatory |  |  |  |  |  |
|  | Interworking with other Network Features |  |  |  |  |  |  |  |  |
| 90 | I | All services available in the network shall continue to be offered in addition to MExE services | 02.57 v7.0.0 12.0.0 |  |  |  |  |  |  |
|  | Network Interworking |  |  |  |  |  |  |  |  |
| 92 | I | All services offered in co-operation with other networks shall continue to be offered in parallel with MExE services | 02.57 v7.0.0 13.0.0 |  |  |  |  |  |  |
|  | Communications |  |  |  |  |  |  |  |  |
| 93 | J | MExE MEs shall support peer to peer communication between MExE devices | 02.57 v7.0.0 6.1.0 | Optional |  |  | Includes e.g. IrDA, Bluetooth, USB (which are outside the scope of MExE specifications), user-to-user signalling, USSD (which are within the scope of MExE specifications) etc. |  |  |
| 95 | J | A MExE application shall be able to access any or all external interfaces present on the MExE ME, subject to the limitations of the MExE ME |  | Optional |  |  | Serial, parallel, infra-red etc. |  |  |
| 96 | J | The means for MExE service provider specific applications and applets on the MExE ME to communicate with other GSM network nodes shall be provided. | 02.57 v7.0.0 6.1.0 |  |  |  | USSD main bearer |  |  |