ETSI SMG4	TSGT2#2(99)253

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 07.07 |  | Version |  | 7.2.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  | X |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | SMG4 | Date: | 15 March 1999 |
| --- | --- | --- | --- |

| Subject: | Additional result codes for +CLIP +CCWA |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The +CLIP: and +CCWA responses cannot display to the user whether a CLI is unavailable or actually withheld by the calling party |
| --- | --- |

| Clauses affected: | 7.6, 7.11 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 7.6	Calling line identification presentation +CLIP

Table 1: +CLIP parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CLIP=[<n>] |  |
| +CLIP? | +CLIP: <n>,<m> |
| +CLIP=? | +CLIP: (list of supported <n>s) |

Description

This command refers to the GSM supplementary service CLIP (Calling Line Identification Presentation) that enables a called subscriber to get the calling line identity (CLI) of the calling party when receiving a mobile terminated call. Set command enables or disables the presentation of the CLI at the TE. It has no effect on the execution of the supplementary service CLIP in the network.

When the presentation of the CLI at the TE is enabled (and calling subscriber allows), +CLIP: <number>,<type>[,<subaddr>,<satype>[,<alpha> [,<CLI validity>]]]response is returned after every RING (or +CRING: <type>; refer subclause "Cellular result codes +CRC") result code sent from TA to TE. It is manufacturer specific if this response is used when normal voice call is answered.

Read command gives the status of <n>, and also triggers an interrogation of the provision status of the CLIP service according GSM 02.81 [3] (given in <m>).Test command returns values supported by the TA as a compound value.

Defined values

<n> (parameter sets/shows the result code presentation status in the TA):

0	disable

1	enable

<m> (parameter shows the subscriber CLIP service status in the network):

0	CLIP not provisioned

1	CLIP provisioned

2	unknown (e.g. no network, etc.)

<number>: string type phone number of format specified by <type>

<type>: type of address octet in integer format (refer GSM 04.08 [8] subclause 10.5.4.7)

<subaddr>: string type subaddress of format specified by <satype>

<satype>: type of subaddress octet in integer format (refer GSM 04.08 [8] subclause 10.5.4.8)

<alpha>: optional string type alphanumeric representation of <number> corresponding to the entry found in phonebook; used character set should be the one selected with command Select TE Character Set +CSCS

<CLI validity>:

0	CLI valid

1	CLI has been withheld by the originator.

2	CLI is not available due to interworking problems or limitations of originating network.

Implementation

Optional.

## 7.11	Call waiting +CCWA

Table 2: +CCWA parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CCWA=[<n>[,<mode>[,<class>]]] | +CME ERROR: <err> when <mode>=2 and command successful +CCWA: <status>,<class1> [<CR><LF>+CCWA: <status>,<class2> [...]] |
| +CCWA? | +CCWA: <n> |
| +CCWA=? | +CCWA: (list of supported <n>s) |

Description

This command allows control of the Call Waiting supplementary service according to GSM 02.83 [5]. Activation, deactivation and status query are supported. When querying the status of a network service (<mode>=2) the response line for ‘not active’ case (<status>=0) should be returned only if service is not active for any <class>. Parameter <n> is used to disable/enable the presentation of an unsolicited result code +CCWA: <number>,<type>,<class>[,<alpha> [,<CLI validity>]]to the TE when call waiting service is enabled. Command should be abortable when network is interrogated.

The interaction of this command with other commands based on other GSM supplementary services is described in the GSM standard.

Test command returns values supported by the TA as a compound value.

Defined values

<n> (sets/shows the result code presentation status in the TA):

0	disable

1	enable

<mode> (when <mode> parameter is not given, network is not interrogated):

0	disable

1	enable

2	query status

<classx> is a sum of integers each representing a class of information (default 7):

1	voice (telephony)

2	data (refers to all bearer services; with <mode>=2 this may refer only to some bearer service if TA does not support values 16, 32, 64 and 128)

4	fax (facsimile services)

8	short message service

16	data circuit sync

32	data circuit async

64	dedicated packet access

128	dedicated PAD access

<status>:

0	not active

1	active

<number>: string type phone number of calling address in format specified by <type>

<type>: type of address octet in integer format (refer GSM 04.08 [8] subclause 10.5.4.7)

<alpha>: optional string type alphanumeric representation of <number> corresponding to the entry found in phonebook; used character set should be the one selected with command Select TE Character Set +CSCS

<CLI validity>:

0	CLI valid

1	CLI has been withheld by the originator.

2	CLI is not available due to interworking problems or limitations of originating network.

Implementation

Optional.