ETSI SMG4	TSGT2#2(99)288

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.07 |  | Version: |  |  | 7.2.0 |  |  |  |
| Submitted to SMG | 29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MS-TE |
| --- | --- |

| Source: | SMG4 | Date: | 990312 |
| --- | --- | --- | --- |

| Subject: | New command for Silent mode |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Addition of Silent Mode-setting |
| --- | --- |

| Clauses affected: | 8.xx |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

6.19	Silence Command +CSIL

Table 1: +CSIL parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSIL=[<mode>] | +CME ERROR: <err> |
| +CSIL? | +CSIL:<mode> +CME ERROR: <err> |
| +CSIL=? | +CSIL:(list of supported <mode>s) +CME ERROR: <err> |

Description

Set command enables/disables the silent mode. When the phone is in silent mode, all sounds from MS are suppressed except voice.

Read command reads the current setting.

Test command lists the supported modes.

Defined values

<mode>:

0	Silent mode off

1	Silent mode on

Implementation

Optional