ETSI SMG4	TSGT2#2(99)062

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.07 |  | Version: |  |  | 7.2.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | x |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | SMG4 | Date: | 990312 |
| --- | --- | --- | --- |

| Subject: | Time Zone |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Added functionality to set up time zone settings and reporting. |
| --- | --- |

| Clauses affected: | 8.xx, Annex B |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

8.x	Automatic Time Zone Update +CTZU

Table 1: +CTZU parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CTZU=<onoff> | +CME ERROR: <err> |
| +CTZU? | +CTZU: <onoff> +CME ERROR: <err> |
| +CTZU=? | +CTZU: (list of supported <onoff>s) +CME ERROR: <err> |

Description

Set command enables and disables automatic time zone update via NITZ. If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Read command returns the current settings in the ME.

Test command returns supported on- and off-values.

Defined values

<onoff>: integer type value indicating:

0 – Disable automatic time zone update via NITZ (default).

1 – Enable automatic time zone update via NITZ.

Implementation

Optional.

8.z	Time Zone Reporting +CTZR

Table 2: +CTZR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CTZR=<onoff> | +CME ERROR: <err> |
| +CTZR? | +CTZR: <onoff> +CME ERROR: <err> |
| +CTZR=? | +CTZR: (list of supported <onoff>s) +CME ERROR: <err> |

Description

This set command enables and disables the time zone change event reporting. If the reporting is enables the ME returns the unsolicited result code +CTZV: <tz> whenever the time zone is changed. If setting fails in an ME error, +CME ERROR: <err> is returned. Refer subclause 9.2 for <err> values.

Read command returns the current reporting settings in the ME.

Test command returns supported <onoff>-values.

NOTE: The Time Zone reporting is not affected by the Automatic Time Zone setting command, +CTZU.

Defined values

<onoff>: integer type value indicating:

0 – disable time zone change event reporting (default).

1 – Enable time zone change event reporting.

Implementation

Optional.

Annex B (normative):
Summary of result codes

V.25ter [14] result codes which can be used in GSM and codes defined in this ETS:

Table B.1: Result codes

| Verbose result code (V.25ter command V1 set) | Numeric (V0 set) | Type | Description |
| --- | --- | --- | --- |
| +CTZV: <tz> | as verbose | unsolicited | refer subclause 8.z |
| +CCCM: <ccm> | as verbose | unsolicited | refer subclause 7.15 $(AT R97)$ |
| +CCWA: <number>,<type> ,<class>[,<alpha>] | as verbose | unsolicited | refer subclause 7.11 |
| +CCWV | as verbose | unsolicited | refer subclause 8.28 |
| +CDEV: <elem>,<text> | as verbose | unsolicited | refer subclause 8.10 |
| +CIEV: <ind>,<value> | as verbose | unsolicited | refer subclause 8.10 |
| +CKEV: <key>,<press> | as verbose | unsolicited | refer subclause 8.10 |
| +CLAV: <code> | as verbose | unsolicited | refer subclause 8. |
| +CLIP: <number> ,<type>[,<subaddr> ,<satype>[,<alpha>]] | as verbose | unsolicited | refer subclause 7.6 |
| +CME ERROR: <err> | as verbose | final | refer subclause 9.2 |
| +COLP: <number> ,<type>[,<subaddr> ,<satype>[,<alpha>]] | as verbose | intermediate | refer subclause 7.8 |
| +CR: <type> | as verbose | intermediate | refer subclause 6.8 |
| +CREG: <stat>[,<lac> ,<ci>] | as verbose | unsolicited | refer subclause 7.2 |
| +CRING: <type> | as verbose | unsolicited | refer subclause 6.11 |
| +CSSI: <code1> [,<index>] | as verbose | intermediate | refer subclause 7.16 |
| +CSSU: <code2> [,<index>[,<number>, <type>[,<subaddr>, <satype>]]] | as verbose | unsolicited | refer subclause 7.16 |
| +CUSD: <m>[,<str>,<dcs>] | as verbose | unsolicited | refer subclause 7.14 |
| +DR: <type> | as verbose | intermediate | refer subclause 6.13 |
| +ILRR: <rate> | as verbose | intermediate | refer subclause 4.3 |
| BUSY | 6 | final | busy signal detected |
| CONNECT | 1 | intermediate | connection has been established |
| CONNECT <text> | manufacturer specific | intermediate | as CONNECT but manufacturer specific <text> gives additional information (e.g. connection data rate) |
| ERROR | 4 | final | command not accepted |
| NO ANSWER | 7 | final | connection completion timeout |
| NO CARRIER | 3 | final | connection terminated |
| NO DIALTONE | 5 | final | no dialtone detected |
| OK | 0 | final | acknowledges execution of a command line |
| RING | 2 | unsolicited | incoming call signal from network |