| SMG4/TSG-CN3/TSG-T2 London, UK, 15-19 March 1999 | TSGT2#2(99)087 |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.5.0 |  |  |  |
| Submitted to SMG |  |  | For approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Motorola | Date: | xx/02/99 |
| --- | --- | --- | --- |

| Subject: | Network Selection |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | X |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | The Helsinki meeting indicated that it would be possible for MExE to support Network Selection (subject to JavaPhone confirming that the required packages are available). Given this proviso, the additional support required from MExE to offer Network Selection as part of Release 98 are proposed |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

### Location of, access to, and security of, the user profile

As multiple user profiles may be defined, the user is able to set up or receive calls/connections associated with different user profiles simultaneously by securely activating a user profile (with each user profile being associated with at least one unique address).  Refer to the Security clause for further details on user profile activation.

The user’s characterisation of the MExE MS in the user profile may be modified at any time by the user and the service provider, and changes affected at the earliest possible opportunity. Refer to the Security clause for further details of user and service provider modification of a user profile.

The security clause shall apply to all user profiles

The user profile is securely managed by the MExE MS as defined in the Security clause, and stored in a secure area (i.e the SIM or the ME) of the MExE MS.  The service provider may also retain parts of the user profile in the network.

### 4.5.2	Characteristics of the user profile

The user profile acts as a repository (which is always available in the MExE MS) defining the MExE MS behaviour, identifying one or more of the following:-

|  | ME specific | Location(s) | Description |
| --- | --- | --- | --- |
| user interface |  |  | Refer to sub-clause 4.6 User interface customisation for details of the specific user interface customisation characterisation which may be stored in the user profile |
| background - colour - pattern/image |  |  |  |
| - font - type - colour - size |  |  |  |
| - interfaces - softkey definition - additional menu items - enabled/disabled extended menus - icon settings |  |  |  |
| - country - language - date - time - currency |  |  |  |
| - Modify supplementary services settings (i.e. diversion, times) | - ? |  | - ? |
| ###### Modify phone settings - ringer tone/vibrate - enabling SIM Lock - Keypad Lock |  |  |  |
| - Preferences |  |  | - 03.57, section 4.4.1 - ”Support of the user profile preferences is subject to the capabilities of the device. Generally the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to MSE.” |
| - terminal properties - language in which the user wishes to work - preferred input mode - amount of memory authorised by the user for MExE - Network Selection procedure User preference for network selection procedure:- - Automatic procedure ( as delivered with the MS), - Manual procedure, or - Downloaded procedure |  |  |  |
| - data transfer - prioritised list of preferred bearers, e.g. SMS, USSD, GPRS - maximum size of transfer - maximum duration of transfer - transfer of location information - transfer of the user profile or parts of it |  |  | - may, or may not be, sent to the MSE - could be shifted in Security section - could be shifted in Security section |
| - codecs - prioritised list of preferred audio codecs - prioritised list of preferred image codecs - prioritised list of preferred video codecs - prioritised list of preferred multimedia codecs |  |  |  |
| Service Management |  |  | Refer to sub-clauses 4.5, 4.7.1, 4.7.2 controls the ability to determine which services are transferred to, resident, configured or executing on the MS, which may be refined by use of one or more of the following characteristics: identified security level specific permitted services types of permitted services date or times for actions priorities of services and identifies service preferences which may be set globally for all services per security level on a per service basis and may be identified at installation of the service, or be subsequently modified. |
| transfer of services |  |  |  |
| automatic configuration |  |  |  |
| execution of services |  |  |  |
| residence on the MExE MS |  |  |  |
| personalisation of services |  |  | 02.57, section 6.1 for further details see section VHE of this table |
| Security |  |  | control of the following permissions maybe refined by use of one or more of the following characteristics: - universal authorisation for all calls/connections/ messages (i.e. yes/no) - authorisation by calls/connections/messages type (i.e. SMS, USSD etc.) - authorisation by identified services - authorisation by service type (i.e. security level) - authorisation by date/time/day of week |
| call authorisation |  |  | 03.57, section 4.5 |
| Implicit permission for the initiation of events (includes responses made by the MS to external events) of transferred applications, applets |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Implicit permission for the transfer of applications, applets and content |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Network event journal - status of the optional network event journal which is used to maintain a record of application initiated network events (i.e. activated/deactivated) |  |  |  |
| Miscellaneous |  |  |  |
| phonebook entry |  |  | 03.57, section 4.6 |
| ###### Redirect - voicemail box number - fax-mail box number |  |  |  |
| - reset call meters |  |  | - 03.57, section 4.6 |
| Classmark 1 specific Entries |  |  |  |
| Classmark 2 specific Entries |  |  |  |
| ###### WAP browser - pre-installed browser overwrite on upgrade - location of user preferred WAP browser |  |  | - applicable to MExE classmark 2 |
| VHE |  |  | TS 22.xx :Provision of Services in UMTS – The Virtual Home Environment (Tdoc SMG1 825/98) |
| - for each VHE profile: - Incoming session routing address |  |  |  |
| - for each service within one VHE profile: - service state - service appearance - service appearance customisation parameters - service functionality customisation parameters - service data |  |  | - e.g. activate/deactivate/not available - e.g. level of functionality complexity - e.g. forwarded-to numbers |

The above list is not exhaustive.  Support of the user profile preferences is subject to the capabilities of the device.

## 4.6	User interface customisation

The MS interface consists of the buttons, menus, screens and MMI as designed and provided by the MS manufacturer; the nature of this MS interface is naturally evolving, MS specific and proprietary to the individual manufacturers of the industry.  This interface is the one normally seen by the user in normal operation of his MS.  This specification does not place any requirements or limitations on the individual manufacturers’ MS interface.

The MExE MMI, in turn, is the interface available to the user to support MExE services and functionality on the MS.  The nature of the MExE MMI interface, like the normal MS interface described above, is not standardised in any way, to allow for manufacturer innovation, cater for evolving market needs, and permit manufacturer differentiation.  The MExE MMI, depending on different manufacturer implementations, may consist of the normal MS interface, the normal MS interface with modifications, a different interface to the normal MS interface, or some combinations thereof etc.  MExE services operate within, and using the capabilities of, the MExE MMI.

### 4.6.2	User interface customisation tokens

To support the user customisation of the interface, and portability of the preferences, table 2 defines the customisation tokens.

|  |  | Security Level |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Token | User Interface Customisation | 1 | 2 | 3 | 4 |
| Logo | Modify service provider logo | O | - | - | - |
| Custom-services | Enable/disable network operator custom services | O | - | - | - |
| Background | Modify background colour | - | - | O | - |
| Font-colour | Modify font colour | - | - | O | - |
| Font-size | Modify font size | - | - | O | - |
| Icon | Add/modify icon | - | - | O | - |
| Menu-modify | Add/remove menu item | - | - | O | - |
| Menu-activation | Enable/disable menu item | - | - | O | - |
| Softkey | Softkey definition | - | - | O | - |
| Phonebook | Add/modify phonebook entry | - | - | O | - |
| Language | Preferred language | - | - | O | - |
| Input-mode | Preferred input mode | - | - | O | - |
| Network-Selection | Network selection procedure | - | - | O | - |
| Supplementary-Services | Modify supplementary services settings (i.e. diversion, times) | - | - | O | - |
| Voicemail | Add/modify voicemail number | - | - | O | - |
| Phone-settings | Modify phone settings (i.e. ringer tone/vibrate, enabling SIM Lock, Keypad Lock etc.) | - | - | O | - |
| Extended-menus | Enable/Disable extended menus | - | - | O | - |
| Meters | Reset call meters | - | - | O | - |
| Add-char | Additional characteristics | - | - | O | - |
| Add-info | Additional Information | - | - | O | - |

Table 2: User interface customisation

Legend:-

M	the user interface customisation is mandatory and supported by the MS

O 	the user interface customisation is optionally supported by the MS, and subject to the capabilities of the terminal.  If supported by the MS, the extent of support of the user interface customisation is purely within the manufacturer’s discretion

-	the user interface customisation is not supported by the MS