3GPP TSG-T WG2 #3	TSGT2#3(99)340

Yokohama, 19-21 April 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.6.0 |  |  |  |
| Submitted to SMG |  |  | For approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Alcatel | Date: | 15/04/99 |
| --- | --- | --- | --- |

| Subject: | Updated security level definition : terminal manager concept |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | X |
| Only shall be | D Editorial modification |  |  | Release 99 |  |
| Marked with an X) |  |  |  | UMTS |  |

| Reason for change: | - Harmonisation of ISRF sreenphone and MExE ME security model (Doc 4m99-088), according business model : terminal manager may be manufacturer, operator, service provider, user … - Let free the choice of actor which is responsible of terminal in order to allow a variety of business model |
| --- | --- |

| Clauses affected: | 8 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: | Terminal manager has right to change native code, change native application, add API, modify right associated to an API |
| --- | --- |

# 8 	Security

## 8.1		Generic security

In order to manage the MExE and prevent attack from unfriendly sources or transferred applications unintentionally damaging the MExE device a security system is required. This section defines the MExE security architecture.

The basis of MExE security is

- a framework of permissions which defines the permissions transferred content, applets or applications have within the MExE

- the secure storage of these permissions (and permission type as defined in clause 8.3) in the user profile

- conditions within the execution environment that ensure that applications, applets and content can only perform actions for which they have permission.

The MExE permissions framework is defined in TS GSM 02.57 and is as follows (there is no implied hierarchy):

MExE Security Level 1 : HPLMN operator : applications, applets and content;

MExE Security Level 2 : Terminal manager : native system software upgrade (applications, file, content, resource drivers midleware, …), root certificate allocation and refresh, , new API access addition, security manager upgrade.;

MExE Security Level 3 :trusted third party : trusted applications, applets and content;

MExE Security Level 4 :untrusted third party : untrusted applications, applets and content;

The level 1 “HPLM operator” can delegate its rights to level 2 authority.

The level 2 “Terminal manager” can delegate its rights to level 1 authority.

## 8.2		Application Permissions

The following table specifies the permissions of each security level.

User Permission is required for all actions by level 1-3 applications, applets and content.  Types of user permission are defined in clause 8.3

|  | MExE Security Levels |  |  |  |
| --- | --- | --- | --- | --- |
| Actions |  | 1 Network Operator | 2 Terminal manager | 3 User Trusted party | 4 User Untrusted Party |
| 1. Read file 2. Write file 3. Get file information 4. Delete file | Yes 2 |  |  | Yes 3, 4 |
| Load native code libraries | No | Yes | No |  |
| 1. Initiate a voice/data connection 5 2. Accept a voice/data connection 5 3. Intercept an incoming voice/data connection 5 | Yes |  |  | No |
| Call forward | Yes | No | Yes 6 | No |
| 1. Terminate an existing connection 2. Hold an existing connection 3. Resume an existing connection | Yes |  |  | No |
| Modification of user invoked connection | Yes |  |  | No |
| 1. Add a phonebook entry 2. Delete a phonebook entry 3. Get a phonebook entry 4. Modify a phonebook entry | Yes |  |  | No |
| Send an SMS | Yes |  |  | No |
| Add an application or applet | Yes |  |  | No |
| Delete a saved application or applet | Yes |  | Yes 7 | No |
| 1. Add a certificate for a given level 2. Delete a certificate for a given level 3. Replace a certificate for a given level | Yes 8 |  |  | No |
| Audio access (e.g. wake-up service) | Yes |  |  | No |
| HTTP Cache manipulation | Yes |  |  | No |
| Get IMEI | Yes |  |  | No |
| Location Information | Yes | No | Yes | No |
| 1. Activate a user profile 2. Modify a user profile | Yes |  |  | No |
| Legend for above footnotes:- 2 - Except in user pre-specified directories and system directories. This list can only be modified by the user. 3 - Only in application/applet’s own directory 4 - Restriction may be required on space allocated to write 5 - A network connection may be via any supported bearer service 6 - To pre-defined numbers in the user profile 7 - Only in pre-specified directory 8 - Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate |  |  |  |  |

Table 3: Security levels and actions

## 8.3		User Permission Types

The term “user permission” is defined to mean that the user can give permission for a specific action in one of the ways defined in Table 4.  Blanket permission and oneshot permission are mandatory, and session permission is optional.

|  | User Permissions |  |  |
| --- | --- | --- | --- |
| Permission Type | Description | Invocation | Revocation |
| blanket permission | The user gives blanket permission to the application for the specified action, and the application subsequently uses the user’s original permission for the identified subsequent actions whenever the application is running. | Typically such permission would be given at application configuration or run time. | The blanket permission may be revoked by the user at any time. The user permission no longer applies once the application has been removed. |
| session permission | The user gives permission to the application for the specified action during a specific run time session of an application, and the application subsequently uses the user’s permission for the identified subsequent actions whilst the application session is still running. | Typically such permission would be given at application run time. | The session permission may be revoked by the user at any time. The user permission no longer applies once the application run time session has terminated. |
| one-shot permission | The user gives a single permission to the application for the specified action; if the application subsequently wishes to repeat the action it must again request the user’s permission for the identified subsequent action. | Typically such permission would be given at application run time. | The user permission no longer applies once the action has terminated. |

Table 4: User Permissions

Editor’s note: the consequences of upgrading of the WAP browser requires to be further elaborated.

## 8.4		Certification and Authorisation Architecture

In order to enforce the MExE security framework a MExE capable MS is required to operate an authentication mechansism for verifying downloaded applications and applets.  A successful authentication will result in the acceptance of the claimed security level.

As the MExE MS may want to authenticate content from many sources, a public key based solution is appropriate.  Before accepting level 1-3 applications, applets or content, the MExE MS will therefore check that the application, applet or content was signed with a private key, for which the MExE MS has the corresponding public key.  The corresponding public key held in the MS must either be a root public key (securely installed in the MS, say at manufacture) or a key provided in a certificate.  The MExE MS must be able to verify this certificate, i.e. have the public key (as a root key or in a certificate) corresponding to the private key used to sign the certificate.  An  certificate chain is therefore envisaged.

The requirements on authorisation and certification are given in clause 8.4.1. An example authorisation and certification process is described in clause 8.4.2.

### 8.4.1		Certification Requirements

A MExE MS cannot verify certified applications, applets or content of a particular level (from levels 1-3) unless it has a root key for that particular level.

Root keys shall be securely installed in the MExE MS, say, at manufacture.

Level 1 and level 2 keys can be identical. In that case the HPLMN operator is responsible for all the terminal management. (TBC)

It is recommended that a “disaster recovery” root key be securely installed on the terminal, to be used to install new root keys when all other root keys on the terminal are invalid.

Level 3 root keys will typically be installed along with and integrated into the MExE ME browser, as is done for PC-based browsers.

An application, applet, content can only be verified if the MExE MS contains a valid certified or root public key corresponding to the private key used to sign the applications, applets or content.

A certificates (other than root certificates) shall only be considered valid if it resides below the trust root certificate in its certification tree, or if the certificate's root CA has been cross-certified by the owner of the trust root certificate.

### 8.4.2		Example Certification Process

The following processes might be followed in order to securely download a level 3 application to a MExE MS.

Root public keys for a number of Certification Authorities (CA’s) are installed in the ME, along with the ME browser, at manufacture.  These root public keys can be used to verify certificates for level 3 applications, applets or content.

A third party software developer generates a private and public key pair (or obtains such a pair from a CA).

The third party software developer obtains a certificate for the public key from a CA.  The certificate contains the developer public key, signed with the private key of the CA.

The MExE MS download an application, applet or content of the third party software developer.

If the MExE does not already have a certificate for the public key of third party software developer, it obtains such a certificate from a public key depository.

The MExE MS verifies the certificate using the root public key, contained in the browser, of the relevant CA, and extracts the third party software developer public key.

The MExE MS verifies that the application, applet or content was signed using the private key corresponding to the third party software developer public key and installs or rejects the application, applet or content accordingly.

These processes are shown schematically in the diagram below.

Public key repository

(5) Certificate

CA

MS

GSM network

(1) Public key

(3) Certificate

3rd party software developer

(2) Certificate for public key

(4) signed application

(6) MS verifies signed application using certificate

Figure 6: Distribution of Keys

Editor’s note: [Discovery of the address of the certification authority by the MS is still work-in-progress in the IETF Secure DNS working group, but the current proposal (available as http://www.ietf.org/internet-drafts/draft-ietf-dnssec-certs-02.txt) shows how a domain name (associated with a certificate) can be generated from the certificate itself. When a domain name is known, a proposed CERT RR (resource record) is retrieved via DNS.]

## 8.5	WAP security

The WTLS security model provides a flexible framework based on TLS, which is independent of the GSM security model.

Editor’s note: Need to specify the interface with SIM based security under definition in WAP.

## 8.6	Java security

Editor‘s note: input to be provided by Avinash Palaniswamy.