ETSI SMG4	TSGT2#2(99)355

Yokohama, 19-21 April 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.6.1 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Nokia, Motorola | Date: | 20.4.1999 |
| --- | --- | --- | --- |

| Subject: | Capability negotiation |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | x |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Utilization of the work being done in WAP Uaprof group. |
| --- | --- |

| Clauses affected: | Capability negotiation |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 4.4	 Capability and content negotiation

Interaction between the MExE MS and the MSE shall be supported by the use of the hypertext transfer protocol HTTP/1.1 [9], or an HTTP/1.1 derived protocol (e.g. WSP/B as defined in Wireless Application Protocol [6]).  Communication between the MExE MS and the MSE supports:-

- Capability negotiation

- The MExE MS connects to the MSE by using HTTP/1.1 or an HTTP/1.1 derived protocol.  Capability negotiation between the MExE MS and the MSE only takes place for the first time after the MExE MS has connected to the MSE, and the MSE is informed about the MExE MS.  Without this first initial contact from the MExE MS, the MSE has no knowledge of the MExE MS, and thereafter the MSE may connect to the MExE MS by using HTTP/1.1 or an HTTP/1.1 derived protocol.

- Capability negotiation represents the mechanism by which the MExE MS and the MSE interact to inform each other of the specific mechanisms, capabilities and support which each is able to provide or support within the scope of a MExE service interaction.  The capability negotiation normally takes place prior to any content transfer between the two entities.

- Capability negotiation is used by the MSE to request the MS’ MExE capabilities, to which a response may be returned by the MExE MS.  Information is normally requested by the MSE and supplied by the MExE MS, however the MExE MS may also be informed by the MSE of its current view of the MExE MS’s capabilities.  The MExE MS may also spontaneously inform the MSE of its capabilities without initially being requested to send them (i.e. following a change in MExE support, such as removal of MExE MS from a docking station with its keyboard, mouse and monitor).  The characteristics which may be requested and transferred between the MExE MS and the MSE during the capability negotiation are identified in subclause 4.4.1 Capability negotiation characteristics.

- Content negotiation

- Content negotiation represents the means by which the MExE MS and the MSE inform each other of the requested and available form of content. If needed, the content negotiation may take place following capability negotiation between the two. The methods for content negotiation are the basic HTTP1.1. or WSP methods explained in [9] and [6].

- Content negotiation is used to select the best representation of an entity when there are multiple representations of the entity available from the MSE.  The entity (e.g. a service, an image, etc) is located behind a URI, and the application in the MExE MS connects to the URI by using HTTP/1.1 or an HTTP/1.1 derived protocol. The best representation of an entity can be decided by the server (server-driven negotiation) or by the client application (agent-driven negotiation).

- Both the capability and the content negotiation has the same purpose: to optimize the content according to client´s capabilities. The term "content negotiation" has been used e.g. in the HTTP specification and the HTTP 1.1. and the WSP contain headers to perform the content negotiation. However, the capability negotiation in MexE aims at extensing the basic HTTP and WSP methods for content negotiation. MexE terminal is free to use both the existing HTTP/WSP content negotiation methods and the new MexE capability negotiation methods.

- The content negotiation transferred between the MExE MS and the MSE is identified in subclause 4.4.2 Client Capability Report onwards.

### 4.4.1	Capability negotiation characteristics

- Method for capability negotiation is based on the CC/PP specification made by W3C, [16]. The properties and the actual schema is based on the WAP Uaprof group specification [17]. The Composite Capability/ Preferences Profiles framework is intended to provide an efficient mechanism for enabling enhanced content and service negotiation through a standardized format for user agent profiles. The use of Resource Description Framework (RDF) in CC/PP allows for interoperable encoding of the profile metadata in XML and supports multiple vocabularies to provide for future extensibility. WAP UAProf is based on the CC/PP framework. The purpose of the Uaprof is to specify

- an RDF based schema and vocabulary for CC/PP in the context of WAP UAProf that includes the class definitions and semantics of attributes described in a user agent profile, and

- guidelines for schema extensibility to support a composite profile that enables future additions to the vocabulary and schema.

- Not all capabilities have to be reported in the reqeust to the server but instead, the client may point to an URL where the server may fetch the properties.

- The Uaprof group will devide the properties into 5 different components: hardware profile, software profile, user agent profile, user preferences and network preferences. After the Uaprof group will finalize their work in July, the W3C will take over their work and develop it further.

- The generic set of capabilities which may be negotiated between the client and the server consists of the subsequently identified properties in the Uaprof schema, [17].:-

- MexE_Classmark

- MexE-Spec

- Vendor

- Model

- Screensize

- ScreenSizeChar

- ColorCapable

- AudioInputEncoders

- PointingResolution

- Language

- AudioEncoderTypes

- VideoEncoderTypes

- LocationType

- Location

Table 1: Capability negotiation characteristics

The support of the above characteristics between the MS and the MSE is either mandatory (marked with an M) or optional (marked with an O), as indicated in the Optionality column.  M indicates that it must be sent by that entity, and O indicates that it may be sent by that entity if it supports it.  A characteristic marked as O, does not necessarily require to be supported by the receiving entity.

Generally, the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to the MExE Service Environment.  As an example, for the support of location information the user’s profile controls  if and when location information may be sent to the MExE Service Environment (e..g. never sent, always sent, only after user confirmation).

The capability negotiation process shall be used by the client to permit transfer of capabilities from client to the server.  By transferring its capabilities, the client will support efficient use of resources both over the radio interface as well as in the client or server.  Capability negotiation shall performed prior to transfer over the radio interface to verify as far as possible the ability of the client to support any services to be downloaded.

In order to transfer the capability information between the MExE MS and the MSE, CC/PP method is used with the schema defined in the WAP Uaprof working group.

### 4.4.2 CC/PP over WSP (Classmark1)

In Classmark 1 the CC/PP is carried over by using CC/PP over WSP, [17].

### 4.4.3. CC/PP over HTTP (Classmark2)

In Classmark 2 the CC/PP is carried over by using CC/PP over HTTP, [15] or CC/PP over WSP, [17].

### 4.4.2	Client Content Capability Report

The client may perform content negotiation to the server by using appropriate HTTP1.1. or WSP request headers. The following methods are available for content negotiation:

Client software (product): User-Agent header

MIME media types: Accept header

Character set: Accept-Charset header

Content encoding: Accept-Encoding header

Language: Accept-Language header

There is no need for MExE to specify any tokens for content negotiation, as these headers are already defined in HTTP and WSP.  The formats for these headers are specified in [9] and [6].

Example:

The following HTTP request reports that the name of client software is "GSM-Phone" version "1.0". The client accepts both compiled WML and compiled WMLScript, and supports both the English and Swedish as languages.

GET / HTTP/1.1

Host: www.company.com

User-Agent: GSM-Phone/1.0

Accept: application/x-wap.wmlc, application/x-wap.wmlscriptc

Accept-Language: en, sv

...

The basic format of the User-Agent: header is:  User-Agent: software-name/version. A comment can be attached enclosed in parentheses to give more specific information. For example, operating system, display size, supported software extensions, script libraries, etc. The format of the comment is, however, not specified in [9].

### 4.4.3	Server role in capability negotiation

The server may request the capabilities of a client whenever required, and shall enquire of the client’s capabilities prior to making each transaction resulting in a set of transfers to the client; the characteristics which may be reported in the client capability report are identified in the list above.

In server-driven negotiation the server signals to the client that the response entity was selected from a set of available representation. To do this, the server attaches the Vary: response header in the response to the client. The Vary: header includes a list of request headers. For example:

HTTP/1.1 200 OK

Vary: User-Agent, Accept-Language

Content-type: application/x-wap.wmlc

Content-language: en

...

Indicates that the entity is available for multiple languages and user-agents. The selected entity is the English version.

### 4.4.4	Client-driven Negotiation

If the server cannot specify an optimal version for the client basing on the CC/PP sent over to the server, the server may also indicate to client which type of versions are available and let the client make the decision. This method is already available in HTTP1.1. In client-driven negotiation the client selects the best representation after having received an initial response from the server. The response from the server is a 300 Multiple Choices response and contains a list of available representations. The selection of the available representations may be done automatically by the client application or by the (human) user from a menu.

It is noted that there is an implicit overhead of (at least) one additional round-trip delay with client-driven negotiation.  The client-driven negotiation will always require an additional request/response iteration, due to the fact that the initial response from the server to the client’s initial request may be a 300 Multiple choices response, or an equivalent presentation of available choices.   After the user has selected one of the available options, the client sends the request for the actual content to the server.