| 3GPP TSG-T2#4 / ETSI SMG4 Miami, US, 14-16 June 1999 | TSGT2#4(99)494 |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.10 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | (TE-MS) multiplexer protocol |
| --- | --- |

| Source: | Ericsson | Date: | 990614 |
| --- | --- | --- | --- |

| Subject: | Clarification of how to handle the length field in basic mode |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | X |
| only shall be | D Editorial modification | X |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | In the 07.10 specification it is not totally clear how the length field in basic mode shall be handle in all situations. To avoid misunderstandings these situations have to be clarified. This CR also corrects an editorial error in the section about the UI frame. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

#### 5.2.1.5	Length Indicator

This field is present only in case when basic option is activated.

It has the following format:

| Bit | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E/A | L1 | L2 | L3 | L4 | L5 | L6 | L7 |

Figure 1: Length field, first byte

The L1 to L7 bits indicates the length of the following data field. The default length is 31 bytes.

According to the rule of ISO/IEC 13239:1997, the range of the length field may be extended by use of the EA bit. When the EA bit is set to 1 in an octet, it is signifies that this octet is the last octet of the length field. When the EA bit is set to 0, it signifies that a second octet of the length field follows. The total length of the length field is in that case 15bits, L1-L15.

The second octet of the length field (only present when the EA field in the first byte is set to 1) format:

| Bit | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L8 | L9 | L10 | L11 | L12 | L13 | L14 | L15 |

Figure 2: Length field, second byte

The length field shall always be present, even if the data field is empty.

#### 5.2.1.6	Frame Checking Sequence Field (FCS)

The FCS shall be the ones complement of the sum (modulo 2) of

a)	the remainder of

xk (x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1)

divided (modulo 2) by the generator polynomial

x8 + x2 + x + 1,

where k is the number of bits in the frame existing between, but not including, the final bit of the opening flag and the first bit of the FCS, excluding start and stop elements (start/stop transmission), and bits (synchronous transmission) and octets (start/stop transmission) inserted for transparency, and

b)	the remainder of the division (modulo 2) by the generator polynomial

x8 + x2 + x + 1

of the product of x8 by the content of the frame existing between, but not including, the final bit of the opening flag and the first bit of the FCS, excluding start and stop elements (start/stop transmission), and bits (synchronous transmission) and octets (start/stop transmission) inserted for transparency.

As a typical implementation, at the transmitter, the initial content of the register of the device computing the remainder of the division is preset to all ones and is then modified by division by the generator polynomial (as described above) of the address, control and information fields; the ones complement of the resulting remainder is transmitted as the 8-bit FCS.

At the receiver, the initial content of the register of the device computing the remainder is preset to all ones. The final remainder after multiplication by x8 and then division (modulo 2) by the generator polynomial

x8 + x2 + x + 1

of the serial incoming protected bits and the FCS, will be  1111 0011 (x7 through x0, respectively) in the absence of transmission errors.

In the case of the UIH frame, the contents of the I-field shall not be included in the FCS calculation. FCS is calculated on the contents of the address, control and length fields only. This means that only the delivery to the correct DLCI is protected, but not the information. The FCS is calculated in the normal manner for all other frames in Table Error: Reference source not found.

### 5.3.5	Unnumbered information with header check (UIH) command and response

The UIH command/response shall be used to send information without affecting the V(S) or V(R) variables at either station. UIH is used where the integrity of the information being transferred is of lesser importance than its delivery to the correct DLCI. For the UIH frame, the FCS shall be calculated over only the address, control and length fields..

Reception of the UIH command/response is not sequence number verified by the data link procedures; therefore, the UIH frame may be lost if a data link exception occurs during transmission of the protected portion of the command, or duplicated if an exception condition occurs during any reply to the command. There is no specified response to the UIH command/response.

### 5.3.6	Unnumbered Information (UI) command and response

The UI command/response shall be used to send information without affecting the V(S) or V(R) variables at either station.. Reception of the UI command/response is not sequence number verified by the data link procedures; therefore, the UI frame may be lost if a data link exception occurs during transmission of the protected portion of the command, or duplicated if an exception condition occurs during any reply to the command. There is no specified response to the UI command/response.

For the UI frame, the FCS shall be calculated over all fields (Address, Control, Length Indicator, Information).

Support of UI frames is optional.