ETSI SMG4/T2	T2-99668

Helsinki

6-9 September 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 07.10 |  | Version: |  |  | 6.3.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x |  | without presentation ("non-strategic") |  |  |  | x |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | Nokia | Date: | 27/08/1999 |
| --- | --- | --- | --- |

| Subject: | Correction of value octets in RPN command |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | x |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The "Value octet1" in the picture of the RPN frame is redundant (should not be there), since the DLCI is the first value octet. The frame RPN has 8 value octets. The first value octet (value octet1) is the DLCI and after DLCI, it comes value octet2, value octet3… and so on. In the picture on page 31, section 5.4.6.3.9 the frame has 9 value octets, the DLCI and 8 more. DLCI and value octet1 are refered to the same octet, so one of them is redundant. There are two options - If the DLCI octet remains on the picture, then value octet 1 should be removed from it. - If value octet1 remains on the picture, then DLCI octet should be removed from it. My suggestion is that in order to be consistent with the tables 10 and 11, option 1 mentioned above is more suitable. |
| --- | --- |

| Clauses affected: | 5.4.6.3.9 |
| --- | --- |

| Other specs | Other releases of same spec | x |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications | x |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

##### 5.4.6.3.9	Remote Port Negotiation Command (RPN)

This command is optional.

This command is used for set the remote port communication settings.

All devices must assure that the communication settings are correctly set, prior sending data. There are default values assigned on all parameters, if no negotiation is performed, the default value is chosen.

During a connection, a device must send the RPN whenever the communication settings are changed. The same applies for the Port Line Status.

| Command RPN | Length 1 or 8 | DLCI |  | Value octet2 optional | Value octet3 optional | Value octet4 optional | Value octet5 optional | Value octet6 optional | Value octet7 optional | Value octet8 optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The Remote Port Negotiation Command use the following type field octet:

Table 1: Type field octet

| Bit | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EA | C/R | 0 | 0 | 1 | 0 | 0 | 1 |

The length byte contains the value 1 or 8 and there are one or eight value octets.

Table 2:  DLCI octet

| Bit No. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EA | 1 |  | D | L | C | I |  |

Bit 2 in the DLCI octet is not used and always set to 1, the EA bit is according to the description in subclause 5.2.1.2. The DLCI field indicated which DLC the command is applied to.

Table 3: Port Value Octets

| Value Octet | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| 3 | D1 | D2 | S | P | PT1 | PT2 | res | res |
| 4 | FLC1 | FLC2 | FLC3 | FLC4 | FLC5 | FLC6 | res | res |
| 5 | XON1 | XON2 | XON3 | XON4 | XON5 | XON6 | XON7 | XON8 |
| 6 | XOF1 | XOF2 | XOF3 | XOF4 | XOF5 | XOF6 | XOF7 | XOF8 |
| 7 | PM1 | PM2 | PM3 | PM4 | PM5 | PM6 | PM7 | PM8 |
| 8 | PM9 | PM10 | PM11 | PM12 | PM13 | PM14 | PM15 | PM16 |

A device transmits a remote port negotiation command to the other device with the fields set to the desired values with the parameter mask indicating which parameters are set..

When the remote port negotiation command is received, the responding station replies according to the following rules:

The DLCI value may not be changed.

The receiver may accept the Port Value Octet bits proposed by the sender,  and reply with a respons with the parameter mask set to 1 for all the parameters accepted. If the receiver does not support any of the proposed values, it replies with the parameter mask set to zero for the parameters not supported. For those parameters with the parameters mask set to 1,