| 3GPP TSG-T2 #5 / ETSI SMG4 Helsinki, FINLAND, 6-9 September 1999 | TSGT2#5(99)733 |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 23.057 |  | Version |  | 1.3.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Nokia | Date: | 8.9.1999 |
| --- | --- | --- | --- |

| Subject: | Security table actions |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | x |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | x |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | To clear the meaning of the groups in the security table and give a definition for each of the groups. The aim is to help to include new actions to MExE. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 8.2	MExE executable permissions

The following table 3 specifies the permissions of operator, manufacturer and third party security domains in the order of restriction.

The actions listed in the security table 3 are generic actions. Application programming interfaces (APIs) which are intrinsically part of the MExE implementation may directly or indirectly cause these actions to be performed. When these actions occur, whether directly or indirectly called by such an API, the security restrictions shall apply.

Explicit user permission is required for all actions by MExE executables in all domains.  Types of user permission are defined in clause 8.3

File access is not permitted for untrusted MExE executables, except that untrusted Java applications can access files only in the application’s own directory. Untrusted MExE executables are not permitted access to any actions which access the phone functionality (phone functionality includes all the actions in the table 3) except for this one qualified exception, and as further identified in 8.2.1.

Actions available using interfaces giving access to the phone functionality (either in existence at the time of approval of this specification or not) that are not listed in the security table 3 shall be categorised into one of the groups in the security table 3 by comparing its action against the groups in order as they are listed in the table 3. If an action can be categorised into a more restrictive group near the top of the table, then it shall not be again categorised into another, less restrictive, group further down in the table. E.g if a new action eventually results in forwarding a call, it shall be categorised into Network access. If the action is totally new, it shall be categorised into some of the groups by comparing its functionality to the group description below and by comparing with the list of actions listed in the table within the group.

Device core function access includes functions, which are an essential part of the phone functionality enabling the phone to be in usage or not.

User Profile management includes functions, which enable management of user profiles and/or activating a particular profile.

Network selection/operator data access includes functions, which enable the management of operator-related data parameters and network settings.

Network access includes all functionalities which result in or need interaction via the operator´s network.

User preference management includes functionalities which allow the applications to modify user´s preferences.

User private data access includes all functionalities which relate to management, reading or modifying of data that the user has stored in the MS.

Certificate management includes the functions which relate to certificate handling in the MS.

Lifecycle management includes the functionalities which are needed for installining or removing MExE excutables in the MS.

Audio access includes the functionalities related to audio management and usage.

Power management includes the functionalities related to power management and usage.

Terminal Data include the rest of the functionalities which are not part of the previous groupsfunctions which relate to accessing terminal data, i.e. not user data.

Table 3: Security domains and actions

|  |  | MExE Security Domains |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Actions |  |  | Operator |  | Manufacturer |  |  | Third Party |  |
| Device core function access - Start/stop radio 2. Turn on/off device |  | No |  |  |  |  |  |  |
| User profile management - Activate a user profile - Modify a user profile |  | No |  |  |  |  |  |  |
| Network selection/operator data access - Get IMSI - Get home network - Select network |  | Yes | No |  |  |  |  |  |
| Network access - Initiate a voice/data connection 3 - Accept a voice/data connection 3 - Call forward 4 - Multiparty call 4 - Call deflection 4 - Explicit call transfer 4 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) 4 - Generate DTMF - Query network status - Get signal level - Get call list |  | Yes |  |  |  | Yes 6 |  |  |
| User preference management Modify user preferences |  | Yes7 |  |  |  |  |  |  |
| User private data access 1 - Read - Write - Get properties - Delete - Get Location Information - Read stored SMS - Delete stored SMS |  | Yes 2 |  |  |  |  |  |  |
| Certificate management - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain |  | Yes 5 |  |  |  |  |  |  |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable |  | Yes |  |  |  |  |  |  |
| Audio access - Sound generation to speaker (e.g. via stream) - Select Melody - Set speaker volume - Get melody list |  | Yes |  |  |  |  |  |  |
| Power management - Monitor the power state - Change the power state |  | Yes |  |  |  |  |  |  |
| Terminal Data 1. Get manufacturer software version |  | Yes |  |  |  |  |  |  |

Table 3 (concluded): Security domains and actions

|  |  | MExE Security Domains |  |  |
| --- | --- | --- | --- | --- |
| Actions |  |  | Operator | Manufacturer | Third Party |
| Legend for above footnotes:- 1 – User private data includes user files, phonebook, etc located on the MS. 2 – The user shall be able to specify data access permissions within the capabilities of the device. 3 – A network connection may be via any supported bearer service 4 – Multiparty, deflection, and explicit call transfer shall be permitted only to numbers explicitly supplied by the user to the MExE Executable. Modification of call forward numbers stored in the network shall only be permitted to numbers explicitly supplied by the user to the operator. 5 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. 6– The Third Party domain's permission to access the networking action depends on the provisioning mechanism as described in section 8.15 7 – Trusted applications only have permission to modify user preferences, and not to activate or de-activate them. The user shall be able to specify for each domain, the preferences that applications in that domain can access. All other preferences shall not be accessible to that domain. The default shall be that there is no access. |  |  |  |  |

The lists in the groups in the table 3 are not exhaustive, and other actions which are of the same category shall be included in the group for the purposes of requesting user permission.

Editor´s note: the table and the categories need more work.