3G TS 23.140 0.1.1 (1999-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Terminals;

Multimedia Messaging Service (MMS);

Functional description;

Stage 2

(3G TS 23.140 version 0.1.1)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTR/TSGS-0122971U

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

# Contents

Contents	3

Intellectual Property Rights	5

Foreword	5

Introduction	5

1	Scope	5

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	General Architecture	7

4.1	Overview	7

4.2	Involved MMS Elements	8

4.3	Involved MMSE Interfaces	9

5	Protocol Framework	10

6	Functional Description of Involved MMS Elements	10

6.1	Terminal (MMS User Agent)	10

6.2	MMS Relay	11

6.3	MMS Subscription database	11

6.4	MMS User Profile Database	11

6.5	Media Type Converters	12

6.6	HLR	12

6.7	“Foreign” MMS Relay/Server	12

7	MMSE Interfaces	12

7.1	MMS Relay – MMS User Agent	12

7.1.1	Functional Description	12

7.1.2	Description of Service Capability Features	12

7.1.3	Description of Media Formats	12

7.2	MMS Relay – MMS Server	13

7.2.1	Functional Description	13

7.2.2	Description of Service Capability Features	13

7.2.3	Description of Media Formats	13

7.3	MMS Relay – MMS Subscription Database	13

7.3.1	Functional Description	13

7.3.2	Description of Service Capability Features	13

7.3.3	Description of Media Formats	13

7.4	MMS Relay – MMS Profile Database	13

7.4.1	Functional Description	13

7.4.2	Description of Service Capability Features	13

7.4.3	Description of Media Formats	13

7.5	MMS Relay – HLR	13

7.5.1	Functional Description	13

7.5.2	Description of Service Capability Features	13

7.5.3	Description of Media Formats	14

7.6	MMS Relay – Media Type Converter	14

7.6.1	Functional Description	14

7.6.2	Description of Service Capability Features	14

7.6.3	Description of Media Formats	14

7.7	MMS Subscription Database – Operator	14

7.7.1	Functional Description	14

7.7.2	Description of Service Capability Features	14

7.7.3	Description of Media Formats	14

7.8	“Home” MMS-C – “Foreign” MMS-C	14

7.8.1	Functional Description	14

7.8.2	Description of Service Capability Features	14

7.8.3	Description of Media Formats	15

Annex: Simplified information flows	17

1.	Terminal originated flows	17

2.	Terminal terminated flows	21

3.	Failure Handling	22

History	25

# Intellectual Property Rights

[tbc]

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TR, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# Introduction

# Scope

This 3GPP Technical Specification defines the stage 2 and stage 3 description of the non realtime Multimedia Messaging Service, MMS. Stage 2 identifies the functional capabilities and information flows needed to support the service described in stage 1.

This TS includes information applicable to network operators, service providers and terminal, switch and database manufacturers.

This TS contains the core functions for a non realtime Multimedia Messaging Service, MMS, which are sufficient to provide a complete service.

MMS uses a number of technologies to realise the requirements of the stage 1 description (3G TS 22.140). This TS describes how the service requirements are realised with the selected technologies. As far as possible existing protocols  and existing message formats shall be used for the realisation of the Multimedia Messaging Service.

This TS contains lists of elements and interfaces involved in the Multimedia Messaging Service Environment. These lists are subject to the finalisation of MMS Stage 1 (3G TS 22.140). All items that will be identified as being outside the scope of standardisation shall be discarded according to a stable version of 3G TS 22.140 to come. This also applies to the corresponding chapters, sections and subsections.

Editor’s note: The paragraph above may be deleted after this TS is adapted to a stable version of 3G TS 22.140.

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	3G TS 22.101 (V3.6.0 onwards): “Service Principles”

[2]	3G TS 22.121 (V3.0.0 onwards): “The Virtual home Environment”

[3]	3G TS 22.140 (V0.3.1 onwards): "Multimedia Messaging Service"

# Definitions and abbreviations

## Definitions

xxxx : xxxx

## Abbreviations

For the purposes of this document the following abbreviations apply in addition to those defined in [3]:

MMSE		Multimedia Message Service Environment

# General Architecture

## Overview

Figure 1: General view of MMS provision within the different networks

Figure 1 shows a generalised view of the Multimedia Message Service architecture for a third generation messaging  system. It shall combine different networks and  network types and shall integrate messaging systems already existent within these networks. The terminal operates with  the Multimedia Messaging Service Environment, MMSE. This environment may comprise 2G and 3G networks, 3G networks with islands of coverage within a 2G network and roamed networks. The MMSE provides all the necessary service elements, e.g. delivery, storage and notification functionality. These service elements could be located within one network or distributed across several networks or network types.

## Involved MMS Elements

Figure 2 : MMS Architecture Overview

- MMS User Agent

- MMS Gateway

- MMS Server

- MMS Subscription Database

- MMS Profile Database

- Media Type Converters

- HLR

- “Foreign” MMS Gateway/Server

## Involved MMSE Interfaces

Figure 3 : MMSE Interfaces

- MMS Gateway – MMS User Agent

- MMS Gateway – MMS Server

- MMS Gateway – MMS Subscription Database

- MMS Gateway – MMS Profile Database

- MMS Gateway – Media Type Converter

- MMS Subscription Database – Operator

- “Home” MMS-C – “Foreign” MMS-C

# Protocol Framework

Figure 4: Protocol Framework to provide MMS

To provide implementation flexibility, integration of existing and new services together with interoperability across different networks and terminals, the MMS shall make use of the protocol framework outlined in Figure 4. In this framework the MMS Client communicates through the MMS Rely with the MMS Server. This MMSyGateway shall provide convergence functionality between server and client and thus enabling the integration of  different server types across different networks. It should be possible to combine Server andyGateway functionality.

# Functional Description of Involved MMS Elements

## Terminal (MMS User Agent)

- MM composition

- MM presentation

- de- and encryption / signing

- presenting notifications to the user

- all aspects of storing  MMs on the terminal and/or USIM

- handling of external devices

- user profile management

- retrieval of MMs (initiate MM delivery to the terminal)

- MMS Server / MMS Centre

- MMSE element which is responsible for storing messages.

- different Servers can be included, e.g. MMS-Server, E-Mail Server, SMS Server (SMSC), Fax, etc.

## MMS Gateway

This MMS Gateway shall provide convergence functionality between server and user agent and thus enabling the integration of  different server types across different networks. It should be possible to combine Server and Gateway functionality.

MMSE element which is responsible for the following functions:-

- receive and send MM

- enable/disable MMS function

- personalised multimedia messaging

- MM deletion

- media type conversion

- media format conversion

- message content retrieval

- MM forwarding

- screening of MM

additional functionality not provided by standard servers and transfer protocols

- negotiate terminal capabilities

- terminal availability

- MMS personalisation

- terminal notification

- charging

## MMS Subscription database

- maintain user subscription information

- controls access to the MMS

- controls the extent of available service capability (e.g. server storage space)

## MMS User Profile Database

- set of rules how to handle incoming messages and there delivery

- current capabilities of the users terminal

## Media Type Converters

- ffs

## MMSE Interfaces

## MMS Gateway – MMS User Agent

### Functional Description

Posting of multimedia message. There should be application-level error control that allows the mobile user to receive and interpret the most common error situation, such as addressing problem or non-subscribed situation.

Addressing of multimedia message. The addressing is either MSISDN or an e-mail address. The email-addressing may be needed in some cases, particularly when messages are delivered to the Internet email.

Notification of multimedia message. In the push model, the multimedia message terminal must be notified first to allow the terminal to setup the data connection. This mechanism also ensures that the mechanism is bearer-independent (e.g., CSD vs. GPRS). In this notification, some basic information of the message should be delivered.

Retrieval of multimedia message. In the retrieval, the capabilities of the terminal must first be delivered to the mobile to allow the server to discard the multimedia messages. Then, the mobile retrieves the message according to its capabilities. The retrieval is automatic.

Delivery report. The delivery report is, if requested, sent to the mobile that originated the multimedia message. It contains information whether the multimedia message has been retrieved, expired, rejected etc. There should be a unambiguous identification mechanism between the delivered multimedia message and the delivery report.

Capability negotiation. The capabilities of the mobile needs to be negotiated before any delivery to minimise the waste of bandwidth because of content that cannot be interpreted. The possibility to connect the mobile to a laptop using external cable should be considered here, since the capabilities of the laptop are potentially better than the mobile. In addition, the mobile may want to retrieve the multimedia message even if it is not capable to present since, for instance, to forward it.

    - Description of Media Formats

Message adopted from MIME, but WAP binary multipart should be used instead.

  - MMS Gateway – MMS Server

### Functional Description

Basically, WAP gateway handles this. According to WAP specifications, the communication between the gateway and the MMSC uses HTTP/1.1.

### MMS Gateway – MMS Subscription Database

### Functional Description

According to specifications stage 1, there is no need for this interface ?

### Description of Media Formats

  - MMS Gateway – MMS Profile Database

### Functional Description

WAP UAProf handles this. There is no need for definitions here

### No need to define this ?

## Editor's Note: According to WAP solution, HLR access is not needed for MMS presence access.

### MMS Gateway – Media Type Converter

### Functional Description

The media type conversion could be activated by the capability negotiation. If the received image is JPEG and the phone support only GIF, a media conversion may be done. The media conversions, however, often produces inacceptable results. The media conversions should be optional, since it is impossible to define what media conversions are necessary. It is more important to produce a

### Description of Media Formats

Most probably, the media converter is tightly coupled with the multimedia messaging centre. Thus, specifying a separate interface between the MMSC and the multimedia messaging converter is not necessary.

## MMS Subscription Database – Operator

### Functional Description

The subscription database includes MMS subscription information, so basically whether a mobile subscriber can send multimedia messages, what is the maximum size for the multimedia message to be sent and similar information. The subscription database is consulted every time the mobile user wants to send a multimedia message.

### Description of Media Formats

The

## “Inter MMS-C Communication

### Functional Description

ffs

### Description of Media Formats

## MMS Security Model

### Delivery of Multimedia Message

The delivery of multimedia message shall be secure so that no-one can send multimedia message without being properly charged. In addition, the sending of malicious multimedia message shall be prevented. The terminal authentication shall be done using tamper-free approach, such as the WTLS and the WIM.

### Retrieval of Multimedia Message

The retrieval is divided into two phases: first notification and then retrieval. First, the notification shall be authenticated so that a valid server is sending the notification. This could be achieved by using the keys provided by the server at subscription phase. Then, at retrieval phase, the terminal access shall be authenticated. This could be obtained by using the WAP tools WTLS and WIM for authentication.

### Delivery Report

The delivery report is unambiguously identified within the scope of one terminal. Thus, there is no need for authentication of the source of the report.

## MMS Presentation

The concept of MMS presentation means the layout, sequencing and timing of multimedia objects on the terminal screen and other devices such as speaker. The layout technique should be future-proof since the future terminals will have more advanced display and sound capabilities than today. In the future, there may be full multimedia slide-shows available for the terminal.

The Synchronised Multimedia Integration Language (SMIL) is a well-known W3C effort for synchronised multimedia. It provides the necessary layout, sequencing and timing tools for the multimedia presentations. Figure Error: Reference source not found presents an example of SMIL having a single image and text parts.

## Media Types for Multimedia Messaging

### Text

The text should be supported by all multimedia messaging terminals.

The character encoding shall be ISO 10646-1 UTF-8 (Unicode).

### Images

Minimum set of supported image formats could be for example:

Baseline JPEG (Joint Photographic Expert Group)

Internet Media Type (mime): image/jpeg

GIF 87, GIF 89a (Graphics Interchange Format)

Internet Media Type (mime): image/gif

### Audio

The minimum set of supported audio formats could be for example AMR.

# Annex: Simplified information flows

### Terminal originated flows

Figure 5: MM origination

The MM is composed on the terminal and sent as a submission to the Gateway. Then the Gateway may check the subscription data if required (subject to prior MM session establishment) and then obtains the personalised information from the profile. MM is processed and stored according to the profile information in preparation for delivery.

Figure 6: Query MM Information

The figure above shows the terminal originated MM query about messages. The terminal sends a query to the Gateway about messages (and message contents) stored in the server. Then the Gateway makes a user data check from the Subscription database if required (subject to prior MM session establishment). The purpose of this action is to ensure that the user has the right to perform this operation.

The Gateway sends a request of MM information to the server. Server responses with message information. This response could contain e.g. the information about the number of messages stored in the server and information what kind of message elements stored messages contains. Then Gateway obtains the personalised information of the recipient from the Profile. Finally the Gateway sends to the terminal notification with message information.

Figure 7: MM pull delivery

The terminal requests a delivery of a specific MM from the Gateway. The whole MM or parts of it could be requested. The Gateway checks the subscription data if required (subject to prior MM session establishment) and then sends a request to the server  to deliver the wanted elements. When the Gateway receives the information from the server, it obtains the personalised information from the profile and converts the MM if necessary. Then the MM is sent to the terminal and the terminal acknowledge the delivery.

Figure 8: MM session establishment

The login request, including the user's credentials, is checked by the Gateway function either when the MM user agent starts or upon initiation by the user.  The purpose of the request is to allow the user to be authenticated and gain access to MM server and establish a MM session.  During this process the MM user agent also sends profile information relating to the capabilities of the terminal and preferences of the user.

Figure 9: MM session termination

User Agent submits logout request to the Gateway. The Gateway may check the subscription data if required (subject to prior MM session establishment) and activate the logout profile if required. Then the Gateway submits the acknowledge of logout request to the User Agent, finally User Agent closes itself.

Figure 10: Profile management

A profile management command (e.g. modify the profile settings) is sent to the Gateway by the terminal. The Gateway may check the subscription data if required (subject to prior MM session establishment) and then gets access to the profile where the requested command is performed. The result of this operation is then sent back to the terminal via the Gateway.

### Terminal terminated flows

Figure 11 : MM push delivery

When a MM has arrived at the Server, the Server notifies the Gateway. This notification contains further information of the MM (e.g. directory of MM elements together type and size). Then the Gateway may check the subscription data if required (subject to prior MM session establishment) and then obtains the personalised information from the profile.

After requesting the MM or MM elements by the Gateway, the server delivers the MM or elements to the Gateway. If conversion is necessary, the Gateway will do the appropriate conversion before the MM or MM elements are delivered to the terminal. If  MM elements are processed according to the profile information at the Server, e.g. MM or MM elements are deleted, forwarded to another server or stored for later retrieval, a notification is embedded within the MM delivery. The MM will be stored and the Terminal sends an acknowledgement to the Gateway.

Figure 12: notification of automatic server actions

When a MM has arrived at the Server, the Server notifies the Gateway. This notification contains further information of the MM (e.g. directory of MM elements together type and size). Then the Gateway may check the subscription data if required (subject to prior MM session establishment) and then obtains the personalised information from the profile. MM is processed according to the profile information at the Server, e.g. MM or MM elements are deleted, forwarded to another server or stored for later retrieval using pull delivery. Subsequent to that server processing, the Gateway may notify, depending on the settings in the profile, the Terminal of the actions undertaken at the Server.

### Failure Handling

This section identifies different failure situations which may occur during the information exchange scenarios.

Figure 13: Failure handling (subscription check failure)

If the subscription check fails, e.g. due to an invalid subscription, the subscription database responds with an error to the Gateway which forwards the error response to the terminal. Then the terminal rejects the originating request.

Figure 14: Failure handling (profile check failure)

Another failure occurs if the Gateway does not receive the expected response form the Profile after obtaining profile information within a specified time frame. Then the Gateway sends an error to the Terminal indicating an unsuccessful operation.

Figure 15: Failure handling (download failure)

Another failure occurs if the Gateway does not receive the expected response from the Server after a request of delivering MM or MM elements within a specified time frame. Then the Gateway sends an error to the Terminal indicating an unsuccessful operation.

Figure 16: Failure handling (upload failure)

Another failure occurs if the Gateway does not receive an expected acknowledgement from the terminal delivering MM or MM elements, within a specified time frame. Then the Gateway sends an error to the Terminal indicating an unsuccessful operation.

Editors note:- confirmation of delivery and confirmation of MM being presented and Gateway action after successful MM delivery must be added

# History

| Document history |  |  |
| --- | --- | --- |
| V. 0.0.0 | October 1999 | Initial Skeleton |
| V. 0.1.0 | October 1999 | Version after TSG-T-WG2#5 |
| V.0.1.1 | November 1999 | Version after TSG-T-WG2 MMS Stage 2 ad hoc |

Rapporteur for 3G TS 23.140 is: Gunnar Schmidt Bosch Telecom GmbH Tel. : +49 5341 285 853 Fax : +49 5341 285 140 Email : gunnar.schmidt@de.bosch.com

This document is written in Microsoft Word version 97